3GPP TSG-RAN WG4 Meeting #113		R4-2418551

Orlando, US, 18th – 22nd November, 2024

Agenda item:	7.16.2

Source:	Apple

Title:	On UE demodulation performance requirements with 8RX

Release:	Rel-19

Document for:	Discussion

# 1. Introduction

In RAN4#112bis the UE demodulation enhancements in Rel-19 was discussed in detail and way forward [1] was agreed. In this contribution we share our views on UE demodulation requirements with 8RX.

# 2. Discussion

## PDSCH requirements with inter cell interference

For PDSCH requirements for inter-cell interference the following agreements were made:

Interference profile and power levels Agreement: - Reuse the same interference profiles (i.e., cover HomNet and Hetnet scenarios) and same power levels (i.e., INR values) defined for Rel-17 2/4Rx. - TBD: Further discuss applicability rules to potentially reduce UE effort to only comply to either 1 or 2 interference cells. Interference signal modulation order Agreement: - Reuse the same interference signal modulation order as 2/4Rx requirements, i.e., random 16QAM symbols. - Interested companies can provide results with 64QAM Network type and SCS/CBW Agreement - FDD: 15kHz/10MHz - TDD: 30kHz/40MHz, 7D1S2U(S=6D+4G+4U) Other parameters Agreement - Use 2Tx for interreference cell. - Frequency shift to the target cell: 300Hz for interference cell 1, -100Hz for interference cell 2 - SSB configuration as Rel-17 - Transmission rank: Random rank with 70% and 30% probability for rank 1 and rank 2 for all interference cells

As a starting point for evaluation initial simulation parameters were agreed. Our simulation results based on agreed simulation parameters for evaluation are presented in [2]. Based on simulation results for PDSCH demodulation in inter-cell interference, our observations for baseline receiver are:

- For all cases we observe gain with MMSE-IRC receiver over MMSE receiver

- For all the cases the SINR is feasible – not very low

- The operating SNR/SINR with rank 1 MCS 17 is similar to R17 requirement with 4RX

- The SNR with 2 layers, 4 layers is high and not practical in intercell interference scenarios

Based on the observations, we propose to introduce requirements with 8RX for inter-cell interference with MCS 17 rank 1.

- For PDSCH demod requirements in inter-cell interference configure target UE MCS 17 with rank 1.

Other parameters

For other parameters not explicitly discussed we propose to reuse the parameters from Rel-17 ICI requirements with 2RX/ 4RX.

- For PDSCH demod requirements in inter-cell interference reuse the parameters from Rel-17 2RX/4RX for those not explicitly discussed/ captured.

## PDSCH requirements with intra cell inter user interference

For PDSCH requirements with intra-cell inter-user interference with 8 RX the following agreements were made:

Network Type and SCS/CBW Agreement: - FDD: 15kHz/10MHz (as priority for initial simulation analysis). - TDD: 30kHz/40MHz, 7D1S2U (S=6D+4G+4U) Interference modelling Agreement: - Beamforming model: MU-MIMO Beamforming Model for Rel-17 2/4Rx in B.4.2 in TS38.101-4 - MO of paired UE: Random 16QAM Number of coUEs Agreement: - Consider only 1 coUE Propagation condition Agreement: - 1+1: TDLA30-10. Reconsider TDLC300-100 if TDLA30-10 found not feasible - 2+2: TDLA30-10 - 4+4: TDLA30-10 Precoding method for the target UE Agreement: - Random precoder Precoding method for the co-scheduled UE Agreement: For initial simulation alignment use: - Rank 1+1: random and orthogonal. - Rank 2+2: orthogonal. - Rank 4+4: orthogonal. For orthogonal precoding, select the precoder to ensure any column of precoder is orthogonal to any column of precoder for the target PDSCH Antenna, rank (target UE + co-scheduled UEs) configuration and MIMO correlation Agreement: For initial simulation analysis: - Rank 1+1 for 2T8R ULA Low - Rank 2+2 for 4T8R ULA Low - Rank 4+4 for 8T8R ULA Low - Note: Will require DMRS len=2, hence optional feature, which means if introduced requirements would also be needed for one mandatory configuration (i.e. either 1+1 or 2+2) - Note: TBD: Down-selection of configurations. DMRS ports Agreement: - Rank 1+1 and 2+2, reuse same configuration as Rel-17 - Rank 4+4, use different CDM group for 4+4 case. Serving UE with ports {1000, 1001,1004, 1005} and co-scheduled UE with ports {1002, 1003, 1006, 1007} as a starting point. Signal power assumptions Agreement: Average target UE signal power is equal to RankTargetUE/RankTotal and average interference UE signal power is equal to RankInterfUE/RankTotal SNR assumptions Agreement: SNR = (STargetUE+ SInterfUE)/N Codebook type Agreement: Type I Single Panel only - 4Tx: (N1, N2) = (2,1) - 8Tx: (N1, N2) = (4,1) PRB bundling size and precoding granularity Agreement: 2PRBs for PRB bundling size and precoding granularity PDCCH and PDSCH allocation in time domain Agreement: - Use symbols #0 and #1 of each slot for PDCCH - PDSCH mapping type A, Start symbol 2, Duration 12 PDSCH allocation in frequency domain Agreement: Full PRB

Simulation parameters for initial evaluation were agreed in [1] and our results are summarized in [2]. Based on the results for baseline receiver we have the following observations:

- With MCS 13 ,1+1 rank combination and orthogonal precoder there is limited gain with MMSE-IRC over MMSE

- With MCS 17 ,1+1 rank combination and orthogonal precoder shows gain with MMSE-IRC over MMSE

- For 2+2 rank combination MCS 13/17/19 give reasonable results and show performance gain over MMSE

For rank combination 1+1 limited gain with MMSE -IRC receiver is observed with orthogonal precoder, especially for lower MCS – MCS13. But with MCS 17, the gain is measurable, and feasible to define requirements.

- For rank combination 1+1 define requirements with MCS 17 with orthogonal precoder.

For other rank combinations, we don’t think 4+4 rank combination is very practical in real deployments. Hence we propose to only define requirements with rank combination 2+2 with 8RX.

In Rel-17 for 2RX/4RX requirements for MU-MIMO were defined with MCS13 for both 1+1 and 2+2. For 8RX, we propose to define requirements with MCS 17 for both 2+2 and 1+1.

- For rank combination 2+2 define requirements with MCS 17 with orthogonal precoder.

- Do not define requirements with rank combination 4+4.

Other parameters

For other parameters not explicitly discussed we propose to reuse the parameters from Rel-17 MU-MIMO requirements with 2RX/ 4RX.

- For PDSCH demod requirements in intra-cell inter-user interference reuse the parameters from Rel-17 2RX/4RX for those not explicitly discussed/ captured.

## Wideband CQI reporting requirements with inter-cell interference

For CQI reporting requirements in ICI it was agreed to define requirements. We discuss the open issues related to test parameters below.

Propagation condition

Proposals

- Option 1: 
Reuse the same configuration for Rel-17 2/4Rx: 
- TDLA30-5 for the serving cell
- AWGN for the interference cell.

- Option 2: 
Further study whether to also include fading channel conditions.

- Option 3: 
CQI requirements under fading channel (If time allows)

- Option 4:
Consider the assumption assumed for Rel-17 CQI reporting fading tests with inter-cell interference as baseline

- Other options not precluded

- For CQI reporting in ICI, reuse the same propagation condition as R17 – TDLA30-5 for serving cell and AWGN for interference cell.

CQI calculation algorithm

Proposals

- Option 1: 
Define CQI reporting requirements as below: 
- Baseline SU-MIMO 8Rx receiver with Baseline CQI calculation
- Simplified SU-MIMO 8Rx receiver with Simplified CQI calculation.

- Option 2: 
RAN4 to define CQI requirements for two receiver types:
- Baseline receiver: UE performs demodulation and CSI calculation with 8Rx joint processing.
- Simplified receiver: UE performs demodulation and CSI calculation with two separate 4Rx processing.

- Option 3:
Only define one CQI calculation assumption to match both receiver types.

- Option 4:
Introduce one CQI calculation assumption independent of receiver type. Reconsider if later proven that two CQI calculation assumptions matching the two receiver types are needed.

- Option 5:
RAN4 to evaluate the throughput ratio for both receiver assumptions based on the agreed parameters/settings. If the throughput ratio is close, then we can go for one requirement. Otherwise, RAN4 can define two sets of requirements for different type of receiver.

- Other options not precluded.

There is only 1 requirement defined for CQI reporting with 8RX in Rel-18, which is for any receiver implementation. We don’t see a precedent for CQI reporting with baseline and simplified receiver in Rel-18. Hence, we propose to define single requirements for CQI reporting in ICI with 8RX.

- In Rel-18 CQI reporting requirements with rank 4 are not defined for baseline and simplified receiver. There is no precedent for defining different requirements for different receiver implementations for CQI reporting.

- Define single requirement for CQI reporting in ICI with 8RX for all receiver implementations.

Test metric

Proposals

- Option 1: 
Reuse the same test metric for Rel-17 2/4Rx:

a) the ratio of the throughput obtained when transmitting the transport format indicated by each reported wideband CQI index subject to an interference source with specified INR and that obtained when transmitting the transport format indicated by each reported wideband CQI index subject to a white Gaussian noise source shall be ≥  b) when transmitting the transport format indicated by each reported wideband CQI index subject to an interference source with specified INR, the average BLER for the indicated transport formats shall be greater than or equal to X%.

- Other options no precluded

- Re-use the same metric as Rel-17 for CQI reporting in ICI.

Rank for the serving cell

Proposals

- Option 1: 
Rank 1

- Option 2: 
Rank 2

- Option 3: 
Rank 4

- Option 4:
 Cover the following for initial study and possible down select based on simulation results:
- Rank 1, 2 & 4

- Other options not precluded.

We prefer to reuse the same setup as Rel-17 and configure rank 1 for target cell for 8RX requirements in ICI. For CQI reporting the codebook subset restriction is also specified. We propose to reuse the same as that in Rel-17.  is

- Use rank 1 for rank for serving cell. Use codebook subset restriction of 000001.

Antenna configuration and MIMO correlation for the serving cell

Proposals

- Option 1: 
2T8R

  - Option 1a: 
- 2T8R with ULA Low

  - Option 1b: 
- 2T8R with rank 1

- Option 2: 
4T8R

  - Option 2a: 
4T8R with ULA Low, Rank 4

  - Option 2b: 
4T8R with Rank 4

- Other options not precluded.

- Configure 2T8R with low correlation for serving cell.

RI Restriction

Proposals

- Option 1: 
no RI restriction (lower than Rank 4) configured for simulation

- Other options not precluded

- Configure rank restriction of 00000001 (rank 1 only).

Antenna configuration the interference cell

Proposals

- Option 1: 
1T8R

- Other options not precluded

- Configure 1T8R for interference cell.

CSI-IM and NZP CSI-RS configurations

Proposals

- Option 1: 
Reuse agreement from R17 2/4Rx inter-cell interference suppressing requirement for simulation: 
- CSI-IM on serving cell overlapping with PDSCH from interference cell
- NZP CSI-RS on serving cell overlapping with NZP CSI-RS from interference cell

- Other options not precluded

Issue 1-4-10: Other parameters

Proposals

- Option 1: 
Re-use the same configuration for Rel-17 2/4Rx

- Option 2:
For not specifically agreed parameters in this WID, reuse the same configuration for Rel-17 2/4Rx for MMSE-IRC as a starting point for initial evaluation.

- Other options not precluded.

- For CSI-IM, NZP CSI-RS configurations and other parameters re-use the same config as Rel-17 requirements for CQI reporting in ICI.

Issue 1-4-11: Interference power level

Proposals

- Option 1: 
Reuse the same configuration for Rel-17 2/4Rx, i.e., INR = 10.04dB with rank 1

- Other options not precluded.

- Reuse the same configuration as Rel-17 2/4Rx for interference power level, i.e., INR = 10.04dB with rank 1.

## Applicability rule and release independence

The following options were captured in [1] for applicability rule and release independence.

Test applicability

Proposals

- Option 1: 
The new requirements for 8Rx MMSE-IRC receiver shall be mandatory requirements for all 8Rx UEs without additional test applicability rule

- Option 2: 
For UEs supporting both 8Rx and 2/4Rx, the 2/4Rx tests under inter cell interference and under intra cell inter user interference can be skipped if the corresponding 8Rx requirements are passed.

- Option 3: 
Introduce applicability rule to skip 2/4Rx requirements if 8Rx tests are passed, if same test scenarios/configurations as 2RX/4RX are defined.

- Option 4: 
- The new requirements for 8Rx MMSE-IRC receiver to be mandatory requirements for all 8Rx UEs
- If agreed to define two set of requirements (baseline and simplified receiver), introduce new applicability rules based on Table 5.1.1.3-1 of TS38.101-4.

- Option 5: 
Introduce applicability rule to skip 2/4Rx requirements if 8Rx tests are passed regardless the concrete test setup.

- Other options not precluded.

Some of options above are not mutually exclusive as one is for mandatory requirements and the other is for test applicability rule. Few options are supported by us.

- The new requirements for 8Rx MMSE-IRC receiver should be mandatory requirements for all 8Rx UEs without additional UE capability signalling.

- Introduce applicability rule to skip 2/4Rx requirements if 8Rx tests are passed.

In addition, we would also like to introduce test applicability for ICI requirements like in Rel-17. To reduce testing burden, it was agreed that if both FDD and TDD are supported, then test case with 2 interference cell (HomNet) will be tested in FDD and test requirements with 1 interference cell (HetNet) will be tested in TDD for 2RX and 4RX.

- For PDSCH demod requirements in ICI with 8RX introduce applicability rule as: If UE supports FDD and TDD modes, requirement with 2 interference cells is applicable in FDD, requirement with 1 interference cell is applicable in TDD.

Release independent

Proposals

- Option 1: 
The new requirements for 8Rx should be release independent from Rel-18

- Option 2: 
The requirement should be released independent from Rel-18 if no new signalling or UE capability introduced.

- Option 3: 
Postpone the discussion on release independency until agreements are made on baseline assumptions, test parameters and test metrics.

- Option 4: 
Discuss release independence requirements at a later stage in the WI.

- Other options not precluded.

Regarding release independence it is very early to discuss this as this is the first few meetings to discuss this WID.

- Discuss release independence requirements at a later stage in the WI.

# 3. Conclusion

In this paper, we provide our views on UE demodulation requirements with 8RX. Our observations and proposals are captured below:

PDSCH requirements with inter cell interference

For all cases we observe gain with MMSE-IRC receiver over MMSE receiver

For all the cases the SINR is feasible – not very low

The operating SNR/SINR with rank 1 MCS 17 is similar to R17 requirement with 4RX

The SNR with 2 layers, 4 layers is high and not practical in intercell interference scenarios

- For PDSCH demod requirements in inter-cell interference configure target UE MCS 17 with rank 1.

- For PDSCH demod requirements in inter-cell interference reuse the parameters from Rel-17 2RX/4RX for those not explicitly discussed/ captured.

PDSCH requirements with intra cell inter user interference

With MCS 13 ,1+1 rank combination and orthogonal precoder there is limited gain with MMSE-IRC over MMSE

With MCS 17 ,1+1 rank combination and orthogonal precoder shows gain with MMSE-IRC over MMSE

For 2+2 rank combination MCS 13/17/19 give reasonable results and show performance gain over MMSE

- For rank combination 1+1 define requirements with MCS 17 with orthogonal precoder.

- For rank combination 2+2 define requirements with MCS 17 with orthogonal precoder.

- Do not define requirements with rank combination 4+4.

- For PDSCH demod requirements in intra-cell inter-user interference reuse the parameters from Rel-17 2RX/4RX for those not explicitly discussed/ captured.

Wideband CQI reporting requirements with inter-cell interference

- For CQI reporting in ICI, reuse the same propagation condition as R17 – TDLA30-5 for serving cell and AWGN for interference cell.

In Rel-18 CQI reporting requirements with rank 4 are not defined for baseline and simplified receiver. There is no precedent for defining different requirements for different receiver implementations for CQI reporting.

- Define single requirement for CQI reporting in ICI with 8RX for all receiver implementations.

- Re-use the same metric as Rel-17 for CQI reporting in ICI.

- Use rank 1 for rank for serving cell. Use codebook subset restriction of 000001.

- Configure 2T8R with low correlation for serving cell.

- Configure rank restriction of 00000001 (rank 1 only).

- Configure 1T8R for interference cell.

- For CSI-IM, NZP CSI-RS configurations and other parameters re-use the same config as Rel-17 requirements for CQI reporting in ICI.

- Reuse the same configuration as Rel-17 2/4Rx for interference power level, i.e., INR = 10.04dB with rank 1.

Applicability rule and release independence

- The new requirements for 8Rx MMSE-IRC receiver should be mandatory requirements for all 8Rx UEs without additional UE capability signalling.

- Introduce applicability rule to skip 2/4Rx requirements if 8Rx tests are passed.

- For PDSCH demod requirements in ICI with 8RX introduce applicability rule as: If UE supports FDD and TDD modes, requirement with 2 interference cells is applicable in FDD, requirement with 1 interference cell is applicable in TDD.

- Discuss release independence requirements at a later stage in the WI.

# Reference

- R4-2416555, “Way Forward for [112bis][320] NR_demod_Ph5_Part2_UE”, Nokia.

- R4-2418552, “Simulation results for ICI and MU-MIMO with 8RX”, Apple.