3GPP TSG-RAN WG4 Meeting #113		R4-2418552

Orlando, US, 18th – 22nd November, 2024

Agenda item:	7.16.2

Source:	Apple

Title:	Simulation results for ICI and MU-MIMO with 8RX

Release:	Rel-19

Document for:	Information

# 1. Introduction

In RAN4#112bis UE demodulation enhancements in Rel-19 was discussed and way forward [1] was agreed. The simulation parameters for evaluation of performance in ICI and MU-MIMO with 8RX were agreed. In this contribution present simulation results for initial evaluation.

# 2. Simulation Results

## PDSCH requirements with inter cell interference

For evaluation of performance in inter-cell interference scenario, the same interference scenarios and interference model was used.

Homogeneous network

2 Interference cells

INR1: 7.77 dB

INR2: 2.29 dB

Heterogeneous network

1 Interference cell

INR1: 7.58 dB

Table 1 presents results with baseline receiver.

Table 1: Simulation Results for PDSCH in ICI with baseline receiver

| TC # | Scenario | MIMO | Rank (target UE) | MCS (target UE) | Channel Model | SNR @ 70% Max TP (dB) |  |  | SINR (dB) with MMSE-IRC |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | MMSE-IRC | MMSE | Gain over MMSE |  |
| 1 | HomNet (2 Inf cells) | 2T8R | 1 | 17 | TDLC300-100 | 6.45 | 10.53 | 4.09 | -2.94 |
| 2 |  | 2T8R | 2 | 17 | TDLA30-10 | 8.39 | 12.53 | 4.14 | -1.00 |
| 3 |  | 4T8R | 4 | 13 |  | 8.89 | 13.74 | 4.84 | -0.49 |
| 4 | HetNet (1 inf cell) | 2T8R | 1 | 17 | TDLA30-10 | 3.48 | 7.02 | 3.54 | -4.80 |
| 5 |  | 2T8R | 2 | 17 |  | 7.07 | 10.89 | 3.82 | -1.21 |
| 6 |  | 4T8R | 4 | 13 |  | 7.18 | 12.35 | 5.17 | -1.10 |
| 7 | HomNet (2 inf cell) | 2T8R | 1 | 19 | TDLC300-100 | 8.41 | 12.19 | 3.78 | -0.97 |
| 8 |  |  | 2 | 13 | TDLA30-10 | 5.18 | 9.60 | 4.42 | -4.20 |
| 9 |  |  |  | 19 |  | 10.29 | 14.35 | 4.07 | 0.90 |
| 10 | HetNet (1 inf cell) | 2T8R | 1 | 19 | TDLA30-10 | 5.13 | 8.65 | 3.52 | -3.15 |
| 11 |  |  | 2 | 13 |  | 4.17 | 8.22 | 4.05 | -4.11 |
| 12 |  |  |  | 19 |  | 8.98 | 12.69 | 3.71 | 0.70 |

Based on the results in table 1, we have the following observations for baseline receiver:

- For all cases we observe gain with MMSE-IRC receiver over MMSE receiver

- For all the cases the SINR is feasible – not very low

- The operating SNR/SINR with rank 1 MCS 17 is similar to R17 requirement with 4RX

- The SNR with 2 layers, 4 layers is high and not practical in intercell interference scenarios

- For all cases we observe gain with MMSE-IRC receiver over MMSE receiver

- For all the cases the SINR is feasible – not very low

- The operating SNR/SINR with rank 1 MCS 17 is similar to R17 requirement with 4RX

- The SNR with 2 layers, 4 layers is high and not practical in intercell interference scenarios

## PDSCH requirements with intra cell inter user interference

For the simulation assumptions agreed in [1] Table 2 below captures the simulation results from initial evaluation with baseline receiver.

Table 2: Simulation results for PDSCH in intra-cell inter user interference with baseline receiver

| TC # | MIMO | Rank for target + Co-UE | MCS (target UE) | Precoder selection | Modulation order for the co-scheduled UE | SNR @ 70% Max TP |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | MMSE-IRC | MMSE | Gain over MMSE |
| 1 | 2T8R | 1+1 | MCS 13 | Random | 16QAM | 4.8 | 9.6 | 4.9 |
| 2 |  |  |  | Orthogonal |  | 2.9 | 3.0 | 0.1 |
| 3 | 4T8R | 2+2 | MCS 13 | Orthogonal |  | 5.8 | 16.9 | 11.1 |
| 4 | 8T8R | 4+4 | MCS 13 | Orthogonal |  | 13.0 | NaN | Inf |
| 5 | 2T8R | 1+1 | MCS 17 | Random | 16QAM | 8.2 | 28.7 | 20.5 |
| 6 |  |  |  | Orthogonal |  | 6.2 | 7.5 | 1.4 |
| 7 | 4T8R | 2+2 | MCS 19 | Orthogonal |  | 10.9 | NaN | Inf |
| 7a | 4T8R | 2+2 | MCS 17 | Orthogonal |  | 9.3 | NaN | Inf |
| 4a | 8T8R | 4+4 | MCS 17 | Orthogonal |  | 18.7 | NaN | Inf |

Based on the results in Table 2, we have the following observations:

- With MCS 13 ,1+1 rank combination and orthogonal precoder there is limited gain with MMSE-IRC over MMSE

- With MCS 17 ,1+1 rank combination and orthogonal precoder shows gain with MMSE-IRC over MMSE

- For 2+2 rank combination MCS 13/17/19 give reasonable results and show performance gain over MMSE

- With MCS 13 ,1+1 rank combination and orthogonal precoder there is limited gain with MMSE-IRC over MMSE

- With MCS 17 ,1+1 rank combination and orthogonal precoder shows gain with MMSE-IRC over MMSE

- For 2+2 rank combination MCS 13/17/19 give reasonable results and show performance gain over MMSE

# 3. Conclusion

In this paper, we provide our simulation results for initial evaluation for UE demodulation requirements with 8RX. Our observations are captured below:

PDSCH demod in inter-cell interference

- For all cases we observe gain with MMSE-IRC receiver over MMSE receiver

- For all the cases the SINR is feasible – not very low

- The operating SNR/SINR with rank 1 MCS 17 is similar to R17 requirement with 4RX

- The SNR with 2 layers, 4 layers is high and not practical in intercell interference scenarios

PDSCH demod in intra-cell inter-user interference

- With MCS 13 ,1+1 rank combination and orthogonal precoder there is limited gain with MMSE-IRC over MMSE

- With MCS 17 ,1+1 rank combination and orthogonal precoder shows gain with MMSE-IRC over MMSE

- For 2+2 rank combination MCS 13/17/19 give reasonable results and show performance gain over MMSE

# Reference

- R4-2416555, “Way Forward for [112bis][320] NR_demod_Ph5_Part2_UE”, Nokia.