# 3GPP TSG-RAN WG4 Meeting #113	R4-2418567

# Orlando, US, 18th – 22nd November, 2024

Agenda item:	7.23.4.2

Source:	Apple

Title:	On RRM core requirements for LP-WUR

Document for:	Discussion

# Introduction

In last RAN4#112bis meeting, the WF for RRM requirements for LP-WUS/WUR was agreed in [1], and open issues have been summarized in [2]. Furthermore, some scopes of this WI have been confirmed in [3] from RAN #105 meeting.

It is proposed to endorse the following in RAN#105 for Rel-19 LP-WUS/WUR WI regarding FR2 support - Confirm in RAN#105 that Rel-19 LP-WUS work item scope includes both FR1 and FR2. - Request RAN1 to make a progress on the FFS “Whether the above is applicable to FR2” (under RAN1#117 agreement on LP-WUS and LP-SS bandwidth) in RAN1#118bis. - Reuse the FR1 design when possible - No additional guidance in RAN#105 for RAN4 work and TU allocation

In this contribution, we continue the discussion on the open issues in [1].

# General aspects

Issue 1-1-2: Measurement requirements to be specified for LP-WUR

- Check whether to update the first bullet of RAN4 110bis’s agreement on issue 2-2-1 as - Measurement requirements for [OOK-based] LP-WUR serving cell measurement based on LP-SS at Idle/Inactive state - Measurement requirements for OFDM-based LP-WUR serving cell measurement based on existing PSS/SSS at Idle/Inactive state

In last meeting RAN4 has no conclusion on the revision of the RAN4#110bis agreement, and at least the OOK based LP-WUR serving cell measurement shall be prioritized. RAN1 still didn’t conclude on the OFDM based LP-WUR to measure LP-SS, and we only found the following relevant working assumption in RAN1 agreements in RAN1#117 and #118bis.

Working Assumption From RAN1 perspective, for the RRM measurement metrics based on SSS for OFDM-based LP-WUR, use the same definition of SS-RSRP and SS-RSRQ for LP-SSS-RSRP and LP-SSS-RSRQ, respectively. - Above is applicable for both time-domain processing or frequency-domain processing - Above does not imply that RAN1 will introduce LP-SSS-RSRP and LP-SSS-RSRQ in the specifications Agreement Support overlaid OFDM sequence(s) for LP-SS: - LP-SS reuses the overlaid OFDM sequence(s) specified for LP-WUS. The design on overlaid OFDM sequence(s) specified for LP-WUS doesn’t target for sync and RRM measurement performance based on overlaid OFDM sequence for LP-SS. - Whether to transmit LP-SS by using a specified overlaid OFDM sequence is configurable. - Applicable at least for OOK-1 and FFS for OOK-4 - From RAN1 perspective, it is not intended to introduce new RAN4 requirements specific to overlaid sequences

RAN1 agreed to support overlaid OFDM sequence for LP-SS, but it doesn’t mean UE has to use OFDM receiver to receive LP-SS with overlaid OFDM sequence. In our view, we firstly need to list all the possible cases according to the RAN1 status so far:

- OFDM receiver based LR to receive SSB

- OFDM receiver based LR to receive LP-SS with/without overlaid ODFM sequence

- OOK receiver based LR to receive LP-SS with/without overlaid ODFM sequence

And as agreed in RAN1, agreeing the overlaid OFDM sequence for LP-SS does not mean to introduce new RAN4 requirements specific to overlaid sequences, and therefore it doesn’t mean RAN4 needs to have a new requirement for OFDM receiver based LR to receive LP-SS with overlaid ODFM sequence.

Proposal 1: At Rel-19 LP-WUR WI, for LP-WUR measurement, RAN4 specifies measurement requirements for the following:

- Measurement requirements for OOK-based LP-WUR serving cell measurement based on LP-SS with/without overlaid OFDM sequence at Idle/Inactive state

- Measurement requirements for OFDM-based LP-WUR serving cell measurement based on existing PSS/SSS at Idle/Inactive state

Proposal 2: same requirement shall be applied for OOK-based LP-WUR serving cell measurement based on LP-SS with and without overlaid OFDM sequence at Idle/Inactive state.

Issue 1-1-4: Threshold for switch between case 1 and case 3

| RRM measurement case index | MR serving cell measurement | MR neighboring cell measurement | LR measurement |
| --- | --- | --- | --- |
| #1 Fully offloading case | Off | Off | ON |
| #3 Relaxed case b | On with relaxation measurement | On with relaxation measurement | ON |

One thing to clarify that, LR measurement will be used for UE to decide switch from case#1 to case#3, that is, serving cell measurement on LR will be used to decide (1)if the neighbor cell measurement is triggered and (2)if the neighbor measurement can be relaxed. Previously in R16/17/18, the neighbor cell triggering and relaxation criteria checking is based on the MR measurement, and therefore it means LR measurement result shall be comparable to MR measurement or shall be equivalent to MR measurement with certain offset/margin (e.g., LR threshold is MR threshold + offset/margin).

Proposal 3: LR measurement can be used to check the criteria for neighbor cell measurement triggering/relaxation (in case #1).

Proposal 4: to support proposal 3, LR measurement result shall be comparable to MR measurement result or shall be equivalent to MR measurement result with certain offset/margin (e.g., LR threshold is MR threshold + offset/margin).

Issue 1-1-5: Criteria (entry/exit conditions) for fully offloading case and MR RRM measurement relaxation

Issue 1-1-6: Criteria (entry/exit conditions) for LP-WUS monitoring

In WID, another important enhancement is: Specify further RRM relaxation of UE MR for both serving and neighbor cell measurements, and UE serving cell RRM measurement offloaded from MR to LP-WUR, including the necessary conditions. Even though conditions for offloading RRM serving cell measurement from MR to LR and conditions for relaxation of MR serving cell and neighbor cell measurements is mainly decided/designed by RAN2, RAN4 can investigate the mobility performance corresponding to such relaxation, and the mismatch/bias between LP-WUR based measurement result and MR based measurement result. For instance, if LP-SS is used for the measurement to decide when the MR shall be waken up or not, such LP-SS based measurement result at least shall be comparable to the MR based measurement results or some bias offset shall be defined; otherwise, LP-SS triggers MR to wake up for mobility but MR based measurement results may tell that the mobility is not needed after MR wakes up.

In previous RAN2 meeting, there was also some discussion that,

RAN2#127: Serving cell measurements related For serving cell measurement offloading (i.e., serving cell measurement fully offloaded to LR and no serving cell measurement via MR is required), RAN2 should focus on specifying the offloading criterion for serving cell for UEs supporting LP-WUS, and assume that RAN4 will define the measurement offloading requirements for serving cell. Neighboring cell measurements related RAN2 understand that the RRM measurement of the neighboring cell can only be performed by MR. Can discuss again if RAN1 inform us otherwise. RAN2 will further discuss the neighbor cell measurement relaxation criteria (if the UE is using LR to measure the serving cell), e.g., considering reuse Rel-16 criteria for ‘not at cell edge’ and ‘low mobility’. RAN2#127bis: Working assumption - For neighbor cell measurement relaxation for UEs capable of LP-WUS, do not define additional MR-based criterion over the R16 criteria. RAN2 assume ‘UE not at cell edge’ is reused, FFS on ‘UE with low mobility’. - FFS (if needed) on enhancements based on R16 criteria (e.g., based on the LR measurements) for the case when MR serving cell measurement results are not available.

Like in power saving feature, the criteria itself has been defined be RAN2 in TS38.304, and therefore here we also think that RAN2 is the main group for criteria (entry/exit conditions) design. However, for RRM relaxation requirement (e.g., scaling factor), RAN4 can work on the RRM measurement relaxations (e.g., scaling factor) and offloading mechanisms based on the criteria defined by RAN2.

Proposal 5: Criteria (entry/exit conditions) for fully offloading case and MR RRM measurement relaxation can be left to RAN2 to decide.

Proposal 6: For the entry/exit condition for LP-WUS monitoring, follow RAN1/2 conclusions or can be triggered by other groups if necessary.

Issue 1-1-7: On jointly consideration on issue 1-1-5, 1-1-6 and 1-1-7

We think the case #1 is the case when UE has very good quality of serving cell, and UE can fully offload from MR to LR. But if the serving cell quality is not that good as case #1, UE may consider to use MR relaxation for serving and neighbor cell measurement. Case #2 is a little bit weird case to consider, since when the LR is ON and neighbor cell measurement is OFF on MR, it seems not necessary for UE to keep MR awake for a relaxed serving cell measurement. In our view the threshold to trigger corresponding offloading and relaxation can be as following figure,

Figure 1. threshold assumption for different cases

If the serving cell measurement shows above the threshold of neighbor cell measurement (or a new threshold of MR offloading), UE can turn off the MR, and only rely on LR to perform serving cell measurement, and no neighbor cell measurement is needed (including higher or equal/lower priority layers measurement). If the neighbor cell measurement is triggered (e.g., below the threshold of intra-frequency or inter-frequency measurement triggering condition) but serving cell quality is still above threshold of LP-WUS monitoring threshold, the MR can be ON with relaxation for serving and neighbor cell measurement while the LR is also ON for serving cell measurement, i.e., case #3. Then if UE is below LP-WUS monitoring threshold but above the not-at-cell-edge threshold for RRM relaxation, then UE will be in a legacy power saving mode, which means, LR is OFF and MR is ON with neighbor cell measurement relaxation (no serving cell relaxation). Even further, if UE is below not-at-cell-edge threshold for RRM relaxation, it means UE is in legacy IDLE/Inactive mode status without any RRM relaxation.

To support the above logic, we have some assumptions for such threshold design, even though the final threshold decision shall be made by RAN2. Firstly, we assume the threshold for MR fully offloading to LR can be same as the threshold for neighbor cell measurement triggering (highest one between SIntraSearchP/SIntraSearchQ and SnonIntraSearchP/SnonIntraSearchQ), or at least offloading threshold can be not lower than threshold for neighbor cell measurement triggering.  Secondly, the threshold to turn on LR or to involve LR for measurement can be equivalent to the threshold of entry condition for LP-WUS monitoring, as LP-WUS monitoring will require the T/F tracking on LR, e.g., via LP-SS.

Proposal 7: assumption of thresholds for RAN4 discussion can be:

- the threshold for MR fully offloading to LR can be same as the threshold for neighbor cell measurement triggering (highest one between SIntraSearchP/SIntraSearchQ and SnonIntraSearchP/SnonIntraSearchQ), or at least offloading threshold can be not lower than threshold for neighbor cell measurement triggering.

- the threshold to turn on LR or to involve LR for measurement can be equivalent to the threshold of entry condition for LP-WUS monitoring.

Based on the illustration in figure 1, we believe the case#2 and case#4 can be removed at least at this stage, but of course we can revisit the cases if RAN1/2 has some conclusions conflicting with RAN4 assumptions. And the following case table can be assumed in RAN4:

| RRM measurement case index | MR serving cell measurement | MR neighboring cell measurement | LR measurement |
| --- | --- | --- | --- |
| #1 Fully offloading case | Off | Off | ON |
| #3 Relaxed case b | On with relaxation measurement | On with relaxation measurement | ON |

Proposal 8: at this stage, remove case#2 and case#4 from the last meeting WF, and only following cases are considered. RAN4 can revisit the cases if in the future RAN1/2 has any conclusion conflicting with RAN4 assumptions.

| RRM measurement case index | MR serving cell measurement | MR neighboring cell measurement | LR measurement |
| --- | --- | --- | --- |
| #1 Fully offloading case | Off | Off | ON |
| #3 Relaxed case b | On with relaxation measurement | On with relaxation measurement | ON |

Issue 1-1-9: LP-WUR status before entering LP-WUS monitoring or after exiting LP-WUS

RAN2 had a working assumption (can revisit if R1/R4 reached different conclusions) in last RAN2#127 meeting: If the entry/exit conditions are configured, besides MR-based thresholds, LP-WUS monitoring entry condition can also include LR-based thresholds. However, it’s still unclear if LR-based threshold is required to check by UE, i.e., whether or not UE is required to use LR measurement to check the LR-based threshold for LP-WUS monitoring entry.

In RAN2#127bis, it was agreed that,

- If NW configure thresholds for both MR and LR measurements, then the entry condition is met when all the measured results are above the configured threshold(s).

- The LPWUS monitoring exit condition does not include MR measurements.

In WID, it was stated that: Specify further RRM relaxation of UE MR for both serving and neighbor cell measurements, and UE serving cell RRM measurement offloaded from MR to LP-WUR, including the necessary conditions. As we discussed, UE may or may not turn off the MR for serving/neighbor cell measurement, and therefore, two cases can be considered, i.e., case#1 and case #3. However, based on RAN2’s agreement, network may configure both MR and LR measurement to UE when UE is in legacy measurement status, and then UE shall turn on the LR for LR measurement to check with the entry condition. Thus, before entering LP-WUS monitoring or after exiting LP-WUS monitoring, the state of UE also needs to be clarified:

Alt 1: MR is ON with RRM measurement on serving cell and neighbour cell (if any) and LP-WUR is ON for serving cell measurement

Alt 2 (to reflect RAN2’s agreement): MR is ON with RRM measurement on serving cell and neighbour cell (if any) and LP-WUR is ON for serving cell measurement, but when and how to turn on LR for serving cell measurement is up to UE implementation.

We can accept Alt 2 understanding based on RAN2’s agreement, but we prefer to leave when and how to turn on LR to UE implementation.

Proposal 9: before entering LP-WUS monitoring or after exiting LP-WUS monitoring, the state of UE also needs to be clarified:

- MR is ON with RRM measurement on serving cell and neighbour cell (if any) and LP-WUR is ON for serving cell measurement, but when and how to turn on LR for serving cell measurement is up to UE implementation.

Issue 1-1-10: Considerations on higher priority frequency layer

In case #1, when UE fully offload from MR to LR, we assume measurement results from LP-WUR will decide whether the MR shall be waken up or not, however, the MR will be turned off or in ultra-deep sleep mode. The FFS part is mainly about whether the higher priority measurement shall still be kept on MR or not. Current higher priority measurement in RAN4 has two main delays: 1min and 1 hour, as followings.

In order to save the power of UE, if MR still needs to wake up every 1min, we didn’t see much gain for this feature, but we are open to discuss if higher priority measurement on MR can be kept and performed as slow as 1 hour per layer. For simplicity of this feature at UE, we prefer to not consider higher priority layer measurement on MR when the MR is fully offloaded. Thus, the MR neighbor cell measurement shall not be considered for case #1.

But we are still open to discuss the condition for higher priority layer measurement in case#1 if majority companies would like to support it.

Proposal 10: higher priority layer neighbor cell measurement shall also be OFF for case #1.

Issue 1-1-12: RRM requirements for FR2

In the past RAN#105 plenary meeting, the FR2 LP-WUS scope has been discussed and agreed in [RP‑242361 Way forward on LP-WUS for FR2] as following:

It is proposed to endorse the following in RAN#105 for Rel-19 LP-WUS/WUR WI regarding FR2 support - Confirm in RAN#105 that Rel-19 LP-WUS work item scope includes both FR1 and FR2. - Request RAN1 to make a progress on the FFS “Whether the above is applicable to FR2” (under RAN1#117 agreement on LP-WUS and LP-SS bandwidth) in RAN1#118bis. - Reuse the FR1 design when possible - No additional guidance in RAN#105 for RAN4 work and TU allocation

Since the FR2 is confirmed as a scope for LP-WUS WI, RAN4 needs to discuss (1) whether we need to design requirement for FR2 LP-WUR based RRM, and (2) when to start the discussion for such requirement design. Our understanding is FR2 LP-WUR based RRM requirement can be discussed after we concluded on the FR1 LP-WUR based RRM requirement, since FR2 case would be more complicated than FR1 by considering the Rx beam sweeping.

In RAN4 Idle/inactive mode requirement for SSB, the Rx beam sweeping has been applied for serving cell measurement and evaluation as following examples:

In LP-SS based serving cell measurement, UE will follow the LP-SS periodicity to do the measurement instead of DRX in legacy case, then we will probably use the different sweeping factor for different LP-SS periodicity; and furthermore if LP-SS has multiple repetitions inside each LP-SS occasion, UE may perform Rx beam sweeping on these repetitions and save the N1 for LP-SS occasion measurement. The most straightforward requirement could be that: the different Rx beam sweeping factor(N1) shall be applied for different LP-SS periodicities, and the N1 shall be decreased when LP-SS periodicity increases. However, since the LP-SS design is still on-going in RAN1, RAN4 can discuss the detailed requirement in future meetings.

Proposal 11: FR2 LP-WUR based RRM requirement can be discussed after we concluded on the FR1 LP-WUR based RRM requirement.

Issue 1-1-14: Impact on specification

We think this issue can be discussed after when the whole mechanism of offloading, LP-SS/LP-WUS design and measurement metrics are concluded.

Proposal 12: this issue can be discussed after when the whole mechanism of offloading, LP-SS/LP-WUS design and measurement metrics are concluded.

# Detail LP-WUR requirements at RRC_IDLE/INACTIVE state

Issue 1-2-1: Accuracy requirements

Regarding the accuracy requirement, if we follow the legacy accuracy requirement principle, the dedicated accuracy requirements in TS38.133 section 10 is mainly for the measurement results which is reported to network, and the only measurement accuracy requirement for IDLE/Inactive mode is the EMR feature related measurement. For LR based RRM measurement, since UE will not report anything back to network in IDLE/Inactive mode, our preference is still not to define a dedicated accuracy requirement in TS38.133 section 10, but instead we can reflect such accuracy performance in the core requirement sections as a margin, like in the following example:

Proposal 13: For RAN4 requirement of LR based RRM measurement in Idle/inactive states, no dedicated accuracy requirement is defined in the performance section, and reflect the accuracy performance as a margin in the core requirement.

Issue 1-2-2: On requirements for entry/exit criteria evaluation for WUS paging monitoring/LP-WUR measurement/MR RRM relaxation

We are open to discuss the requirement for entry/exit criteria evaluation for WUS paging monitoring/LP-WUR measurement/MR RRM relaxation, but depending on different cases, UE may use MR measurement, or LR measurement, or MR+LR measurement to check the criteria.

Regarding the time period for UE to meet the criteria, RAN1 had a working assumption as following:

Working Assumption From RAN1 perspective, for the entry/exit conditions for LP-WUS monitoring in IDLE/inactive mode, - The UE may start LP-WUS monitoring if - the serving cell measurement performed by the MR is above entry threshold(s), if configured by the gNB, and/or - FFS other conditions, and if any, whether all or one or some of the conditions need to be satisfied

Similar as the criteria checking for power saving feature, TS38.304 specified the non-at-cell-edge condition as following:

5.2.4.9.2 Relaxed measurement criterion for UE not at cell edge The relaxed measurement criterion for UE not at cell edge is fulfilled when: - Srxlev > SSearchThresholdP, and, - Squal > SSearchThresholdQ, if SSearchThresholdQ is configured, Where: - Srxlev = current Srxlev value of the serving cell (dB). - Squal = current Squal value of the serving cell (dB).

In our view, for this entry/exit criteria checking for WUS paging monitoring/LP-WUR measurement/MR RRM relaxation, we didn’t see strong motivation to have a new evaluation requirement, but we can just use the MR measurement period or LR measurement period to check the criteria for WUS paging monitoring/LP-WUR measurement/MR RRM relaxation., i.e., MR measurement delay or LR measurement delay are sufficient for criteria checking; and do not need a dedicated evaluation delay requirement (e.g., in how long time the MR/LR measurement results can meet the condition).

Proposal 14: to check the criteria for WUS paging monitoring/LP-WUR measurement/MR RRM relaxation,

- use the MR measurement period and/or LR measurement period, i.e., MR measurement delay or LR measurement delay are sufficient for criteria checking; and do not need a dedicated evaluation delay requirement (e.g., in how long time the MR/LR measurement results can meet the condition).

However, for the fully offloading threshold checking, we had some discussion in last meeting that the MR will enter an ultra/deep sleep after the offloading threshold is met, and therefore in this case we need an evaluation period to make sure there is no ping-pong effect for UE to turn off or turn on the MR.

Proposal 15: to check the criteria for MR fully offloading,

- use the MR measurement period and/or LR measurement period for measurement filtering;

- and need a dedicated evaluation delay requirement (i.e., in how long time the MR/LR measurement results can meet the criteria).

Issue 1-2-2-2: LR measurement metrics used for entry/exit threshold evaluation for LP-WUS monitoring/RRM relaxation

In previous RAN2 meeting, it was agreed that,

### Procedure and configuration of LP-WUS in RRC_IDLE/INACTIVE - Baseline: The network does not need to be aware of whether the UE is monitoring LP-WUS or not in RRC_IDLE/INACTIVE - Separate entry/exit thresholds can be configured for OFDM-based and OOK-based WUR if a cell supports both types of LRs. Signalling details are FFS. - Working assumption (can revisit if R1/R4 reached different conclusions): If the entry/exit conditions are configured, besides MR-based thresholds, LP-WUS monitoring entry condition can also include LR-based thresholds. - The metrics for serving cell quality measured by MR/LR for entry condition includes (LP-)RSRP and optional (LP-)RSRQ. - The metrics for serving cell quality measured by LR for exit condition includes (LP-)RSRP and optional (LP-)RSRQ.

Regarding issue 1-2-2-2, it’s clear that RAN2 had some conclusions already and still working on the threshold design. We recommend to just follow the RAN2 working assumption and agreements, and no need to discuss it in RAN4.

Proposal 16: For LP-WUR, LR measurement metrics have (LP-)RSRP and optional (LP-)RSRQ.

Issue 1-2-3: On interruption related requirements for LP-WUR

Issue 1-2-3: On interruption related requirements for LP-WUR - Proposals - P1: Interruption requirements could be defined for the duration from LP-WUS received paging indication to MR ready for paging monitoring (CMCC vivo Samsung Docomo CT) - P2: Interruption requirements may need to be specified for the following:(CMCC) - Between LP-WUR measurement result fulfills exit criteria and MR to start measurement - Between MR measurement result fulfills entry criteria and LP-WUR to start measurement - P3: RAN4 only needs to define the wake-up delay requirement, if necessary, where the wake-up delay means minimum gap time between LP-WUS reception and MR to start PDCCH monitoring (Apple) - P4: Wait for more RAN1 progress on PO monitoring before discussing the possible interruption requirements related to LR (Huawei) - P5: RRM requirements for interruption should be based on the RAN1 agreements for wake-up delay (NTT Docomo) - P6: RAN4 shall clarify the timeline from decoding LP-WUS to MR wake-up in order to monitor legacy PO. (ZTE)

In last RAN1 #118 and #118bis meeting, the following agreement for wake-up delay has been achieved,

Agreement For the wake-up delay, consider the following options: Option 2: UE reports one value from X candidate values for the wake-up delay via UE capability reporting. - FFS: X is 2, 3, 4 Above applies for IDLE/INACTIVE mode. Definition of wake-up delay: Minimum gap time between LP-WUS reception and MR to start PDCCH monitoring Agreement For the offset value(s) between an LO and a reference PO/PF, consider the following options: - Definition: The gap between an LO and a PO is considered to be no less than the wake-up delay a UE supports if the gap between the end of the last LP-WUS MO the UE monitors in the LO and the start of the PO is no less than the wake-up delay.

The definition of wake-up delay is: Minimum gap time between LP-WUS reception and MR to start PDCCH monitoring, that is same as what RAN4 discussed in last meeting, i.e., interruption requirement in P1 above. The gap between an LO and a PO is considered to be no less than the wake-up delay a UE supports if the gap between the end of the last LP-WUS MO the UE monitors in the LO and the start of the PO is no less than the wake-up delay. Network needs to make sure the gap between LO for UE to monitor and PO shall be NO less than this wake-up delay, and that means it will not have any interruption to the PO reception due to the wake-up delay.

In short, we think such interruption requirement has already been reflected by UE capability indication as agreed in RAN1, thus no dedicated interruption requirement needs to be defined for this case. RAN4 only needs to define the wake-up delay requirement as defined in RAN1, if such requirement is necessary.

Proposal 17: RAN4 only needs to define the wake-up delay requirement, if such requirement is necessary from majority companies’ view, where the wake-up delay means minimum gap time between LP-WUS reception and MR to start PDCCH monitoring.

Issue 1-2-4: On LP-WUR cell selection evaluation requirements for case 1

As illustrated in our figure 1, we think the typical case is the S criterion threshold is lower than the MR offloading threshold and neighbor cell measurement triggering threshold, and therefore we don’t need to discuss the LR based S criterion evaluation in case#1.

Proposal 18: No need to discuss the LR based S criterion evaluation in case#1.

Issue 1-2-6: LP-SS based RRM requirements periodicity and filtering

Agreement: Measurement delay requirements for LP-WUR based on LP-SS in IDLE/Inactive mode shall be defined based on LP-SS periodicity. Measurement delay requirements for LP-WUR based on SSB in IDLE/Inactive mode shall be based on: - ■Option 1: Based on DRX - ■Option 2: follow the same periodicity of LP-SS if it is available ■Option 3: A fix value (like 320ms), FFS on the value

In previous meeting, RAN1 also discussed the LP-WUS in eDRX, but no conclusion has been made. In our view, besides LP-WUS, we also need to investigate the UE RRM measurement for LP-SS in eDRX, e.g., whether LP-SS measurement shall follow the eDRX/DRX cycle or not. If the LP-SS based measurement is not following eDRX cycle or DRX cycle in IDLE/Inactive mode, i.e., LP-SS based measurement will follow the LP-SS periodicity, the RRM requirement shall only be defined based on the LP-SS periodicity. However, if the LR is measuring the SSB, it’s unclear so far which periodicity UE shall follow. RAN1 had some agreements in last meeting that,

Agreement For LP-SS periodicity, support at least 320ms - FFS: 80ms,160ms, 640ms,1280ms, 2560ms, 5120ms, 10240ms Agreement Confirm the following working assumption for iDRX: For each UE, the periodicity of LO is the same as its iDRX cycle. Agreement For the mapping between LO and PO, supports at least Option 1 (UEs monitoring the same PO monitor the same LO). Agreement From RAN1 perspective, when a UE is monitoring LP-WUS (based on the entry/exit condition for LP-WUS), a UE is not required to monitor a PO if - it does not detect a LP-WUS on the monitored LO - or the LP-WUS does not indicate a wake-up indication for the UE’s corresponding subgroup

If LP-SS periodicity is configured but UE is using LR to measure SSB, the measurement interval shall be comparable the LO periodicity, since UE needs the T/F tracking information from SSB measurement to monitor/receive LO; i.e., if the SSB measurement periodicity is too much bigger than LO periodicity, UE may be not able to correctly receive the LO. On the other hand, we also think the LR based SSB measurement shall have similar power saving gain as LR based LP-SS measurement, and therefore the LP-SS periodicity shall also be considered in this case. The LP-SS periodicity is not decided yet in RAN1, e.g., LP-SS periodicity might be smaller than 320ms. Thus, we propose to use max(LP-SS periodicity, DRX cycle) or max(LP-SS periodicity, LO periodicity), where periodicity of LO is the same as DRX cycle.

If the LP-SS periodicity is not configured and UE is using LR to measure SSB, the LO periodicity could be the factor to consider this measurement interval. Thus, we propose to use DRX cycle or LO periodicity for SSB measurement periodicity.

Proposal 19: Measurement delay requirements for LP-WUR based on SSB in IDLE/Inactive mode shall be based on:

- if LP-SS periodicity is configured, use max(LP-SS periodicity, DRX cycle) or max(LP-SS periodicity, LO periodicity)

- if LP-SS periodicity is not configured, use DRX cycle or LO periodicity

- where periodicity of LO is the same as DRX cycle based on RAN1 agreement.

# MR RRM relaxation

Issue 1-3-1: MR RRM relaxation for serving cell/neighbour cell

In the WID, the MR relaxation scope is as following:

Specify further RRM relaxation of UE MR for both serving and neighbor cell measurements, and UE serving cell RRM measurement offloaded from MR to LP-WUR, including the necessary conditions

In the power saving feature, the RRM relaxation for IDLE and connected mode is only for neighbor cell measurement, and no relaxation is allowed for serving cell measurement. The neighbor cell measurement relaxation is decided by serving cell measurement results. RAN4 needs to carefully check if both neighbor cell measurement and serving cell measurement shall share the same relaxation criteria or relaxation requirement (e.g., scaling factor). In our view, if the neighbor cell measurement relaxation is based on the serving cell measurement, it implies that neighbor cell measurement shall always have the more relax measurement than serving cell measurement. The rationale is also obvious: only if UE can get reliable serving cell measurement results, then it can use such results to decide whether neighbor cell measurement can be relaxed or not.

Proposal 20: regarding RRM relaxation of UE MR for both serving and neighbor cell measurements, neighbor cell measurement shall have the more relaxation than or equivalent relaxation as serving cell measurement.

As proposed by companies, we think 16 could be used as starting point for relaxation, and based on proposal 13, we would like to propose 16 for serving cell measurement relaxation and neighbor cell measurement relaxation.

Proposal 21: regarding RRM relaxation of UE MR for both serving and neighbor cell measurements, consider to use 16 for serving cell measurement relaxation and neighbor cell measurement relaxation .

# LP-WUR CONNECTED mode

Issue 1-4-1: LP-WUR at CONNECTED mode

In the WID, the scope for connected mode is to discuss the procedures to allow UE MR PDCCH monitoring triggered by LP-WUS including activation and deactivation procedure of LP-WUS monitoring, however, like in R16 WUS design, it would not have any RRM impact by using LP-WUS in connected mode. Furthermore, WID mentioned that,

In CONNECTED mode, UE MR ultra-deep sleep is not considered, and UE RRM/RLM/BFD/CSI measurements are performed by MR

All the RRM/RLM/BFD/CSI measurement would be as same as legacy case, i.e., use MR to perform these functionalities. Thus, no RRM scope shall be needed for connected mode in this WI.

Proposal 22: no RRM objectives is needed for connected mode in this LP-WUR/LP-WUS WI.

# Others

Issue 1-5-2: LR based RRM with EMR in IDLE/inactive mode

In RAN4 #111 and 112 meeting, the different cases for MR offloading and relaxation are captured in the WF R4-2410296 and R4-2413899, and so far, at least case#1 and case#3 are valid case (like in proposal 13) to consider in this WI,

| RRM measurement case index | MR serving cell measurement | MR neighboring cell measurement | LR measurement |
| --- | --- | --- | --- |
| #1 Fully offloading case | Off | Off | ON |
| #3 Relaxed case b | On with relaxation measurement | On with relaxation measurement | ON |

However, the EMR measurement has not been considered yet for both case #1 and case #3. In legacy IDLE/Inactive mode RRM requirement, the relaxation is not allowed when EMR is configured on that frequency layer and T331 is running, like followings:

Then, if EMR measurement is also configured and TS331 is running for case#1 and case#3, the MR status may need further discussion. For a UE which supports idleInactiveNR-MeasReport-r16 or idleInactiveEUTRA-MeasReport-r16, and serving cell configures carriers for idle mode CA/DC measurement reporting with T331 running, 	UE may keep the MR ON for EMR measurement regardless of the MR offloading condition is met or not.

Proposal 23: For a UE which supports idleInactiveNR-MeasReport-r16 or idleInactiveEUTRA-MeasReport-r16, and serving cell configures carriers for idle mode CA/DC measurement reporting with T331 running, UE shall keep the MR ON for EMR measurement regardless of the MR offloading condition is met or not.

# Conclusion

In this contribution, we continue the discussion on the open issues in [2].

Proposal 1: At Rel-19 LP-WUR WI, for LP-WUR measurement, RAN4 specifies measurement requirements for the following:

- Measurement requirements for OOK-based LP-WUR serving cell measurement based on LP-SS with/without overlaid OFDM sequence at Idle/Inactive state

- Measurement requirements for OFDM-based LP-WUR serving cell measurement based on existing PSS/SSS at Idle/Inactive state

Proposal 2: same requirement shall be applied for OOK-based LP-WUR serving cell measurement based on LP-SS with and without overlaid OFDM sequence at Idle/Inactive state.

Proposal 3: LR measurement can be used to check the criteria for neighbor cell measurement triggering/relaxation (in case #1).

Proposal 4: to support proposal 3, LR measurement result shall be comparable to MR measurement result or shall be equivalent to MR measurement result with certain offset/margin (e.g., LR threshold is MR threshold + offset/margin).

Proposal 5: Criteria (entry/exit conditions) for fully offloading case and MR RRM measurement relaxation can be left to RAN2 to decide.

Proposal 6: For the entry/exit condition for LP-WUS monitoring, follow RAN1/2 conclusions or can be triggered by other groups if necessary.

Proposal 7: assumption of thresholds for RAN4 discussion can be:

- the threshold for MR fully offloading to LR can be same as the threshold for neighbor cell measurement triggering (highest one between SIntraSearchP/SIntraSearchQ and SnonIntraSearchP/SnonIntraSearchQ), or at least offloading threshold can be not lower than threshold for neighbor cell measurement triggering.

- the threshold to turn on LR or to involve LR for measurement can be equivalent to the threshold of entry condition for LP-WUS monitoring.

Proposal 8: at this stage, remove case#2 and case#4 from the last meeting WF, and only following cases are considered. RAN4 can revisit the cases if in the future RAN1/2 has any conclusion conflicting with RAN4 assumptions.

| RRM measurement case index | MR serving cell measurement | MR neighboring cell measurement | LR measurement |
| --- | --- | --- | --- |
| #1 Fully offloading case | Off | Off | ON |
| #3 Relaxed case b | On with relaxation measurement | On with relaxation measurement | ON |

Proposal 9: before entering LP-WUS monitoring or after exiting LP-WUS monitoring, the state of UE also needs to be clarified:

- MR is ON with RRM measurement on serving cell and neighbour cell (if any) and LP-WUR is ON for serving cell measurement, but when and how to turn on LR for serving cell measurement is up to UE implementation.

Proposal 10: higher priority layer neighbor cell measurement shall also be OFF for case #1.

Proposal 11: FR2 LP-WUR based RRM requirement can be discussed after we concluded on the FR1 LP-WUR based RRM requirement.

Proposal 12: this issue 1-1-14 can be discussed after when the whole mechanism of offloading, LP-SS/LP-WUS design and measurement metrics are concluded.

Proposal 13: For RAN4 requirement of LR based RRM measurement in Idle/inactive states, no dedicated accuracy requirement is defined in the performance section, and reflect the accuracy performance as a margin in the core requirement.

Proposal 14: to check the criteria for WUS paging monitoring/LP-WUR measurement/MR RRM relaxation,

- use the MR measurement period and/or LR measurement period, i.e., MR measurement delay or LR measurement delay are sufficient for criteria checking; and do not need a dedicated evaluation delay requirement (e.g., in how long time the MR/LR measurement results can meet the condition).

Proposal 15: to check the criteria for MR fully offloading,

- use the MR measurement period and/or LR measurement period for measurement filtering;

- and need a dedicated evaluation delay requirement (i.e., in how long time the MR/LR measurement results can meet the criteria).

Proposal 16: For LP-WUR, LR measurement metrics have (LP-)RSRP and optional (LP-)RSRQ.

Proposal 17: RAN4 only needs to define the wake-up delay requirement, if such requirement is necessary from majority companies’ view, where the wake-up delay means minimum gap time between LP-WUS reception and MR to start PDCCH monitoring.

Proposal 18: No need to discuss the LR based S criterion evaluation in case#1.

Proposal 19: Measurement delay requirements for LP-WUR based on SSB in IDLE/Inactive mode shall be based on:

- if LP-SS periodicity is configured, use max(LP-SS periodicity, DRX cycle) or max(LP-SS periodicity, LO periodicity)

- if LP-SS periodicity is not configured, use DRX cycle or LO periodicity

- where periodicity of LO is the same as DRX cycle based on RAN1 agreement.

Proposal 20: regarding RRM relaxation of UE MR for both serving and neighbor cell measurements, neighbor cell measurement shall have the more relaxation than or equivalent relaxation as serving cell measurement.

Proposal 21: regarding RRM relaxation of UE MR for both serving and neighbor cell measurements, consider to use 16 for serving cell measurement relaxation and neighbor cell measurement relaxation .

Proposal 22: no RRM objectives is needed for connected mode in this LP-WUR/LP-WUS WI.

Proposal 23: For a UE which supports idleInactiveNR-MeasReport-r16 or idleInactiveEUTRA-MeasReport-r16, and serving cell configures carriers for idle mode CA/DC measurement reporting with T331 running, UE shall keep the MR ON for EMR measurement regardless of the MR offloading condition is met or not.

# References

[1] R4-2416861, WF on RRM requirements for NR_LPWUS, Moderator (vivo), RAN4 #112bis

[2] R4-2415664, Topic summary for [112bis][217] NR_LPWUS, vivo, RAN4 #112bis

[3] RP-242361, Way forward on LPWUS for FR2, vivo, RAN #105