# 3GPP TSG-RAN WG4 Meeting #113	R4-2418568

# Orlando, US, 18th – 22nd November, 2024

Agenda item:	7.26.4.1

Source:	Apple

Title:	On R19 NTN (e)RedCap RRM requirements

Document for:	Discussion

# Introduction

The WF on RRM requirements for Rel-19 NR NTN phase3 [1] has been agreed in RAN4#112, and the open issues have been summarized in the topic summary for [112][226] NR_NTN_Ph3 [2].

In this contribution, we continue the discussion on the RRM requirement for R19 NTN (e)RedCap.

# RRM for support of Rel-17 RedCap and Rel-18 eRedCap UEs with NR NTN

Issue 2-2-1: Further discussion on cell reselection requirement for (e)RedCap UE with FR1-NTN

<Agreement>: - The existing cell reselection requirements in RRC_IDLE/RRC_INACTIVE state defined for Rel-17 and Rel-18 (e)NTN can be the baseline for (e)RedCap 2Rx UE with FR1-NTN bands. - Both time-based and location-based cell reselection measurement criteria will be defined. - For both earth fixed and earth moving scenario - Both NTN-NTN and NTN-TN (from NTN to TN and from TN to NTN) cell reselection requirements will be defined. - FFS for the measurement requirements for inter-RAT (NTN to E-UTRA TN) measurement.

In Rel-17 NTN, RAN4 has cell reselection requirement for NTN to LTE TN (inter-RAT E-UTRAN cells with TN carrier). That means: from the network deployment perspective, it’s a feasible scenario to consider. For RedCap requirement in R17, RAN4 also has requirement for measurements of inter-RAT E-UTRAN cells for RedCap UE. Thus, we think it’s necessary to have the measurement requirements for inter-RAT (NTN to E-UTRA TN) measurement for (e)RedCap UE, and the existing cell reselection requirements in RRC_IDLE/RRC_INACTIVE state defined for Rel-17 and Rel-18 (e)NTN can be the baseline for (e)RedCap 2Rx UE with FR1-NTN bands.

Proposal 1: RAN4 to define measurement requirements for inter-RAT (NTN to E-UTRAN TN) measurement for (e)RedCap UE. The existing requirements for measurements of inter-RAT E-UTRAN cells with TN carrier can be used as baseline.

Issue 2-3-2: eDRX configuration

<Agreement>: - For GSO: - Define eDRX related requirements in RRC_Idle / RRC_Inactive mode for (e)RedCap UE with FR1-NTN. - For GSO, All DRX and eDRX cycle can be supported, and the DRX and eDRX related requirements for RedCap UE in TN are re-used. - Further discuss for LEO.

RAN4 needs more discussion on whether new requirement shall be introduced for RRM relaxation and eDRX. For example, in legacy NTN requirement, the 2.56s DRX is not assumed to be well utilized in the scenario where earth-moving LEO is deployed as the target satellite or serving satellite for measurement. The main reason is: if earth-moving LEO is used and the footprint on earth is not that large, e.g., couple kilometers, then the longer measurement delay due to using 2.56s will fail the mobility. If eDRX>10.24sec is considered/configured (PTW window is used) and earth-moving LEO is serving cell or is configured as target satellite for UE’s measurement, we think if DRX in PTW window is not greater than1.28s, UE shall complete the target NTN satellite measurement/evaluation within single PTW window, i.e., no cross PTW window measurement/evaluation; however, if DRX in PTW window is greater than 1.28s, we still think UE shall be allowed to skip or drop the measurement/detection/evaluation, i.e., UE is not required to meet the requirement for 2.56s DRX cycle length for earth moving LEO deployment when eDRX with PTW window is configured.

However, if the eDRX is without PTW(eDRX <=10.24s) and UE follows the eDRX cycle to perform the measurement, the minimum eDRX cycle is 2.56s and it’s equivalent to the note 2 in the legacy NTN requirement. Thus, we think UE is not required to meet the requirement with eDRX without PTW for earth moving LEO deployment. In order to simplify the requirement, we propose that UE is not required to meet the RRM requirement with eDRX for earth moving LEO deployment.

Proposal 2: UE is not required to meet the RRM requirement with eDRX for earth moving LEO deployment.

# Conclusion

In this contribution, we discuss the RRM requirement scope for WI of Non-Terrestrial Networks (NTN) for NR Phase 3.

Proposal 1: RAN4 to define measurement requirements for inter-RAT (NTN to E-UTRAN TN) measurement for (e)RedCap UE. The existing requirements for measurements of inter-RAT E-UTRAN cells with TN carrier can be used as baseline.

Proposal 2: UE is not required to meet the RRM requirement with eDRX for earth moving LEO deployment.

# References

[1] R4-2416865, WF on RRM requirements for NR_NTN_Ph3_Part1, CATT, RAN4#112bis

[2] R4-2415668, Topic summary for [112bis][221] NR_NTN_Ph3_Part, CATT, RAN4#112bis