# 3GPP TSG-RAN WG4 Meeting #113	R4-2418569

# Orlando, US, 18th – 22nd November, 2024

Agenda item:	7.26.4.2

Source:	Apple

Title:	On R19 other NTN RRM requirements

Document for:	Discussion

# Introduction

The WF on RRM requirements for Rel-19 NR NTN phase3 [1] has been agreed in RAN4#112, and the open issues have been summarized in the topic summary for [112][226] NR_NTN_Ph3 [2].

In this contribution, we continue the discussion on the other RRM requirements for R19 NTN.

# RRM for downlink coverage enhancements

### Issue 1-1: SSB periodicity enhancement

Agreement: - RAN4 to wait for RAN1’s definitive decision on the maximum periodicity of SSB and the range of SMTC before discussing whether and how to modify the relevant RRM requirements. FFS on the detailed list of RRM requirements to be affected, e.g., - uplink timing requirements, RRM requirements during transmission between different SSB periodicities, default assumption on the SSB and SMTC periodicities in handover/satellite switch with re-sync/RRC re-establishment/RRC connection release with redirection, cell identification, measurements, etc.

In last RAN1 and RAN2 meetings, some agreements for DL coverage have been made as followings:

Agreement in RAN1 #118bis For NR NTN, support extended periodicity of the half frames with SS/PBCH blocks assumed by UE during initial access. - The maximum of the additional default value (apart from the existing 20ms value) is at least 160 ms. - FFS: whether 320ms can be supported as the maximum of the additional default value instead of or in addition to 160ms Agreement in RAN2 #127bis 1. If it turns out that there is a need to bar UEs not supporting DL-CE, Rel-19 UEs not supporting DL-CE can be barred from accessing a cell operating with DL-CE using the existing NTN bar bit, in the same way as pre-Rel19 NTN UEs (this is an extension of the previous agreement to include also Rel-19 UE not supporting DL-CE) 2. If it turns out that there is a need to bar UEs not supporting DL-CE, then we need to introduce a barring mechanism to control access of UEs supporting Rel19 NTN DL-CE. FFS on the details. (This also implies that UEs supporting Rel19 NTN DL-CE will not consider the existing NTN barring bit) 3. (also depending on the details of the RAN1 solution) we can further consider methods to allow UEs not supporting DL CE to down-prioritize or prevent re-selection to cells operating with DL CE.

The SSB periodicity extension for DL coverage enhancement can probably impact UE initial access procedure compared with the legacy assumption of 20ms. RAN4 may need to double check if any RRM requirement shall be clarified when the SSB periodicity is changed to a value different from 20ms.

In the current initial access assumption for cell selection, it was defined in RAN1 that,

In the current NTN network (R17, and R18), by default NTN UE will try the initial access to network assuming SSB periodicity as 20ms, however, if the R19 NTN network changes the SSB periodicity to be more than 20ms, it will cause the initial access failure for the R17/R18 NTN UEs. Even for the R19 NTN UE, it has no clear idea if the network behavior is following R17/R18 or R19, and the UE behavior is unclear for implementation during the initial access.

Besides the initial access, there are other UE behaviors requiring assumption of the by-default SSB periodicity in the previous NTN networks (R17/R18). Some examples as followings:

    - For RACH based HO, CHO,  RACH-less HO and satellite switch with re-sync, if the UE is not provided SMTC configuration or measurement object on this frequency, the requirement in this clause is applied with Trs=5ms assuming the SSB transmission periodicity is 5ms. There is no requirement if the SSB transmission periodicity is not 5ms.

    - For RRC Re-establishment for Satellite Access, the definition of TSMTC,i is the periodicity of the SMTC occasion configured for the inter-frequency carrier i. If it is not configured, the UE may assume that the target SSB periodicity is no larger than 20 ms.

    - For RRC connection release with redirection to NR, if the UE is not provided with SMTC configuration or measurement object for the frequency which is also configured for the RRC connection release with redirection then: the requirement in this clause is applied with Trs = 20 ms if the SSB transmission periodicity is not larger than 20 ms; otherwise, there is no requirement if the SSB transmission periodicity is larger than 20ms.

With the SSB periodicity change(e.g., change to a SSB periodicity > 20ms), the above corresponding requirement needs to be modified.

Proposal 1: With the SSB periodicity change for DL coverage enhancement, the following assumptions in RRM requirements need to be modified:

    - Trs assumption for RACH based HO, CHO, RACH-less HO and satellite switch with re-sync, if the UE is not provided SMTC configuration or measurement object on this frequency.

    - TSMTC,i assumption for RRC Re-establishment for Satellite Access, if it is not configured.

    - Trs assumption for RRC connection release with redirection to NR, if the UE is not provided with SMTC configuration or measurement object for the frequency which is also configured for the RRC connection release with redirection.

If the SSB periodicity is selected to be greater than 160ms, many of the RRM requirements need to be revisited, we identified two key categories: (1) SMTC/MG periodicity; (2) UE timing requirement.

(1)SMTC periodicity and MGRP

The current upper bound of SMTC periodicity and MGRP is 160ms, and thus they might need to be modified if the SSB periodicity can be 320ms. However, it shall be decided after RAN1 has conclusion on whether 320ms SSB periodicity can be adopted or not for DL coverage enhancement. Accordingly, for instance, if MGRP is extended to 320ms, RAN4 needs to discuss how to modify the MG patterns table in section 9.1.2 of TS38.133.

(2)UE timing requirement

In current NTN timing requirement, it’s specified that the UE shall meet the Te_NTN requirement for an initial transmission provided that at least one SSB is available at the UE during the last 160 ms. However, if SSB periodicity is chosen to be greater than 160ms, UE may not be able to get an available SSB in the last 160ms before the UL transmission, as shown in following figure.

Figure 1. UE may not get SSB in last 160ms for Tx timing decision

In our view, TRS is always a reliable way for UE to acquire the DL serving cell timing if the SSB is not available. Thus, RAN4 may consider to have the requirement as: The UE shall meet the Te_NTN requirement for an initial transmission provided that at least one SSB or TRS is available at the UE during the last 160 ms.

Proposal 2: If SSB periodicity is 320ms for DL coverage enhancement, RAN4 needs to revisit the UE timing requirement.

Proposal 3: RAN4 needs to check whether requirement change is needed for cell identification/measurement and MG patterns, if the SMTC periodicity and MGRP is extended to 320ms.

# Conclusion

In this contribution, we discuss the RRM requirement scope for WI of Non-Terrestrial Networks (NTN) for NR Phase 3.

Proposal 1: With the SSB periodicity change for DL coverage enhancement, the following assumptions in RRM requirements need to be modified:

    - Trs assumption for RACH based HO, CHO, RACH-less HO and satellite switch with re-sync, if the UE is not provided SMTC configuration or measurement object on this frequency.

    - TSMTC,i assumption for RRC Re-establishment for Satellite Access, if it is not configured.

    - Trs assumption for RRC connection release with redirection to NR, if the UE is not provided with SMTC configuration or measurement object for the frequency which is also configured for the RRC connection release with redirection.

Proposal 2: If SSB periodicity is 320ms for DL coverage enhancement, RAN4 needs to revisit the UE timing requirement.

Proposal 3: RAN4 needs to check whether requirement change is needed for cell identification/measurement and MG patterns, if the SMTC periodicity and MGRP is extended to 320ms.

# References

[1] R4-2416866, WF on RRM requirements for NR_NTN_Ph3_Part2, Qualcomm, RAN4#112bis

[2] R4-2415669, Topic summary for [112bis][222] NR_NTN_Ph3_Part2, Qualcomm, RAN4#112bis