# 3GPP TSG-RAN WG4 Meeting #113	R4-2418570

# Orlando, US, 18th – 22nd November, 2024

Agenda item:	7.8.3.4

Source:	Apple

Title:	On less than 5MHz for NTN RRM

Document for:	Discussion

# Introduction

The WF for RRM of NTN with channel BW less than 5MHz [1] has been agreed in RAN4#112, and the open issues have been summarized in the topic summary for [112bis][207]NR_IoT_NTN_req_test_enh [2].

In this contribution, we continue the discussion on the RRM requirement for less than 5MHz in NR NTN.

# Discussion

#### Issue 1-2-1-2 CHO delay

Agreements/ Way-forward: - for CHO without L3 measurement: - T∆ in CHO requirement for NTN in exiting TS38.133 shall be extended to [3 Trs] - FFS on for CHO with L3 measurement: - Tmeasure which is same as the definition for Tidentify_intra_with_index and Tidentify_inter_with_index in issue 1-2-4-2 will be extended - T∆ in CHO requirement for NTN in exiting TS38.133 shall be extended to [3 Trs]

In current spec, Tmeasure is referred to clause 6.1C.2.2.2, and there are many conditions in the clause to decide the value of Tmeasure, e.g.,

In our view, as long as we conclude on how to extend the Tidentify_intra_with_index and Tidentify_inter_with_index, it’s not necessary to change anything for Tmeasure in CHO requirement with L3 measurement. However, to define a minimum requirement to consider all possible UE implementations, even though UE has extension during Tmeasure, the T∆ in CHO requirement for NTN in exiting TS38.133 shall still be extended to 3 Trs, which is similar as CHO without L3 measurement.

Proposal 1: for CHO with L3 measurement:

- No need to change Tmeasure in CHO requirement with L3 measurement, as long as extension for Tidentify_intra_with_index and Tidentify_inter_with_index is concluded.

- T∆ in CHO requirement for NTN in exiting TS38.133 shall be extended to [3 Trs].

#### Issue 1-2-4-2: How to define the new requirements for SSB time index detection

Agreements/Wayforward - The number of SSB samples for Rel-18 NTN measurement requirements (Tidentify_xxx_with_index) will be extended by X and Y more samples for intra-frequency and inter-frequency cases respectively. - For X and Y, the values from TN less than 5MHz as starting point.

In the current TN requirement for less than 5MHz, it’s specified that sample extension is X=4 and Y=3 for intra-frequency and inter-frequency cases respectively. In R17 NTN requirement, we used TN measurement delay and accuracy as baseline, and therefore here, we propose also to use the extension values from TN less than 5MHz as baseline.

Proposal 2: The number of SSB samples for Rel-18 NTN measurement requirements (Tidentify_xxx_with_index) will be extended by X and Y more samples for intra-frequency and inter-frequency cases respectively.

- X=4 and Y=3.

# Conclusion

In this contribution, we continue the discussion on the RRM requirement for less than 5MHz in NR NTN.

Proposal 1: for CHO with L3 measurement:

- No need to change Tmeasure in CHO requirement with L3 measurement, as long as extension for Tidentify_intra_with_index and Tidentify_inter_with_index is concluded.

- T∆ in CHO requirement for NTN in exiting TS38.133 shall be extended to [3 Trs].

Proposal 2: The number of SSB samples for Rel-18 NTN measurement requirements (Tidentify_xxx_with_index) will be extended by X and Y more samples for intra-frequency and inter-frequency cases respectively.

- X=4 and Y=3.

# References

[1] R4-2416851, WF on RRM requirements for [112bis][207] NR_IoT_NTN_req_test_enh, Xiaomi, RAN4#112bis

[2] R4-2415654, Topic summary for [112bis][207] NR_IoT_NTN_req_test_enh, Xiaomi, RAN4#112bis