# 3GPP TSG-RAN WG4 Meeting #113	R4-2418574

# Orlando, US, 18th – 22nd November, 2024

Agenda item:	5.21.3

Source:	Apple

Title:	On R18 NTN RRM core and performance requirement maintenance

Document for:	Discussion

# Introduction

In last RAN4#112 meeting, the WF for eNTN has been agreed in [1] and moderator has captured all open issues in the topic summary [2]. There are still some opens issues for further discussion, and in this contribution, we continue the discussion on remaining RRM performance requirements for eNTN.

# Discussion on Core part

### Issue 4-1: TN to NTN cell reselection

Agreement (online): - For NTN incapable UE, if both TN and NTN carriers are broadcasted for neighbour cells measurement in IDLE/Inactive mode, the existing TN-to-TN cell reselection requirements apply. FFS: - For TN to NTN cell re-selection requirement when Srxlev ≤ SnonIntraSearchP or Squal ≤ SnonIntraSearchQ, if UE is configured by network to have at least one high priority carrier which contains NTN cells, the requirements for GNSS ON shall be applied

As discussed in previous meetings, when UE is camping on a TN serving cell but may perform cell re-selection to a target NTN cell, the difficult part is when UE shall turn on its GNSS modem for its GNSS location acquisition. Since UE is in TN coverage and serving cell is good enough, it will waste power if UE keeps the GNSS modem ON. However, if UE turns on the GNSS when mobility/re-selection is triggered, then it will need much long time for UE to acquire the GNSS location (first time to fix for GNSS localization is time consuming).

Observation 1: Since the first fix of GNSS will take very long time during the cell reselection procedure, it’s not necessary to always consider UE turns on the GNSS only when the neighbor NTN measurement is triggered.

In our view, there might be some scenarios can help UE to determine GNSS is ON or OFF. For instance, if UE is configured by network to have at least one high priority carrier which contains NTN cells, UE shall keep GNSS ON during such high priority frequency layer measurement even the UE is in TN coverage. As shown in the following spec text of NTN requirement, even though UE is not required to continuously measure the detected cell to evaluate the ongoing possibility of reselection after determining that reselection has not occurred, the high priority carrier searching is always there after configured in SIB4.

If Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ, and the distance between UE and serving cell reference location is smaller than distanceThresh if distanceThresh is configured and UE has location information, then the UE shall search for inter-frequency layers of higher priority at least every Thigher_priority_search where Thigher_priority_search is described in clause 4.2C.2.9. -------Unrelated section omitted------- When higher priority cells are found by the higher priority search, they shall be measured at least every Tmeasure,NR_Inter. If, after detecting a cell in a higher priority search, it is determined that reselection has not occurred then the UE is not required to continuously measure the detected cell to evaluate the ongoing possibility of reselection. However, the minimum measurement filtering requirements specified later in this clause shall still be met by the UE before it makes any determination that it may stop measuring the cell. If the UE detects on a NR carrier a cell whose physical identity is indicated as not allowed for that carrier in the measurement control system information of the serving cell, the UE is not required to perform measurements on that cell.

Thus, we think at least we can assume the GNSS is always ON when higher priority search is configured.

Observation 2: if UE is configured by network to have at least one high priority carrier which contains NTN cells, UE will keep GNSS ON during such high priority frequency layer measurement even the UE is in TN coverage.

Thus, we propose:

Proposal 1: For TN to NTN cell re-selection requirement when Srxlev ≤ SnonIntraSearchP or Squal ≤ SnonIntraSearchQ, if UE is configured by network to have at least one high priority carrier which contains NTN cells, the requirements for GNSS ON shall be applied.

### Issue 5-2-S: Soft’ Satellite switch

FFS: - RAN4 to not consider PDD reporting between serving and target satellites involved in the satellite switching without PCI change. FFS: - To enhance scheduling restriction in soft satellite switching, consider modifying the capability rule such that: if UE indicates to support soft satellite switching without PCI change, it also means UE supports both simultaneousRxDataSSB-DiffNumerology and parallelMeasurementWithoutRestriction-r17.

How to optimize the scheduling restriction is not fully concluded yet as highlighted in yellow above. In previous meeting, some companies proposed to use PDD measurement/report to enhance the scheduling restriction during the soft satellite switch. However, RAN2 had following agreements that,

3. We don’t introduce the support for configuring and reporting measurements on the target cell (from target satellite) in the soft switch case 4. RAN2 understands that the NW is not expected to configure PDD reporting between serving and target satellites involved in the satellite switch

In addition, when to start target satellite synchronization is up to UE implementation, even though network can somehow know the time location of received target SAT’s SSB occasion at UE, network may still need to apply scheduling restriction on all possible SSB occasions, therefore PDD reporting benefit is very limited. So RAN4 shall not consider PDD reporting between serving and target satellites involved in the satellite switch. Regarding the scheduling restriction itself, if UE is not capable of parallelMeasurementWithoutRestriction-r17, network can still configure hard satellite switching for simplicity, and it can always be alternative for these special cases. From UE capability perspective, our understanding is: If UE indicates to support soft satellite switching without PCI change, it would be typical for UE also to indicate to support simultaneousRxDataSSB-DiffNumerology and parallelMeasurementWithoutRestriction-r17 as well. And in this case, then no scheduling restriction will be applied during the time period of [t-serviceStart, t-Service] during the soft satellite switching.

Proposal 2: not consider PDD reporting between serving and target satellites involved in the satellite switching without PCI change.

Proposal 3: to enhance scheduling restriction in soft satellite switching, consider modifying the capability rule such that: if UE indicates to support soft satellite switching without PCI change, it also means UE supports both simultaneousRxDataSSB-DiffNumerology and parallelMeasurementWithoutRestriction-r17.

# Discussion on Performance part

The previous agreement and open issue for UE accuracy requirement over 10GHz is:

### Issue 6-2-2: (FR2-NTN) Rx beam gain

Agreement (online): - For the minimum SSB_RP condition for electronic steering antenna, - RAN4 to confirm Y (gain difference between fine and rough beams) = 0 - Remove the bracket for the following 1dB relaxation: - The existing absolute measurement accuracy requirement and relative measurement accuracy requirement of TN FR2 (including intra-frequency and inter-frequency) can be applied for NTN UE above 10GHz with 1dB relaxation Agreement (ad-hoc): - For RLM for FR2-NTN, RAN4 to discuss the following options - Update core requirements (PDCCH parameters, evaluation period) as for R17 RedCap Agreement (ad-hoc): - Value of Gmin: - Further discuss the detailed parameter assumptions to be aligned with core specification TS 38.101-5 and TR 38.863. - Separate values will be introduced for NTN VSAT Type 3 and other VSAT types. Agreement (ad-hoc): - Gmax leave to be declaration basis. - Note: FFS whether RAN4 needs to specify the possible range for Gmax

In the legacy TN FR2 accuracy requirement, the minimum SSB power is determined based on the following criteria:

In NTN above 10GHz, the same principle can be applied to derive the mini SSB_RP, which is based on the equation from TS38.863section 7.4.3.3.2,

Based on the above analysis of equation in TS38.863, to derive the Rx beam gain, we may consider the following formulas,

- REFSENS_50MHz = -174 dBm/Hz + 10log10(50MHz) + NF + SNR (QPSK) + IM - Ant_Gain

Therefore Ant_Gain can be derived as,

- Ant_Gain = -174 dBm/Hz + 10log10(50MHz) + NF + SNR (QPSK) + IM - REFSENS_50MHz

For FR2-NTN, two REFSENS_50MHz values have been specified in TS38.101-5,

- -122 dBm for VSAT type 1/2/4/5

- -115.6 dBm for VSAT type 3

With the following parameter assumptions,

- NF=10*log10 ((T+T0)/T0) = 6dB for type 3 and 2.5dB for other types, since antenna temperature is TBD in table 6a.2.2.2-1 (Rx chain temperature is unknown), thus we used the agreements from R4-2317648.

Issue 1-4: UE types Agreement: - RAN4 to consider the following UE Types: 1/ Terminal Type 1 (Electronic steering antenna) with the following NFs: - 2.5 dB NF (Class 1) - for both LEO and GEO; - 6 dB NF (Class 2) - for LEO only. 2/ Terminal Type 2 (Mechanical steering antenna) with the following NFs: - 2.5 dB NF (Class 1) – for both LEO and GEO.

- SNR = -1 dB

- IM = 2 dB

We can derive the following two Ant_Gain values,

- For VSAT type 1/2/4/5:

  - Ant_Gain = -174dBm + 77dB + 2.5dB + (-1dB) + 2dB - (-122dBm) = 28.5dB

- For VSAT type 3,

  - Ant_Gain = -174dBm + 77dB + 6dB + (-1dB) + 2dB - (-115.6dBm) = 25.6dB

Thus, based on our calculation the lower bound of Rx beam gain can be as following:

- For VSAT type 1/2/4/5, the lower bound of Rx beam gain is 28.5dB.

- For VSAT type 3, the lower bound of Rx beam gain is 25.6dB.

However, the calculation can only be used to decide the lower bound of beam gain, and we still need VSAT vendor to claim the upper bound of the Rx beam gain.

Proposal 4:

•	For VSAT type 1/2/4/5, the lower bound of Rx beam gain is 28.5dB.

•	For VSAT type 3, the lower bound of Rx beam gain is 25.6dB.

Proposal 5: VSAT vendor to claim the upper bound of the Rx beam gain.

# Conclusion

In this contribution, we discuss the NR-NTN core and performance requirement maintenance in above 10 GHz bands.

Observation 1: Since the first fix of GNSS will take very long time during the cell reselection procedure, it’s not necessary to always consider UE turns on the GNSS only when the neighbor NTN measurement is triggered.

Observation 2: if UE is configured by network to have at least one high priority carrier which contains NTN cells, UE will keep GNSS ON during such high priority frequency layer measurement even the UE is in TN coverage.

Proposal 1: For TN to NTN cell re-selection requirement when Srxlev ≤ SnonIntraSearchP or Squal ≤ SnonIntraSearchQ, if UE is configured by network to have at least one high priority carrier which contains NTN cells, the requirements for GNSS ON shall be applied.

Proposal 2: not consider PDD reporting between serving and target satellites involved in the satellite switching without PCI change.

Proposal 3: to enhance scheduling restriction in soft satellite switching, consider modifying the capability rule such that: if UE indicates to support soft satellite switching without PCI change, it also means UE supports both simultaneousRxDataSSB-DiffNumerology and parallelMeasurementWithoutRestriction-r17.

Proposal 4:

•	For VSAT type 1/2/4/5, the lower bound of Rx beam gain is 28.5dB.

•	For VSAT type 3, the lower bound of Rx beam gain is 25.6dB.

Proposal 5: VSAT vendor to claim the upper bound of the Rx beam gain.

# References

[1] R4-2411801, Topic summary for [112][206] NR_NTN_enh, Qualcomm Incorporated, RAN4 #112

[2] R4-2414100, WF on RRM requirements for NR_NTN_enh, Qualcomm Incorporated, RAN4#112