3GPP TSG-RAN WG4 Meeting #113                                   						     R4-2418578

# Orlando, US, 18th – 22th November, 2024

Source: 	Apple

Title: 	On reference architecture and RF capability for type 4 UE

Agenda Item:	7.5.2.3

Document for:	Discussion

# Introduction

## < Issue 3-4: Reference UE architecture >

Way Forward:

- Continue further discussion on the following aspects for reference UE architecture in the next meeting.

- Comments of proposed reference UE architecture table

- Partially/fully shared

- Differentiate Type 1a and 1b

- If identified, other additional issues

There were some discussions on the problem for reference architecture for type 4 UE based on [1,2,3]. It was agreed to have further discussion and clarification Reference [4]

This contribution gives our view on how solve this issue.

# Discussion

As discussed in [5], type 4a and 4b architecture were assumed as the baseline for type 4 UE capability and requirement development which indicates that type 4 UE has 6/8 independent Rx chain from LNA to analog BB without further splitting after LNA for each Rx chain.

Table 2.1-1 reference architecture

| UE Type | CC# | antenna / LNA |  | Mixer | Analog BB | #Rx | Frequency Separation between 2cc | NRCA/ENDC | power imbalance | comment |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | 4 shared |  | 4 shared | 4 shared | 4Rx | ≤ X MHz | NRCA,ENDC | 6dB full range | Baseline architecture (i.e. legacy architecture) |
|  | 2 |  |  |  |  | 4Rx |  |  |  |  |
| 2 | 1 | 2 | 4 total | 2 | 2 | 2Rx | No limitation or ≤ X MHz | NRCA,ENDC | 25dB full range | Reuse of baseline architecture restricted to 2Rx/band but need 2LO frequencies |
|  | 2 | 2 |  | 2 | 2 | 2Rx |  |  |  |  |
| 3a | 1 | 4 shared |  | 4 | 4 | 4Rx | No limitation or ≤ X MHz | ENDC | 6<P≤25dB partial range | Reuse of baseline RFFE architecture adding RF split after 2 LNAs + 1BB/Rx => common AGC on LNA => 25dB only for some range |
|  | 2 |  |  | 2 | 2 | 2Rx |  |  |  |  |
| 3b | 1 | 4 shared |  | 4 | 4 | 4Rx | No limitation or ≤ X MHz | NRCA,ENDC | 6<P≤25dB partial range | Reuse of baseline RFFE architecture adding RF split after 2 LNAs + 1BB/Rx => common AGC on LNA => 25dB only for some range |
|  | 2 |  |  | 4 | 4 | 4Rx |  |  |  |  |
| 4a | 1 | 4 | 6 total | 4 | 4 | 4Rx | No limitation or ≤ X MHz | ENDC | 25dB full range | Requires 6 antennas and LNA => is it compatible with smartphone? (for which frequency range), FWA only |
|  | 2 | 2 |  | 2 | 2 | 2Rx |  |  |  |  |
| 4b | 1 | 4 | 8 total | 4 | 4 | 4Rx | No limitation or ≤ X MHz | NRCA,ENDC | 25dB full range | Requires 8 antennas and LNA => is it compatible with smartphone? (for which frequency range), FWA only |
|  | 2 | 4 |  | 4 | 4 | 4Rx |  |  |  |  |

It was also agreed for type 4 UE to support the following 2 capability sets [3] with the assumption that type 4 UE will do hardware reconfiguration between collocated condition and non-collocated condition. When 8 MIMO layers per CC is supported in collocated condition, type 4 UE needs to fully share 6/8 Rx chains for both CCs to support non-contiguous CA between these 2 CCs which is type 1 architecture as shown in Table 2.1-1.

Observation 1: Type 4 UE assumes fully shared Rx chain RF architecture between non-contiguous CCs when collocated condition is met.

While this seems not compliant with what is agreed for intra-band non-contiguous CA as indicated in TR 36.823. As shown in Section 6.2.3.1 (reproduced below), to support non-contiguous intra-band CA, separate Rx chain is needed for each CC. Only the first LNA is shared between CCs and the signal will be divided and handled by different Rx chain after the signal division component. The motivation on this reference architecture is that both non-contiguous carriers should satisfy the Rel-8 ACS requirements for the in-GAP frequency range.

Observation 2: Partially shared Rx RF architecture was assumed for non-contiguous intra-band CA as shown in Figure 6.2.3.1-1 in TR 36.823.

There were some discussions on how to solve this issue based on [1,2,3] while non consensus was reached. The problem for the current architecture together with the agreed capability sets is that it is only workable when it is interference free at in-GAP frequency range. Otherwise, the RF reconfiguration to collocated condition for Rx chain sharing will result in performance degradation or even not workable depending on the interference level observed by the UE. But the in-GAP interference is caused by another operator and not controllable. There could be different options to solve this issue.

Option 1: Rediscuss the reference architecture for type 4 supporting both capability sets.

Option 2: Keep the current architecture unchanged, but do not require type 4 UE supporting reconfiguration to 8Rx type 1 capability. i.e. type 4 UE only supports type 4 capability in non-collocated condition.

Option 3: Keep the current architecture and agreed capability sets unchanged, add some assumption when RF reconfiguration is workable.

- The drawback of this option is that it is still not a robust solution without studies.

Option 3: Keep the current architecture and agreed capability sets unchanged, have some analysis on in-GAP interference to support type 4 UE reconfiguration.

Proposal: RAN4 discuss how to solve the the identified issue for type 4 UE based on the following options.

Option 1: Rediscuss the reference architecture for type 4 supporting both capability sets.

Option 2: Keep the current architecture unchanged, but do not require type 4 UE supporting reconfiguration to 8Rx type 1 capability. i.e. type 4 UE only supports type 4 capability in non-collocated condition.

Option 3: Keep the current architecture and agreed capability sets unchanged, add some assumption when RF reconfiguration is workable.

- The drawback of this option is that it is still not a robust solution without studies.

Option 3: Keep the current architecture and agreed capability sets unchanged, have some analysis on in-GAP interference to support type 4 UE reconfiguration.

Observation 3: Same problem exists for type 2 UE given the assumption is the same between type 2 and type 4.

# Summary

This contribution presented our view on new identified issue related to type 4 UE reference architecture/capability. The following observation and proposal are concluded.

Observation 1: Type 4 UE assumes fully shared Rx chain RF architecture between non-contiguous CCs when collocated condition is met.

Observation 2: Partially shared Rx RF architecture was assumed for non-contiguous intra-band CA as shown in Figure 6.2.3.1-1 in TR 36.823.

Proposal: RAN4 discuss how to solve the the identified issue for type 4 UE based on the following options.

Option 1: Rediscuss the reference architecture for type 4 supporting both capability sets.

Option 2: Keep the current architecture unchanged but do not require type 4 UE supporting reconfiguration to 8Rx type 1 capability.

- i.e. type 4 UE only supports type 4 capability in non-collocated condition.

Option 3: Keep the current architecture and agreed capability sets unchanged and add some assumption when RF reconfiguration is workable.

- The drawback of this option is that it is still not a robust solution without studies.

Option 3: Keep the current architecture and agreed capability sets unchanged, have some analysis on in-GAP interference to support type 4 UE reconfiguration.

Observation 3: Same problem exists for type 2 UE given the assumption is the same between type 2 and type 4.

# References

[1] R4-2414951, Discussion on the referenced UE architecture table for non-collocated, RAN4#112bis, Samsung

[2] R4-2415514, On reference architecture and RF capability for type 4 UE, RAN4#112bis, Apple

[3] R4-2416260, How to introduce UE types into the specifications and rectifying errors UE architecture types, RAN4#112bis, Huawei

[4] R4-2417115, WF on rest issues for intra-band non-collocated EN-DC/NR-CA, RAN4#112bis, KDDI