3GPP TSG-RAN WG4 Meeting #113                                   R4-2418581

Orlando, US, 18-22 November 2024

Source: 	Apple

Title: 	Remaining issue on RRM requirement for ATG

Agenda Item:	7.10.4

Document for:	Discussion

# Introduction

ATG RRM requirement was extensively discussed in the last RAN4 meeting, and the open issue are captured in WF [1]

This contribution presents our further views on the requirement impact due to introduction of CA for ATG.

# Discussion

## General

The following issue was captured in the WF.

Issue 1-1-2: UE antenna type for inter-band CA

Agreement:

- Refine the agreement from last meeting as:

  - For intra-band contiguous CA, same antenna type should be applied on a band.

  - For inter-band CA, either different or same antenna type can be applied on different bands.

  - For inter-band CA, consider the following cases:

    - 1. ATG UE with omnidirectional antennas on the bands of PCC and SCC.

    - 2. ATG UE with an omnidirectional antenna on the band of PCC and an antenna array on the band of SCC.

    - 3. ATG UE with an antenna array on the band of PCC and an omnidirectional antenna on the band of SCC.

    - [4. ATG UE with an antenna array on the bands of PCC and SCC, and one common Rx beam assumed.]

    - 5. ATG UE with an antenna array on the bands of PCC and SCC, and two simultaneous separate Rx beams assumed.

Further check which understanding is correct:

- Understanding #1: Case 4 is excluded in RF session.

- Understanding #2: Case 4 is not explicitly discussed in RF session, since no case 4 specific RF requirement. Case 4 UE implementation is possible.

Regarding case 4, our understanding is Case 4 is not explicitly discussed in RF session and hence it’s hard to say whether specific RF requirements need to be developed. if the assumption is separate RF chain for each band and using common antenna array for collocated scenario only, it seems workable.

## Timing requirements

Issue 1-3-2: MRTD

Agreement:

- For ATG intra-band contiguous CA, not define MRTD requirement.

- For ATG inter-band CA, MRTD requirement is:

  - Option 1: A little larger than 3us, i.e., 3.545 µs

  - Option 2: 33us

  - Option 3: 3us

  - Option 4: further discuss

The following open issues were discussed in RAN4#112bis meeting but not concluded

Regarding MRTD, although the current scenario is inter-band CA with collocated condition, usually separate RF chain will be used, and the UE can handle large MRTD of 33us as specified in 38.133. while some concern was raised on its relation to the antenna type for ATG UE supporting inter-band CA, e.g. case 4. We think it sufficient to adopt Option 3 in R19 since only collocated scenario is considered.

Proposal 1: It is proposed define MRTD requirement for ATG inter-band CA as 3us in R19.

## Interruption requirement

The below table summarized our view on signalling characteristics for ATG UE.

Table 2.3-1 interruption requriement for ATG UE

| Sub-clause | Requirement | Comments |
| --- | --- | --- |
| 8 | Signaling characteristics |  |
| 8.1 | Radio link monitoring |  |
| 8.2 | Interruption |  |
| 8.2.2 | Introduction | Some adaption on the requirement applicability is needed depending on the supported feature by ATG UE. |
| 8.2.2.2.1 | Interruption at Scell addition/release | The requirement can be reused by focusing on the applicable part for ATG by considering the same corresponding scenario. FR2 and FR1 non-collocated part should be deleted. |
| 8.2.2.2.2 | Interruptions at SCell activation/deactivation | The requirement can be reused by focusing on the applicable part for ATG by considering the same corresponding scenario. FR2 and FR1 non-collocated part should be deleted. |
| 8.2.2.2.3 | Interruptions during measurements on deactivated SCC | The requirement can be reused by focusing on the applicable part for ATG by considering the same corresponding scenario. FR2 and FR1 non-collocated part should be deleted. |
| 8.2.2.2.4 | Interruptions at UL carrier RRC reconfiguration | The requirement can be reused by focusing on the applicable part for ATG by considering the same corresponding scenario. SUL part should be deleted. |
| 8.2.2.2.5 | Interruptions due to Active BWP switching Requirement | The requirement can be reused by focusing on the applicable part for ATG by considering the same corresponding scenario. |
| 8.2.2.2.6 | Interruptions at inter-frequency SFTD measurement | The requirement can be reused by focusing on the applicable part for ATG by considering the same corresponding scenario. |
| 8.2.2.2.7 | Interruptions at SCell activation/deactivation with multiple downlink SCells | Not applicable to ATG UE due to single Scell in Rel-19. |
| 8.2.2.2.8 | Interruptions due to UE-specific CBW change | Can be completely reused for ATG UE. |
| 8.2.2.2.9 | Interruptions at NR SRS carrier based switching | Can be reused for ATG UE. |
| 8.2.2.2.10 | DL Interruptions at UE switching between two uplink carriers | UL carrier switching for handheld UE was motivated mainly by - Implementation difficulty to support too many chains - For some band combination (e.g. FDD in low band and TDD band in high band), it is beneficial to switch the chain in low band to a high band to achieve high data rate by using MIMO. This seems to be a solution for handheld UE with good tradeoff between implementation difficulty and performance gain. While it will introduce interruptions due to switching which is also not a best solution from performance perspective. While for ATG, it seems not a big issue to implement as many chains as needed so that the best performance without interruption can be achieved. Further for the example band of CA_n3-n39, there is little gain to do so as they are so close in frequency. Hence, we think it’s not necessary to consider this feature for ATG. |
| 8.2.2.2.10A | DL Interruptions at UE switching between two uplink carriers with two transmit antenna connectors | Same comment as for 8.2.2.10. |
| 8.2.2.2.10B | DL Interruptions at UE switching between one uplink band with one transmit antenna connector and one uplink band with two transmit antenna connectors | Same comment as for 8.2.2.10. |
| 8.2.2.2.10C | DL Interruptions at UE switching between two uplink bands with two transmit antenna connectors | Same comment as for 8.2.2.10. |
| 8.2.2.2.10D | DL Interruptions at UE switching across three or four uplink bands | Same comment as for 8.2.2.10. |
| 8.2.2.2.11 | Interruptions at direct SCell activation | Can be reused. |
| 8.2.2.2.12 | Interruptions due to SCell dormancy | Can be reused. |
| 8.2.2.2.13 | Interruptions at transitions between active and non-active during DRX | Not applicable |
| 8.2.2.2.14 | Interruptions when identifying CGI of an NR cell with autonomous gaps | Seems not useful for ATG UE. |
| 8.2.2.2.15 | Interruptions when identifying CGI of an E-UTRA cell with autonomous gaps | Not applicable |
| 8.2.2.2.16 | Interruptions at NR SRS antenna port switching | Can be reused. |
| 8.2.2.2.17 | Interruptions at fast SCell activation | Can be reused. |
| 8.2.2.2.18 | Interruptions due to PUCCH SCell activation/deactivation | Can be reused. |
| 8.2.2.2.19 | Interruptions due to measurements without gap carried out by UE supporting [NeedForInterruptionInfoNR-R18] | Can be reused for ATG. |
| 8.2.2.2.20 | Interruptions due to PDCCH ordered RACH on target LTM cell | ATG cell switching is not so frequent. And LTM feature is still in the process of optimizing and being enhanced. We think it’s not necessary to complicate the ATG mobility at this stage. We propose to skip this feature for the time being. If there is real demand for this feature, can be reconsidered in future release when LTM feature is stable for TN. |

Proposal 2: it is proposed to consider the proposal on interruption as listed in table 2.3-1.

# Summary

This contribution presented some analysis on ATG UE RRM requirements. The following proposals are concluded.

Proposal 1: It is proposed define MRTD requirement for ATG inter-band CA as 3us in R19.

Proposal 2: it is proposed to consider the proposal on interruption as listed in table 2.3-1.

# References

[1] R4-2416852, WF on RRM requirements for [112bis][208] NR_ATG_enh, CMCC, RAN4#112bis