3GPP TSG-RAN WG4 Meeting #113                              			      				 R4-2418582

Orlando, US, 18th – 12nd November 2024

Source: 	Apple

Title: 	On CSI-RS based L1-RSRP measurement

Agenda Item:	7.24.2.2

Document for:	Discussion

# Introduction

During the past meetings, there have been extensive discussions on CSI-RS based L1 measurement for NR mobility enhancements WI [1]. WF in [2] captured all the conclusions and open issues for CSI-RS based L1 measurement in RAN4#112bis meeting.

This contribution will give further analysis on the open issues and give our proposals.

# Discussion

## Pre-requisite for CSI-RS based L1-RSRP

The following is concluded from RAN4#112bis meeting.

#### Issue 2-2: known cell definition in CSI-RS based L1 measurement

For CSI-RS based L1-RSRP measurement, the cell is considered as known if the following conditions are met:

- Agreement:

  - The UE has performed L3 measurement on the target cell during the last 5 seconds, and

    - Further check whether to consider SSB-based L3 measurement only

  - The SSB from the target cell configured for L3[/L1] measurement remains detectable according to the cell identification requirements in clause 9.2.

    - L1 measurement in the 2nd bullet is not needed for FR1.

    - Further discuss whether to keep or remove L1 measurement for FR2-1 in the 2nd bullet.

  - The CSI-RS from the target cell configured for L1 measurement remains [detectable/measurable].

    - Further discuss whether to use detectable or measurable.

    - Further discuss the side condition

As for which type of L3 measurement should be used to determine the ranking for target cells for CSI-RS based L1-RSRP measurement. Our view is that only considering SSB-based L3 measurement is sufficient. In the current RRM spec, there are SSB based L3 measurement and CSI-RS based L3 measurement. From technical perspective, both can be used as the pre-requisite condition for cell quality ranking. While we all know that CSI-RS based L3 measurement is only specified assuming the existence of associated SSB and it is not a standalone measurement. In this regard, using CSI-RS based L3 measurement as condition will unnecessarily complicate the measurement design. Our view is that it is sufficient to only use SSB based L3 measurement as the prerequisite condition to determine the target cells for CSI-RS based L1-RSRP measurement.

Proposal 1: It is proposed to consider SSB based L3 measurement as the pre-requisite condition only. E.g. no need to consider CSI-RS based L3 measurement as condition.

In the last meeting, it was discussed whether to keep SSB based L1 measurement as the condition for CSI-RS based L1 measurement in FR2. The UE needs to perform Rx beam sweeping on CSI-RS resource for neighbour cells without prior SSB based L1 measurement as the basis, which will make the measurement delay for neighbour cell quite long (N times scaling). In this regard, we think it make sense to keep SSB based L1 measurement as one of the pre-requisite conditions for CSI-RS based L1 measurement in FR2.

Proposal 2: Keep SSB based L1 measurement as one of the pre-requisite conditions for CSI-RS based L1 measurement in FR2.

Regarding measurable v.s. detectable, our preference is detectable since measurable does not always mean the cell is detectable. And in legacy CSI-RS L1 measurement accuracy is defined as ±5dB at SINR=-3dB (48 PRBs and the density is 3, one sample). It is suggested to reuse the existing side condition for CSI-RS based L1-RSRP measurement for serving cell as the starting point for CSI-RS based L1-RSRP measurement for neighbour cell.

Proposal 3: it is proposed to use detectable and legacy side condition of SINR=-3dB.

## Measurement procedure for CSI-RS based LTM candidat4e cell

The following was concluded from RAN4#112bis meeting.

Alternative 1:

    - RAN4 to consider the following measurement procedure for CSI-RS based LTM candidate cell measurements:

      - Step 1: SSB based L3 measurements [and reports]

      - Step 2: For FR2-1, SSB-based L1 measurements and reports on the same candidate cell

        - Further discuss whether Step 2 can be skipped under certain conditions. FFS the details of the conditions.

        - Note: Step 2 is removed for FR1.

      - Step 3: CSI-RS based L1 measurements on the same candidate cell

      - Note: The CSI-RS in Step 3 should be QCL’ed with the SSB in Step 1 or 2.

        - Note: For FR1, the CSI-RS in Step 3 should be QCL’ed with the SSB in Step 1.

Alternative 2:

    - RAN4 to consider the following measurement procedure for CSI-RS based LTM candidate cell measurements:

      - Step 1: SSB based L3 measurements [and reports]

      - Step 3: CSI-RS based L1 measurements on the same candidate cell

      - Note: The CSI-RS in Step 3 should be QCL’ed with the SSB in Step 1.

With the discussion in 2.1.1, the measurement procedure for CSI-RS based LTM candidate cell should be,

      - Step 1: SSB based L3 measurements [and reports]

      - Step 2: For FR2-1, SSB-based L1 measurements and reports on the same candidate cell

        - Note: Step 2 is removed for FR1.

      - Step 3: CSI-RS based L1 measurements on the same candidate cell

        - Note: The CSI-RS in should be QCL’ed with the SSB.

Proposal 4: The measurement procedure for CSI-RS based LTM candidate cell should be,

      - Step 1: SSB based L3 measurements [and reports]

      - Step 2: For FR2-1, SSB-based L1 measurements and reports on the same candidate cell

        - Note: Step 2 is removed for FR1.

      - Step 3: CSI-RS based L1 measurements on the same candidate cell

        - Note: The CSI-RS in should be QCL’ed with the SSB.

## Definition of intra/inter-frequency and requirement category

The following options were put on table after discussion in the last RAN4 meeting.

#### Issue 2-14: definition of intra-frequency and inter-frequency for CSI-RS based L1 measurement (already discussed during online session)

- Candidate solutions:

  - Option 1: Following CSI-RS based L3 measurement (CTC, CMCC, Apple, Samsung, OPPO, [Nokia?])

    - A measurement is defined as a CSI-RS based intra-frequency L1 measurement provided that:

      - the SCS of the CSI-RS resource of the neighbour cell configured for L1 measurement is the same as the SCS of the CSI-RS resource on the serving cell indicated for L1 measurement, and

      - the CP type of the CSI-RS resource of neighbour cell configured for L1 measurement is the same as the CP type of the CSI-RS resource of the serving cell indicated for L1 measurement, and

        - It is applied for SCS = 60KHz

      - the centre frequency of the CSI-RS resource of the neighbour cell configured for L1 measurement is the same as the centre frequency of the CSI-RS resource of the serving cell indicated for L1 measurement

  - Option 2: For definition of intra-frequency and inter-frequency, suggest to wait for more progress in other WGs.

  - Option 3: For backward compatibility, RAN4 better to have a definition for intra- and inter-frequency CSI-RS based L1-RSRP measurement as legacy.

    - One possible way is to revise the legacy definition and clarify that the reference centre frequency can be any of centre frequencies of the CSI-RS resources of the serving cell.

  - Option 4: Considering the introduction of CSI-RS based LTM L1 measurement, whether an LTM cell switch is intra-frequency or inter-frequency may depend on whether the associated SSB reported in the CSI-RS-based measurement result is intra-frequency or inter-frequency.

    - Whether the associated SSB is intra-frequency or inter-frequency follows the R18 rules

    - No additional definition of intra-F/inter-F for CSI-RS based L1 measurements.

  - Option 5: Intra-frequency CSI-RS based L1 measurement can be defined that the NZP-CSI-RS resource(s) of a candidate cell is within the DL active BWP of the serving cell and the SCS and CP of NZP-CSI-RS resource(s) of a candidate cell is same as those of the DL active BP of the serving cell.

    - It should be further discussed when the DL active BWP of the serving cell is within the bandwidth of NZP-CSI-RS resource(s) of a candidate cell.

Regarding option 2, it seems unnecessary since RAN1 already concluded that it is up to RAN4 to decide the intra-frequency/inter-frequency definition for CSI-RS based L1 measurement.

Regarding Option 4, we think the definition of intra-frequency/inter-frequency for CSI-RS based L1 measurement does not need to be coupled with the associated SSB. This is also the case for intra-frequency/inter-frequency definition for CSI-RS based L3 measurement. Same logic applies for CSI-RS based L1 measurement definition.

Regarding option 5, the benefit of this option looks like it can simplify the categorization of measurement and combination of measurement type with GAP. While it may complicate the definition of frequency layer since frequency layer definition is usually based on the centre frequency of resources to be measured. Even though we can avoid the discussion of differentiating multiple CSI-RS resources with active BWP, it still needs to do so when discussing the frequency layer later. And the past measurement requirements are all classified based on intra/inter-frequency definition in the specification. We would prefer not to change the specification structure too much and follow the previous approach as much as possible. Otherwise, it will put a lot of unnecessary effort on requirement introduction and eventually the specification readability may be unnecessarily impacted.

One problem for intra-frequency/inter-frequency definition of CSI-RS based L1 measurement is how to determine the reference resource since multiple CSI-RS resources may be configured for both serving cell and neighbour cell. To solve this issue, it seems a clarification as suggested in option 3 should be workable. So, we still prefer to follow the same logic of intra-frequency/inter-frequency definition for CSI-RS based L3 measurement and consider additional clarifications on the reference CSI-RS resources. E.g. a measurement is defined as a CSI-RS based intra-frequency measurement provided that:

-	the SCS of the CSI-RS resource of the neighbour cell configured for measurement is the same as the SCS of any of the CSI-RS resources on the serving cell indicated for measurement, and

-	the CP type of the CSI-RS resource of neighbour cell configured for measurement is the same as the CP type of any of the CSI-RS resources of the serving cell indicated for measurement, and

-	It is applied for SCS = 60KHz

-	the centre frequency of the CSI-RS resource of the neighbour cell configured for measurement is the same as the centre frequency of any of the CSI-RS resources of the serving cell indicated for measurement.

Proposal 5: it is proposed to follow intra-frequency/inter-frequency definition for CSI-RS based L3 measurement, e.g. a measurement is defined as a CSI-RS based intra-frequency L1 measurement provided that:

-	the SCS of the CSI-RS resource of the neighbour cell configured for measurement is the same as the SCS of any of the CSI-RS resources on the serving cell indicated for measurement, and

-	the CP type of the CSI-RS resource of neighbour cell configured for measurement is the same as the CP type of any of the CSI-RS resources of the serving cell indicated for measurement, and

-	It is applied for SCS = 60KHz

-	the centre frequency of the CSI-RS resource of the neighbour cell configured for measurement is the same as the centre frequency of any of the CSI-RS resources of the serving cell indicated for measurement.

The following was from RAN4#112bis meeting and need to be confirmed in this meeting. Based on the above discussion on intra-frequency/inter-frequency definition, it is natural to adopt option 2.

#### Issue 2-13: category of CSI-RS based L1 RSRP measurement (already discussed during online session)

- Candidate solutions:

  - Option 1: Categorize CSI-RS based L1-RSRP measurement into CSI-RS based L1-RSRP measurement within active BWP and outside active BWP for further discussion (MTK, CATT, E///, vivo)

  - Option 2: Categorize CSI-RS based L1-RSRP measurement into intra-frequency and inter-frequency (CATT, CTC, CMCC, Apple, Samsung, HW, OPPO, Nokia, ZTE)

Proposal 6: It is proposed to categorize CSI-RS based L1 measurement into intra-frequency and inter-frequency.

## Measurement type w.r.t. GAP

The following was captured in the WF agreed in RAN4#112bis meering,

#### Issue 2-3: CSI-RS based L1 measurement outside active BWP or with measurement gap

- Candidate solutions

  - Option 1: Deprioritize CSI-RS based L1-RSRP measurement outside active BWP (or with measurement gap) in R19. (MTK, QC, vivo, OPPO, Nokia)

  - Option 1a: RAN4 can first discuss measurement within active BWP for CSI-RS based L1 measurement and wait for progress in other WG about whether measurement gap will be supported. (Xiaomi)

  - Option 1b: RAN4 to define intra-frequency CSI-RS measurement requirements first. Support inter-frequency L1-RSRP CSI-RS measurements without gap. FFS for inter-frequency with gap. (Nokia)

  - Option 2: RAN4 at least support the following measurement types for CSI-RS based L1 measurement. (CATT)

    - Without gap

    - With gap

  - Option 2a: for CSI-RS based LTM, it is proposed to consider intra-frequency measurement, inter-frequency measurement without gap and inter-frequency measurement with gap, which are aligned with Rel-18 SSB based LTM. (CMCC)

  - Option 2b: prioritize intra-frequency measurement without GAP, inter-frequency measurement with GAP. (Apple, Samsung, HW, OPPO, CATT)

  - Option 2c: CSI-RS based L1 measurements for candidate cells shall support

    - Measurement within active BWP, Measurements outside active BWP. (E///, ZTE)

L3 based CSI-RS only considered intra-frequency without GAP and inter-frequency with GAP. As for CSI-RS based L1 RSRP measurement, it is suggested to follow the same approach as CSI-RS based L3 measurement to prioritize intra—frequency measurement without GAP and inter-frequency measurement with GAP as starting point. Inter-frequency measurement without GAP seems beneficial from measurement delay perspective. While CSI-RS resource configuration is quite flexible and complicated. Our preference would be to deprioritize inter-frequency measurement without GAP before resource configuration issue is clear.

Also, RAN1 agreed to focus on CSI-RS based L1-RSRP measurement and FFS on L1-SINR measurement. RAN4 could follow RAN1 decision on this aspect.

Proposal 7: It is proposed to prioritize intra-frequency measurement without GAP, inter-frequency measurement with GAP for CSI-RS based L1 measurement.

Proposal 8: RAN4 focus on CSI-RS based L1-RSRP measurement and FFS on L1-SINR measurement.

## Measurement configuration and GAP

The following was captured in the WF.

#### Issue 2-7: measurement gap related

- Candidate solutions

  - Proposal 1: In Rel-19 LTM, RAN4 needs to discuss whether to reuse or introduce the new gap for CSI-RS based L1 measurement. (CATT)

  - Proposal 2: RAN4 at least can follow the same approach as CSI-RS based L3 measurement for gap-based measurement, i.e., RAN4 to define requirements applicability for CSI-RS based L1 measurement. (CATT)

    - Motivation is to make sure “existing gap can cover the CSI-RS configuration in the same frequency layer”

  - Proposal 3: For CSI-RS based L1 with gap measurement, at least the type 1 MG needed to be supported in Rel-19 LTM. (CATT)

    - RAN4 to discuss whether to support other types of gap and wait for more RAN1 conclusions.

  - Proposal 4: For the decision on MG to use, RAN4 to wait for further RAN1 progress. (E///)

  - Proposal 5: defined a window to restrict the CSI-RS within gap which is used for CSI-RS based L3 measurement (ZTE).

CSI-RS resource allocation can be very flexible, which may largely increase the complexity in measurement requirement development. In Rel-16 CSI-RS based L3 measurement, for simplicity the requirement is developed based on some specific assumptions on the resource configuration as below.

-    All CSI-RS resources on one inter-frequency layer are configured within a window of up to 5ms, and

-    The periodicity of the configured CSI-RS resources is 10ms, 20ms or 40ms, and

-    CSI-RS resources for measurements and the associated SSB for cell identification are configured within measurement gap.

Since both measurement GAP and frequency layer definition is related to the CSI-RS resource configuration, it is proposed RAN4 conclude the resource configuration issue at first. E.g. whether to follow the same approach of L3 measurement by restricting the CSI-RS configuration in the spec or introduce some measurement timing configuration (e.g. L1-CMTC) for CSI-RS based L1 measurement. If the latter approach, L1-CMTC is to be configured by network via RRC signaling and should include at least the following information:

  - Frequency location (e.g. center frequency + bandwidth)

  - Time location (e.g. time offset + periodicity)

  - CSI-RS resource ID

The benefit of introducing CSI-RS measurement timing configuration can be several folds: on one hand it can avoid hard limitation for resource configuration; on the other hand, the UE only need to perform measurement on the resources indicated by the timing configuration, which can also reduce scheduling restriction to avoid throughput degradation since the more symbols UE needs to measure the more scheduling restrictions apply. Thirdly it can help to simplify the definition of frequency layer. In the last meeting, there were some discussions on how to define frequency layer for CSI-RS based L1 measurement. Due to the flexibility of CSI-RS configuration, the frequency layer definition needs to consider periodicity factor in addition to those included in the intra/inter-frequency definition. If we address the periodicity factor in the L1-CMTC, the definition of frequency layer can be simple by only considering SCS, CP type, centre frequency and BW.

#### Issue 2-15: definition of frequency layer in CSI-RS based L1-RSRP measurement

  - Option 1: Following CSI-RS based L3 measurement, for CSI-RS resources of the same L1 frequency layer, SCS, CP type, center frequency, BW and periodicity are the same. (MTK)

  - Option 2: Two CSI Resource are of same frequency layer if they have same SCS, CP type, BW, center frequency, and has overlapping periodicity. (E///)

Measurement GAP can be considered later after the above issue is concluded.

Proposal 9: It is proposed to introduce a CSI-RS based L1 measurement timing configuration.

## RTD assumption

The following was concluded in RAN4#112bis meeting.

- RTD is the received time difference among CSI-RS and SSB from different cells.

  - Alt 1: define requirements for both RTD<CP and RTD>CP. Introduce an optional UE capability for RTD>CP.

  - Alt 2: only define requirements for RTD<CP.

    - NW shall only enable this when RTD<CP is guaranteed.

We think RTD<CP should be taken as the baseline. It is noted that R18 SSB based L1 RSRP measurement on neighbor cell already addressed the case when RTD is larger than CP by UE capability. UE capabiltiy should also be considerd if RAN4 agrees to consider the case where RTD is larger than CP between serving cell and neighbour cell on the same carrier since it will put a lot complexity for UE implementaiton in terms of inreased number of FFTs. Both otpion 1 and option 2 are ok for us. For otpion 2, the group need to introduce RTD reporting to assit network make the judgement on when RTD<CP is satisfied.

Proposal 10: Either option 1 or option 2 is ok. RTD level reporting might be needed if option 2 is adopted.

## CSI-RS resource type

We agree with the following proposal on CSI-RS based L1 measurement.

#### Issue 2-10: CSI-RS resource type

- Candidate solutions:

  - Option 1: RAN4 prioritize CSI-RS based L1 measurement requirements for periodic CSI-RS. (vivo)

Proposal 11: RAN4 prioritize CSI-RS based L1 measurement requirement for periodic CSI-RS.

## Accuracy

In legacy CSI-RS L1 measurement accuracy is defined as ±5dB at SINR=-3dB (48 PRBs and the density is 3, one sample). It is suggested to reuse the existing side condition for CSI-RS based L1-RSRP measurement for serving cell as the starting point for CSI-RS based L1-RSRP measurement for neighbour cell.

Proposal 12: The existing side condition for CSI-RS based L1-RSRP measurement for serving cell is used as the starting point.

## LS to RAN1

LS to RAN1 on intra-frequency/inter-frequency definition is needed if proposal 5 is agreed.

Proposal 13: it is proposed to send a LS to RAN1 as below:

# Overall description

RAN4 discussed intra-frequency and inter-frequency definition for CSI-RS based L1 measurement and concluded that a measurement is defined as an intra-frequency CSI-RS based L1 measurement provided that:

-	the SCS of the CSI-RS resource of the neighbour cell configured for measurement is the same as the SCS of any of the CSI-RS resources on the serving cell indicated for measurement, and

-	the CP type of the CSI-RS resource of neighbour cell configured for measurement is the same as the CP type of any of the CSI-RS resources of the serving cell indicated for measurement, and

-	It is applied for SCS = 60KHz

-	the centre frequency of the CSI-RS resource of the neighbour cell configured for measurement is the same as the centre frequency of any of the CSI-RS resources of the serving cell indicated for measurement.

A measurement is defined as an inter-frequency CSI-RS based L1 measurement if the intra-frequency condition is not met.

# Actions

To RAN1:

ACTION: 	RAN4 respectfully asks RAN1 to consider the RAN4 decision on intra-frequency and inter-frequency definition for CSI-RS based L1 measurement.

Summary

This contribution presented our consideration on CSI-RS based L1-RSRP measurement. The following proposals are concluded.

Proposal 1: It is proposed to consider SSB based L3 measurement as the pre-requisite condition only. E.g. no need to consider CSI-RS based L3 measurement as condition.

Proposal 2: Keep SSB based L1 measurement as one of the pre-requisite conditions for CSI-RS based L1 measurement in FR2.

Proposal 3: it is proposed to use detectable and legacy side condition of SINR=-3dB.

Proposal 4: The measurement procedure for CSI-RS based LTM candidate cell should be,

      - Step 1: SSB based L3 measurements [and reports]

      - Step 2: For FR2-1, SSB-based L1 measurements and reports on the same candidate cell

        - Note: Step 2 is removed for FR1.

      - Step 3: CSI-RS based L1 measurements on the same candidate cell

        - Note: The CSI-RS in should be QCL’ed with the SSB.

Proposal 5: it is proposed to follow intra-frequency/inter-frequency definition for CSI-RS based L3 measurement, e.g. a measurement is defined as a CSI-RS based intra-frequency L1 measurement provided that:

-	the SCS of the CSI-RS resource of the neighbour cell configured for measurement is the same as the SCS of any of the CSI-RS resources on the serving cell indicated for measurement, and

-	the CP type of the CSI-RS resource of neighbour cell configured for measurement is the same as the CP type of any of the CSI-RS resources of the serving cell indicated for measurement, and

-	It is applied for SCS = 60KHz

-	the centre frequency of the CSI-RS resource of the neighbour cell configured for measurement is the same as the centre frequency of any of the CSI-RS resources of the serving cell indicated for measurement.

Proposal 6: It is proposed to categorize CSI-RS based L1 measurement into intra-frequency and inter-frequency.

Proposal 7: It is proposed to prioritize intra-frequency measurement without GAP, inter-frequency measurement with GAP for CSI-RS based L1 measurement.

Proposal 8: RAN4 focus on CSI-RS based L1-RSRP measurement and FFS on L1-SINR measurement.

Proposal 9: It is proposed to introduce a CSI-RS based L1 measurement timing configuration.

Proposal 10: Either option 1 or option 2 is ok. RTD reporting might be needed if option 2 is adopted.

Proposal 11: RAN4 prioritize CSI-RS based L1 measurement requirement for periodic CSI-RS.

Proposal 12: The existing side condition for CSI-RS based L1-RSRP measurement for serving cell is used as the starting point.

Proposal 13: it is proposed to send a LS to RAN1 as below:

# Overall description

RAN4 discussed intra-frequency and inter-frequency definition for CSI-RS based L1 measurement and concluded that a measurement is defined as an intra-frequency CSI-RS based L1 measurement provided that:

-	the SCS of the CSI-RS resource of the neighbour cell configured for measurement is the same as the SCS of any of the CSI-RS resources on the serving cell indicated for measurement, and

-	the CP type of the CSI-RS resource of neighbour cell configured for measurement is the same as the CP type of any of the CSI-RS resources of the serving cell indicated for measurement, and

-	It is applied for SCS = 60KHz

-	the centre frequency of the CSI-RS resource of the neighbour cell configured for measurement is the same as the centre frequency of any of the CSI-RS resources of the serving cell indicated for measurement.

A measurement is defined as an inter-frequency CSI-RS based L1 measurement if the intra-frequency condition is not met.

# Actions

To RAN1:

ACTION: 	RAN4 respectfully asks RAN1 to consider the RAN4 decision on intra-frequency and inter-frequency definition for CSI-RS based L1 measurement.

# References

[1] RP-234036, New WID: NR mobility enhancements Phase 4, Apple, RAN#102

[2] R4-2416862, WF on RRM requirements for NR_Mob_Ph4_Part1, Apple, RAN4#113.