# 3GPP TSG-RAN WG4 Meeting #113 　　　　　　　　　　　　　　　　　	R4- 2418583

Orlando, US, 18th  – 22nd November, 2024

Source: 	NTT DOCOMO, INC.

Title:	Discussion on CC activation based MPR improvement for FR2

Agenda item:	7.1.1.2.3

Document for:	Discussion

- Introduction

In Rel-19, the WI “UE RF enhancements for NR FR1/FR2 and EN-DC, Phase 4” [1], includes MPR reduction as one of its objectives. RAN4#112bis endorsed the draft CR for FR2 configuration-based MPR improvement [2] and agreed to further discuss CC activation-based MPR improvement in the WF [3]. This paper presents our perspective on these issues.

#### Issue 3-2-2: CC activation-based MPR improvement - WF - Further discuss activated based MPR improvement. - FFS using common UE capability between configuration based and activation based, or another UE capability for activated-based improvements. - FFS MPR can be determined based on activated UL CCs BW instead of Cumulative aggregated channel bandwidth (CABW). (i.e., activation based also applies to UL CA case) - FFS limitation the enhancement to cases where the activated UL CCs form a contiguous block.

- Discussion

UE capability for CC activation based MPR improvement

RAN4#112bis endorsed the draft CR for FR2 configuration-based MPR improvement [2]. As a second step, RAN4 could consider CC activation-based MPR improvement.

For CC activation-based MPR improvement, there may be implementation burdens associated with meeting the activation timeline. If one common UE capability is specified for both configuration-based and activation-based improvements, a UE that is capable of supporting only configuration-based improvement would not be able to support this feature. Considering these implementation challenges, it is preferable to specify two different UE capabilities for configuration-based and activation-based to encourage more UEs to support the feature. This issue was raised in R4-2404209 from Qualcomm in RAN4#110bis [4].

This argument applies not only to the case where multiple CCs are activated, as described below, but also to instances where only one UL CC is activated, which has already been agreed upon.

Proposal 1: For CC activation-based MPR improvement, another UE capability can be specified separately from the configuration-based if there are technical challenges related to implementation.

MPR applicability for UL activated CCs

Regarding WF [3], RAN4#112bis agreed that single carrier MPR requirements can be applied when only one UL CC is activated.

#### Issue 3-1-2: Applicable MPR for FR2 UL CA with DL intra band CA Agreement in main session: - In general, with UE indication of new capability [FR2 MPR enhancements-R19] for MPR improvement, MPR based on UL BWchannel_CA applies instead that based on cumulative aggregated channel BW (CABW). - If only 1 UL CC is activated, the MPR requirements of single carrier could be reused. - …

To further extend this activation-based MPR improvement, when multiple CCs are activated, the bandwidth of the activated CCs can be used as a bandwidth basis instead of the CABW.

Proposal 2: The bandwidth of activated UL CCs can be used as a bandwidth basis for MPR instead of CABW, even when multiple UL CCs are activated.

Whether UL CC should form a contiguous block

Regarding the previous proposal, there are potential cases where activated multiple CCs form a discontinuous block. This issue was raised in R4-2413226 from Qualcomm in RAN4#112 [5], but it has not been fully discussed. In order to support such cases, a complex specification would need to be developed, which may lead to problems with the readability of the provisions. However, such cases are considered to occur infrequently and it is therefore preferable to limit the discussion to cases where the activated UL CCs form a contiguous block. The proposal is also reasonable as non-contiguous CA is out of scope in configuration-based and limited to contiguous CA.

Proposal 3: The enhancement can be limited to cases where the activated UL CCs form a contiguous block.

- Conclusion

Here we summarize our proposals:

Proposal 1: For CC activation-based MPR improvement, another UE capability can be specified separately from the configuration-based if there are technical challenges related to implementation.

Proposal 2: The bandwidth of activated UL CCs can be used as a bandwidth basis for MPR instead of CABW, even when multiple UL CCs are activated.

Proposal 3: The enhancement can be limited to cases where the activated UL CCs form a contiguous block.

4	References

[1] RP-241656, “Revised WID: UE RF enhancements for NR FR1/FR2 and EN-DC, Phase 4”, RAN4 chair (Huawei)

[2] R4-2417110, “draft CR to TS 38.101-2 for FR2 configuration based MPR improvement”, Huawei, HiSilicon

[3] R4-2417111, “WF on power domain enhancements”, Huawei, HiSilicon

[4] R4-2404209, “Power boosting and MPR reduction”, Qualcomm

[5] R4-2413226, “Activation-based FR2 MPR enhancement”, Qualcomm