3GPP TSG-RAN WG4 Meeting #113	 R4-2418587

Orlando, USA, 18th – 22nd November, 2024

Agenda Item:	7.24.2.1

Source: 	Apple

Title:	Discussion on L1 event triggered measurement reporting

Document for:	Discussion

# Introduction

L1 event triggered measurement reporting has been widely discussed in previous RAN4 meeting. The latest agreements can be found in [1], according to which there are quite many open issues. In this contribution, we continue discussing the open issues.

# Discussion

Issue 1-1: RRM scope of Event triggered L1 measurement reporting

- Agreement:

  - RAN4 to define event-triggered L1 reporting requirement for SSB based and CSI-RS based measurements.

    - To start RAN4 discussion for SSB based L1 measurement firstly, and check how to apply/extend the agreements for SSB based L1 measurement to accommodate CSI-RS based L1 measurement.

  - From RAN4 RRM requirement perspective, for FR2-1, RRM requirements for multi-Rx based LTM L1 measurement is not considered in the WI scope.

  - RRM scope of Event triggered L1 measurement reporting

    - In addition to intra-frequency measurement and inter-frequency measurement without gap, support gap-based inter-frequency measurement for SSB-based L1 measurement. Further discuss for CSI-RS based L1 measurement.

Based on progress on the previous RAN4#112bis, SSB based L1 event triggered measurement reporting has already been confirmed to be supported in this WI. However, CSI-RS based L1 event triggered measurement reporting is still open. We would like to point out that RAN2 actually has already agreed that CSI-RS is also included:

RAN2#126: 4. Support the beam config of both SSB and CSI-RS in L1 measurement resource configuration in LTM config. Working assumption: Same RS type should be used for both serving and neighbouring cell for event LTM3 and event LTM5. RAN2#127bis: 8. Confirms WA (Same RS type should be used for both serving and neighbouring cell for event LTM3 and event LTM5).

Besides, from RAN4 requirement point of view, we don’t see too much extra effort to support CSI-based L1 reporting given that 1) requirements framework is likely to be same as that of SSB based L1 reporting; 2) CSI-RS based L1 measurement requirements will be defined in this work item under the dedicated agenda.

Observation 1: CSI-RS based L1 reporting is configured to be supported by RAN2.

Observation 2: RRM requirements framework for CSI-RS based event triggered L1 reporting is likely to be same as SSB based event triggered L1 reporting, and the later one is already confirmed to be defined in this work item.

Observation 3: CSI-RS based L1 measurement requirements will be defined in this work item under the dedicated agenda.

Proposal 1: confirm that CSI-RS based event triggered L1 reporting is included in RAN4 scope of this work item.

Regarding reporting type, event triggered periodic reporting is supported by RAN2:

RAN2#127bis: - Event triggered periodic MR can be supported, based on NW configuration.

Therefore, RAN4 shall at least define requirements for event triggered periodic reporting. Others can be FFS.

Proposal 2: RAN4 shall at least define requirements for event triggered periodic reporting. Others can be FFS.

Issue 1-3: Requirements for reporting criteria per measurement category

- Candidate solutions:

  - Option 1: (MTK)

    - Define a separate capability for supported number of events triggered L1 measurement reporting.

    - Focus on SSB based L1-RSRP measurement for event triggered reporting at first.

    - For SSB based event triggered L1-RSRP measurement reporting, regard the events related to different candidate cells on different frequency layers as different events. Discuss how to differentiate intra-frequency and inter-frequency events. Further discuss the following options:

      - Option 1: if the LTM-CSI-ReportConfig is linked to multiple cells on different frequency layer, then the number of events related to this LTM-CSI-ReportConfig should be counted no less than the number of frequency layers.

      - Option 2: inform RAN2 that the events related to different candidate cells on different frequency layers are regarded as different events and ask RAN2 to take this into consideration when design the signaling.

  - Option 2: discuss and confirm in RAN4#112bis that whether requirements for reporting criteria per measurement category needs to be updated to cover L1-RSRP measurement on neighbor cell. If needed, it is proposed to use the following table as baseline. (Apple)

| Measurement category | Ecat | Note |
| --- | --- | --- |
| Intra-frequency Note 1,2,3,4,5 | 9 | Events for any one or a combination of intra-frequency SS-RSRP, SSB based L1-RSRP, CSI-RS based L1 RSRP, SS-RSRQ, SS-SINR, CSI-RSRP, CSI-RSRQ, and CSI-SINR for NG-RAN intra-frequency cells |
| Inter-frequency Note 2,3,4,5 | 10 | Events for any one or a combination of inter-frequency SS-RSRP, SSB based L1-RSRP, CSI-RS based L1 RSRP, SS-RSRQ, SS-SINR, CSI-RSRP, CSI-RSRQ, and CSI-SINR for NG-RAN inter-frequency cells |
| Inter-RAT (E-UTRA FDD, E-UTRA TDD) Note 2,4,5 | 10 | Only applicable for UE with this (inter-RAT) capability. These reporting criteria apply for any E-UTRA carrier frequencies other than the carrier frequency of the E-UTRA PSCell or E-UTRA SCell. |
| Inter-RAT (E-UTRA FDD, E-UTRA TDD) RSTD Note 2,4,5 | 1 | Inter-RAT RSTD measurement reporting for UE supporting OTDOA; 1 report capable of minimum 16 inter-RAT cell measurements. Only applicable for UE with this (inter-RAT RSTD via LPP [22]) capability. These reporting criteria apply for any E-UTRA carrier frequencies other than the carrier frequency of the E-UTRA PSCell or E-UTRA SCell. |
| Inter-RAT (E-UTRA FDD, E-UTRA TDD) RSRP and RSRQ measurements for E-CID Note 2,4,5 | 1 | Inter-RAT RSRP and RSRQ measurements for E-CID reported to E-SMLC via LPP [22]. One report capable of at least in total 10 inter-RAT RSRP and RSRQ measurements. Applicable to UE capable of reporting inter-RAT RSRP and RSRQ to E-SMLC via LPP. These reporting criteria apply for any E-UTRA carrier frequencies other than the carrier frequency of the E-UTRA PSCell or E-UTRA SCell. |
| Intra-frequency RSSI and channel occupancy measurements with CCA Note 1,2,3 | 1 | One report capable of one RSSI and one channel occupancy measurements over a channel [TS 37.213] with CCA. Applicable for UE capable of performing and reporting RSSI and channel occupancy on carrier frequencies under CCA. |
| Inter-frequency RSSI and channel occupancy measurements with CCA Note 2,3 | 1 | One report capable of one RSSI and one channel occupancy measurements over a channel [TS 37.213] with CCA. Applicable for UE capable of performing and reporting RSSI and channel occupancy on carrier frequencies under CCA. |
| Intra-frequency SSB-based measurements for NR E-CID Note 1,2,3,4,5 | 1 | Intra-frequency SS-RSRP and SS-RSRQ measurements for NR E-CID reported to LMF via LPP [34]. One report capable of at least in total 9 intra-frequency SS-RSRP and SS-RSRQ measurements. Applicable to UE capable of reporting at least one of SS-RSRP and SS-RSRQ to LMF via LPP. |
| Intra-frequency CSI-RS based measurements for NR E-CID Note 1,2,3,4,5 | 1 | Intra-frequency CSI-RSRP and CSI-RSRQ measurements for NR E-CID reported to LMF via LPP [22]. One report capable of at least in total 9 intra-frequency CSI-RSRP and/or CSI-RSRQ measurements. Applicable to UE capable of reporting any of CSI-RSRP and CSI-RSRQ to LMF via LPP, as indicated in nr-ECID-MeasSupported-r16. |
| Inter-frequency SSB-based measurements for NR E-CID Note 2,3,4,5 | 1 | Inter-frequency SS-RSRP and SS-RSRQ measurements for NR E-CID reported to LMF via LPP [34]. One report capable of at least in total 10 inter-frequency SS-RSRP and SS-RSRQ measurements. Applicable to UE capable of reporting at least one of SS-RSRP and SS-RSRQ to LMF via LPP. |
| Inter-frequency CSI-RS based measurements for NR E-CID Note 2,3,4,5 | 1 | Inter-frequency CSI-RSRP and CSI-RSRQ measurements for NR E-CID reported to LMF via LPP [22]. One report capable of at least in total 10 inter-frequency CSI-RSRP and CSI-RSRQ measurements. Applicable to UE capable of reporting any of CSI-RSRP and CSI-RSRQ to LMF via LPP, as indicated in nr-ECID-MeasSupported-r16. |
| DL RSTD Note 2,4,5 | 1 | DL RSTD measurement reporting; 1 report capable of multiple (within the UE PRS measurement capability, nr-DL-TDOA-MeasCapability, indicated via LPP [34]) DL RSTD measurements and if supported also multiple corresponding DL PRS-RSRP measurements configured for DL-TDOA. Only applicable for UE capable of reporting measurements for DL-TDOA to LMF via LPP [34]. |
| UE Rx-Tx Note 2,4,5 | 1 | UE Rx-Tx measurement reporting; 1 report capable of multiple (within the UE PRS measurement capability, nr-DL-PRS-MeasCapability, indicated via LPP [34] for multi-RTT) UE Rx-Tx measurements and if supported also multiple corresponding DL PRS-RSRP measurements configured for multi-RTT. Only applicable for UE capable of reporting measurements for multi-RTT to LMF via LPP [34]. |
| DL PRS-RSRP Note 2,4,5 | 1 | DL PRS-RSRP measurement reporting; 1 report capable of multiple (within the UE PRS measurement capability, nr-DL-PRS-MeasCapability, indicated via LPP [34] for AoD) DL PRS-RSRP measurements configured for DL-AoD. Only applicable for UE capable of reporting measurements for DL-AoD to LMF via LPP [34]. |
| DL PRS-RSRPP Note 2,4,5 | 1 | DL PRS-RSRPP measurement reporting; 1 report capable of multiple (within the UE PRS measurement capability, nr-DL-PRS-MeasCapability, indicated via LPP [34] for AoD) DL PRS-RSRPP measurements configured for DL-AoD. Only applicable for UE capable of reporting measurements for DL-AoD to LMF via LPP [34]. |
| SRS-RSRP Note 2,3,4,5 | 1 | SRS-RSRP measurement reporting for CLI; 1 report capable of up to 32 SRS resources measurements. Only applicable for UE supporting cli-SRS-RSRP-Meas-r16. |
| CLI-RSSI Note 2,3,4,5 | 1 | CLI-RSSI measurement reporting for CLI; 1 report capable of up to 64 CLi-RSSI resources measurements. Only applicable for UE supporting cli-RSSI-Meas-r16. |
| NOTE 1: When the UE is configured with PSCell and SCell carrier frequencies, Ecat for Intra-frequency is applied per corresponding NR serving frequency. NOTE 2: Applicable for UE configured with SA NR operation mode. NOTE 3: Applicable for UE configured with EN-DC operation mode. NOTE 4: Applicable for UE configured with NE-DC operation mode. NOTE 5: Applicable for UE configured with NR-DC operation mode. |  |  |

- Option 3: RAN4 to introduce requirements of “Capabilities for Support of Event Triggering and Reporting Criteria” (OPPO)

We continue supporitng option 2. In previous meetings we encouraged companies to discuss whether it is necessary to update the requirements of Capabilities for Support of Event Triggering and Reporting Criteria specified in TS38.133 section 9.1.4 to cover event triggered L1 measurement reporting. Since new events have already been confirmed by RAN2, we think it is better to update the table (Table 9.1.4.2-1) for reporting criteria per measurement category.

Option 1 indeed raises some meaningful points that we need to consider. For instance,  RAN4 needs to discuss how to differentiate intra/inter-frequency events, which depends on conclusion of other issues. We are aslo fine to define a separate capability for supported number of events triggered L1 measurement reporting. Regarding the scope, we believe CSI-RS based event triggered L1 reporting shall also be included.

Proposal 3: confirm that requirements for reporting criteria per measurement category need be updated to include new events introduced by RAN2 on LTM.

Proposal 4: both SSB and CSI-RS based event triggered L1 reporting shall be included.

Proposal 5: details can be FFS depend on, e.g. whether and how to differentiate intra/inter-frequency.

Issue 1-4: How to coordinate with R19 MIMO

- Candidate solutions:

  - Option 1: (MTK)

    - Events configured for mobility and MIMO should be considered as different events even when they are configured for the same beams and the same cell with the same threshold.

    - Define the capability for L1 events shared by mobility L1 events and MIMO L1 events.

Procedure wise, we don’t think we need to discuss this in this work item. Although they are similar in some extent, they are still separate features. New events introduced by RAN1/2 for these two work items are also different.

Proposal 6: RAN4 doesn’t need to discuss how to coordinate with R19 MIMO in this work item, given that R19 MIMO and R19 LTM are two separate work items.

Issue 1-5: filtering assumption in event evaluation

- Candidate solutions:

  - Option 1: Wait for more RAN1 progress to discuss how to handle L1 filtering in RAN4 for event triggered L1 report (MTK, Apple, HW, Nokia, E///, Qualcomm, CATT).

  - Option 1a: For whether to consider the RRM impact due to L1 filtering for event triggered L1 report, RAN4 needs to wait for more RAN1 input. (CATT)

    - If L1 filtering is introduced in RAN1, RAN4 needs to define additional SSB based L1 measurement delay requirements with filtering for event triggered L1 report.

    - Otherwise, there is no impact on RAN4.

  - Option 2: For filtering, we think it is better to wait the conclusion of RAN1. If it is up to UE implementation, it is unnecessary to define the related requirements. (CTC)

    - Event triggered reporting delay is up to whether the filtering is needed, when filtering is used an additional delay can be expected.

    - In case without filtering, we think current LTM L1-RSRP measurement delay can be as baseline.

  - Option 3: no need to define additional SSB based L1 measurement delay requirements with filtering, since the agreement about filtering in RAN2 is about filtering with filter coefficient as defined in Clause 5.5.3.2, TS38.331 or about Time-to-Trigger (TTT), neither of them have impact on L1 measurement delay requirements. (CMCC)

RAN1 has already confirmed that no L1 specific filtering for time and spatial domain will be introduced.

RAN1 agreements in RAN1#118bis The agreement “Rel-18 LTM CSI reporting framework is the baseline for CSI-RS based L1-measurement report by gNB scheduled measurement reporting” made in RAN#118 is further clarified for L1-RSRP as follows: - UCI format defined in Table 6.3.1.1.2-8C of TS38.212 can be used by replacing SSBRI with CRI. - Whether the L1-RSRP(s) of serving cell is always included is configurable (in line with Rel-18) - The quantization method defined in clause 5.2.1.4.3 of TS38.214 and bit width defined in Table 6.3.1.1.2-6 of TS38.212 can be used - No L1 specified filtering for time and spatial domain is introduced - No enhancement on how to report L cells x M beams - Periodic reporting on PUCCH is supported - FFS: semi-persistent reporting on PUCCH/PUSCH, and aperiodic reporting on PUSCH

Proposal 7: confirm that L1 filtering is not considered in RAN4 requirements.

Issue 1-7: beam level or cell level measurement result for new LTM event evaluation

- Candidate solutions:

  - Option 1: For LTM event triggered L1 reporting, beam level measurement result will be used for evaluation. (Xiaomi, CMCC, Apple, Samsung)

In RAN2#127, it was agreed that:

Agreements on L1 MR event evaluation

1.     Event LTM1 is not defined.

2.     Current beam (i.e. a beam corresponding to the indicated TCI state) is used for event evaluation in L1 measurement reporting for serving cell.

3.     Any beam in candidate RS configuration can be used for LTM event evaluation.

4.     Beam level measurement result, not cell level measurement result, is used LTM event evaluation. FFS on the conditional LTM case.

5.     R19 LTM event triggered measurement configuration can be taken as baseline:

- LTM measurement resource configuration is provided in LTM-config.

- Event triggered report config is provided in serving cell config.

6. MAC layer handles the event evaluation and measurement report triggering.

Therefore, RAN4 doesn’t need to discuss this issue. The issue can be closed.

Issue 1-9: L1 event triggered reporting delay

- Candidate solutions:

  - Option 1: The existing framework in L3 event triggered reporting RRM requirements can be as a starting point. (CTC, Samsung, ZTE)

  - Option 2: RAN4 to discuss how to consider the total measurement time if multiple cells are measured with different measurement period. (Xiaomi)

    - It’s possible that the periodicity of RS is different from each other and then the measurement period is different. How to define total measurement time for multiple cells is FFS.

  - Option 3: (CMCC)

    - for SSB based event triggered L1 measurement reporting, it is proposed to define L1 measurement reporting delay requirements.

      - The delay is less than TL1-RSRP_Measurement_Period or TL1-RSRP_Measurement_Perioa + TSSB_time_index, which are the L1-RSRP measurement period defined for Rel-18 LTM.

    - for CSI-RS based event triggered L1 measurement reporting, it is proposed to define L1 measurement reporting delay requirements, and L1 measurement reporting accuracy requirements

  - Option 4: Reporting delay is defined as one L1-RSRP measurement period on target cell. (Apple)

  - Option 4a: The event triggered measurement reporting delay, shall be less than LTM L1-RSRP measurement period based on SSB or CSI-RS. (Samsung)

  - Option 5: SSB-based LTM L1 event triggered reporting delay is TL1-RSRP_Measurement_Period_SSB_intra, TL1-RSRP_Measurement_Period_SSB_inter or TL1-RSRP_Measurement_Period_SSB_inter_withoutGap (Nokia)

  - Option 6: RAN4 to discuss and define the event evaluation time (i.e., the minimum time required to complete the event evaluation and be ready to send on the first reporting occasion from the measurement occasion). (Ericsson)

Options are not far away from each other. We propose to discuss this issue using option 4 as baseline since it generally cover both SSB based and CSI-RS based event triggered L1 reporting.

Proposal 8: Reporting delay is defined as one L1-RSRP measurement period on target cell.

- For SSB based event triggered L1 reporting, existing measurement period defined in clause 9.14 and 9.15 can be reused, respectively for intra and inter-frequency.

- For CSI-RS based event triggered L1 reporting, whatever measurement period will be agreed under CSI-RS related agenda can be reused in this objective.

Issue 1-10: where to capture event triggered L1 report requirement

- Candidate solutions:

  - Option 1: new requirements for event triggered L1 measurement reporting shall be introduced in 9.14.3 and 9.15.3, respectively for intra-frequency and inter-frequency. (CATT, Apple, vivo, HW)

This issue shall not be controversial. It is just a guidence on where to capture the new requirements.

Proposal 9: confirm that new requirements for SSB based event triggered L1 measurement reporting shall be introduced in 9.14.3 and 9.15.3, respectively for intra-frequency and inter-frequency. For CSI-RS based event triggered L1 measurement reporting, it will be captured in the new section for requirements of CSI-RS based L1 measurement on neighbor cell.

Issue 1-11: Impact of UL resource waiting time

- Candidate solutions:

  - Option 1: RAN4 to discuss whether to consider the impact of UL resource waiting time on L1 reporting requirement. (Xiaomi)

This shall be same as legacy. It is already reflected in existing event triggered L3 measurement reporting.

Proposal 10: regarding impact of UL resource waiting time, it can be handled in the same way as that in requirements for event triggered L3 measurement reporting.

# Conclusion

In this contribution, we provide discussion on measurement related enhancements for LTM. After discussion, the following conclusions are provided:

Observation 1: CSI-RS based L1 reporting is configured to be supported by RAN2.

Observation 2: RRM requirements framework for CSI-RS based event triggered L1 reporting is likely to be same as SSB based event triggered L1 reporting, and the later one is already confirmed to be defined in this work item.

Observation 3: CSI-RS based L1 measurement requirements will be defined in this work item under the dedicated agenda.

Proposal 1: confirm that CSI-RS based event triggered L1 reporting is included in RAN4 scope of this work item.

Proposal 2: RAN4 shall at least define requirements for event triggered periodic reporting. Others can be FFS.

Proposal 3: confirm that requirements for reporting criteria per measurement category need be updated to include new events introduced by RAN2 on LTM.

Proposal 4: both SSB and CSI-RS based event triggered L1 reporting shall be included.

Proposal 5: details can be FFS depend on, e.g. whether and how to differentiate intra/inter-frequency.

Proposal 6: RAN4 doesn’t need to discuss how to coordinate with R19 MIMO in this work item, given that R19 MIMO and R19 LTM are two separate work items.

Proposal 7: confirm that L1 filtering is not considered in RAN4 requirements.

Proposal 8: Reporting delay is defined as one L1-RSRP measurement period on target cell.

- For SSB based event triggered L1 reporting, existing measurement period defined in clause 9.14 and 9.15 can be reused, respectively for intra and inter-frequency.

- For CSI-RS based event triggered L1 reporting, whatever measurement period will be agreed under CSI-RS related agenda can be reused in this objective.

Proposal 9: confirm that new requirements for SSB based event triggered L1 measurement reporting shall be introduced in 9.14.3 and 9.15.3, respectively for intra-frequency and inter-frequency. For CSI-RS based event triggered L1 measurement reporting, it will be captured in the new section for requirements of CSI-RS based L1 measurement on neighbor cell.

Proposal 10: regarding impact of UL resource waiting time, it can be handled in the same way as that in requirements for event triggered L3 measurement reporting.

# Reference

- R4-2416862, WF on RRM requirements for NR_Mob_Ph4_Part1, Apple