3GPP TSG-RAN WG4 Meeting #113	 R4-2418588

Orlando, USA, 18th – 22nd November, 2024

Agenda Item:	7.24.2.3

Source: 	Apple

Title:	Discussion on conditional Intra-CU LTM

Document for:	Discussion

# Introduction

In RANP#105, an updated WID for NR mobility enhancement phase 4 was approved in [1]. According to the update, conditional intra-CU LTM is to be supported in this release:

- Specify support of conditional Intra-CU LTM [RAN2, RAN3, RAN1] - Specify UE evaluated conditions for triggering LTM - Aim to support conditional LTM including subsequent LTM - Limit specifying the conditional LTM to the scenario where the UE is in non-DC - Checkpoint at RAN#107 to review the objective on whether Intra-CU conditional LTM can be specified to DC scenarios and if so, to which cases. RAN WG work to not start before this checkpoint

Correspondingly, RAN4 needs to specify RRM requirements to support the new procedure. Conditional LTM was preliminarily discussed in RAN4#112bis. The latest agreements can be found in the approved WF [2]. In this contribution, we further discuss the open issues.

# Discussion

## Scope of conditional LTM cell switch

The first issue we would like to discuss is about the applicable scenario. RAN4 agreed the following applicable scenarios:

Issue 1-1-2: The applicable scenarios for conditional LTM

< Agreement >

- Rel-19 CLTM requirements are applicable for NR SA scenarios defined in Rel-18 LTM

- PCell switch to a neighboring LTM candidate cell

- FR1 cell to FR1 cell

- FR1 cell to FR2 cell

- FR2 cell to FR2 cell

- FR2 cell to FR1 cell

- PCell switch to an LTM candidate cell that is a serving SCell in MCG

- FR1 cell to FR1 cell

- FR2 cell to FR2 cell

- Further discuss the DC case if it will be included in the WID based on further RAN plenary discussion.

Regarding PCell switch to LTM candidate cell that is a serving SCell in MCG, we would like to point out that if the CLTM is triggered by L1 measurement, then the LTM candidate cell shall be an active serving SCell in MCG. If CLTM is triggered by L3 measurement, then the LTM candidate cell could be either an active serving SCell or a deactivated SCell. Because if the CLTM is triggered by L1 measurement, Tmeasure for L1 needs to be included in CLTM cell switch delay requirements. However, RAN4 doesn’t have requirements for L1 measurement on deactivated SCell.

Proposal 1: in the agreed applicable scenario of “PCell switch to an LTM candidate cell that is a serving SCell in MCG”, clarify that if CLTM is triggered by L1 measurement (e.g. event LTM3 and LTM5) the scope is limited to active SCell.

Issue 1-1-3: Measurement requirements on conditional LTM

< Agreement >

- On whether L3 measurement based conditional LTM in FR1 can be supported

- RAN4 can further discuss this issue in FR1 and make recommendation to RAN2 if RAN4 consensus can be reached.

- As generic procedure for the objective led by RAN2, if any conclusion will be made in RAN2, RAN4 will follow the RAN2 conclusion unless any technical issue is identified.

In conditional LTM procedure, UE keeps evaluating conditions based on network configuration. Once conditions are met, UE automatically triggers LTM. In conditions evaluation phase, UE performs measurement same as legacy, e.g. based on R18 L1-RSRP measurement requirements. Thus no new measurement requirement is expected. New requirements for CSI-RS based L1 RSRP measurement are needed. However, they are expected to be discussed in the dedicated agenda. Note that in late stage of R18, L1-RSRP measurement and LTM cell switch were decoupled. To increase applicability of conditional LTM cell switch, it is proposed to consider L3 measurement in event configuration for conditional LTM. Note this has RAN2 impact. RAN4 shall inform RAN2 if this is agreeable. We also provide a draft LS in Annex.

Proposal 2: no new measurement requirements are needed under this agenda.

Proposal 3: since L1-RSRP measurement and LTM cell switch was decoupled in R18, L3 measurement shall also be considered in event configuration for conditional LTM.

Issue 1-1-4: Subsequent conditional LTM

< Way Forward > FFS the following proposals:

- Proposal 1: FFS whether there is any difference in the delay for initial and subsequent CLTM cell switch.

- Proposal 2: RAN4 should further clarify the scenario for conditional LTM including subsequent LTM.

- Proposal 3: The starting point of the subsequent conditional LTM cell switch delay is the time UE transmits RRC Reconfigurationcomplete message for the previous PCell.

- Proposal 4: Apart from the starting point, the subsequent conditional LTM cell switch delay can be discussed based on the conclusion of conditional LTM delay.

The relationship between CLTM and subsequent CLTM is quite similar with conditional PSCell change and subsequent conditional PSCell change. Based on the experience of the latter case, standardization effort is quite limited on top of the baseline procedure (CLTM in this case). Following the same framework, we have the following proposals:

Proposal 4: assuming subsequent CLTM is to be supported by RAN2, RAN4 requirements for subsequent CLTM can be defined using CLTM requirements as baseline with the following updates:

- The starting point of subsequent CLTM is the time UE transmits RRCReconfigurationcomplete message for the previous CLTM.

  - Note: RAN2 agreed that the LTM completion defined for Rel-18 intra-CU LTM is reused for conditional LTM.

- TEvent_DU is the delay uncertainty which is the time from when UE transmits RRCReconfigurationcomplete message for the previous CLTM until a condition exists at the measurement reference point which will trigger the subsequent CLTM.

Issue 1-1-5: The specification structure of the requirements for conditional LTM

< Way Forward > FFS the following options:

- Option 1: For specification structure, the requirements for conditional LTM can be defined in the clauses of Rel-18 LTM.

- Other option is not precluded.

Option 1 is ok. Specifically, new requirements for CLTM can be defined in a new sub-section 6.3.1A. New requirements for subsequent CLTM can be defined in a new sub-section 6.3.1B.

Proposal 5: new requirements for CLTM can be defined in a new sub-section 6.3.1A. New requirements for subsequent CLTM can be defined in a new sub-section 6.3.1B.

Issue 1-2-1: General aspects on Conditional LTM delay

< Way Forward > FFS the following options:

- Option 1: CHO requirements framework can be used as baseline.

  - Option 1a: Rel-18 LTM requirements framework can be used as references.

- Option 2: Revisit conditional LTM delay when other WGs achieve further conclusions.

- Option 3: Wait for RAN2 progress for defining cell switch delay requirements for conditional LTM.

- Option 4: RAN4 to define cell switch delay requirements for conditional PCell switch in intra-CU scenario.

- Option 5: Rel-18 UE capability for early ASN.1 decoding or fast RRC processing shall be supported for Rel-19 Conditional LTM.

## Conditional LTM cell switch delay

Issue 1-2-1: General aspects on Conditional LTM delay

< Way Forward > FFS the following options:

- Option 1: CHO requirements framework can be used as baseline.

  - Option 1a: Rel-18 LTM requirements framework can be used as references.

- Option 2: Revisit conditional LTM delay when other WGs achieve further conclusions.

- Option 3: Wait for RAN2 progress for defining cell switch delay requirements for conditional LTM.

- Option 4: RAN4 to define cell switch delay requirements for conditional PCell switch in intra-CU scenario.

- Option 5: Rel-18 UE capability for early ASN.1 decoding or fast RRC processing shall be supported for Rel-19 Conditional LTM.

Issue 1-2-2: The starting point of conditional LTM delay

< Way Forward > FFS the following options:

- Option 1: The starting point of conditional LTM delay is the end of the last TTI containing the RRC command for conditional LTM.

- Option 2: The starting point of conditional LTM delay is the time when UE transmits an RRCReconfigurationComplete message.

- Option 3: The starting point of CLTM cell switch delay depends on when the condition evaluation begins, which is up to RAN1/RAN2.

Issue 1-2-3: The ending point of conditional LTM delay

< Way Forward > FFS the following options:

- Option 1:

  - The ending point of conditional LTM delay is the time when the UE transmits the first UL message on the target cell.

  - FFS the support of RACH-based and/or RACH-less, which is up to RAN1/2 agreement.

  - It can be revisited if any technical issue will be identified considering the progress in other WGs.

- Other option is not precluded.

Issue 1-2-4: The components of conditional LTM delay

< Way Forward > FFS the following options:

- Option 1: DCLTM = TRRC + TEvent_DU + TTCI-state activate + Tmeasure + TCLTM_execution + Tinterrupt.

- Option 2: DLTM_Conditional = TEvent_DU + Tmeasure + TLTM_execution + TLTM-interrupt.

- Option 3: DCLTM = TRRC + TEvent_DU + Tmeasure + TLTM-interrupt + TCLTM_execution.

- Option 4: DCLTM = TLTM_event_DU + TLTM_measure + TLTM-processing + TLTM-IU.

  - need of TLTM-RRC-processing, Tfirst-RS, TRS-proc and any additional components in the cell switch delay depends on when DL sync and ASN.1 decoding are to happen, which may depend on RAN1/2 CLTM design.

- Option 5: At least the delay uncertainty (TEvent_DU), the measurements time (Tmeasure) and the conditional execution preparation time (TCLTM_execution) also need to be considered for Rel-19 conditional LTM Cell Switch delay.

Issue 1-2-5: TRRC

< Way Forward > FFS the following options:

- Option 1: TRRC is the RRC procedure delay defined in clause 12 in TS 38.331.

- Other option is not precluded.

Issue 1-2-6: TEvent_DU

< Way Forward > FFS the following options:

- Option 1: TEvent_DU is the delay uncertainty which is the time from when the UE successfully decodes a conditional LTM command until a condition exists at the measurement reference point which will trigger the conditional LTM.

- Option 2: TEvent_DU is the delay uncertainty which is the time from when UE transmits RRCReconfigurationComplete message confirming the LTM configuration or the RRCReconfigurationcomplete message for the previous cell switch until a condition exists at the measurement reference point which will trigger the conditional LTM.

- Option 3: Wait for more progress in RAN2.

Issue 1-2-7: Tmeasure

< Way Forward > FFS the following options:

- Option 1: Tmeasure is measurement time.

  - Option 1a: Tmeasure is defined from the end of TTCI-state activate until UE executes LTM to a target cell and interruption time starts.

  - Option 1b: Tmeasure is the measurements time. It is defined from the end of TEvent_DU until UE executes  LTM cell switch to a target cell and interruption time starts.

- If SSB based L1-RSRP measurement is used in the event, Tmeasure is same as one measurement period of SSB based L1-RSRP measurement as introduced in R18.

- If CSI-RS based L1-RSRP measurement is used in the event, Tmeasure is same as one measurement period of CSI-RS based L1-RSRP measurement which will be introduced in R19.

- If L3 measurement is used in the event, Tmeasure is same as one measurement period of legacy L3 measurement.

- Option 2: Wait for more progress in RAN2.

Issue 1-2-8: TCLTM_execution / TLTM_execution

< Way Forward > FFS the following options:

- Option 1: TCLTM_execution / TCLTM_execution is the UE execution preparation time for conditional LTM and starts after UE realizes the condition of LTM is met and identity of the target cell is determined.

  - Option 1a: TCLTM_execution can be up to [10ms].

- Option 2: TCHO_execution may need update in conditional LTM.

Issue 1-2-9: Tinterrupt

< Way Forward > FFS the following options:

- Option 1: Tinterrupt is the time between when the UE starts to execute the conditional LTM to the target cell and the time the UE starts to transmit the first UL message on the target cell.

- Option 2: TLTM-interrupt is the interruption time as specified in Rel-18 LTM cell switch.

- Option 3: Tinterrupt  may induce ambiguity when define conditional LTM delay, as interruptions may also happen in early PRACH procedure (if supported in conditional LTM).

- Option 4: The interruption time defined in Rel-18 LTM Cell Switch delay can be a baseline.

- Option 5: RAN4 to discuss the starting point of the CLTM interruption, and the components included in TCLTM_interrupt. The goal shall be that CLTM interruption is shorter than CHO interruption.

Issue 1-2-10: TTCI-state activate

< Way Forward > FFS the following options:

- Option 1: TTCI-state activate is the time for active the TCI state of candidate cell(s).

- Other option is not precluded.

Issue 1-2-11: The components of Tinterrupt

< Way Forward > FFS the following options:

- Option 1: Tinterrupt = TLTM-processing + Tfirst-RS + TRS-proc + TLTM-IU.

- TLTM-processing is the time for UE processing, consisting of applying the target cell parameters and L1/L2 change.

- Tfirst-RS is the time for fine time tracking and acquiring full timing information of the target cell.

- TRS-proc is the time for SSB processing.

- TLTM-IU is the interruption uncertainty during LTM cell switch.

In general we can borrow the design spirit of CHO. Corresponding requirements are very stable now. Conditional LTM cell switch is similar with conditional handover. CHO requirements framework can be used as baseline when discussing conditional LTM cell switch delay requirements.

Proposal 6: CHO requirements framework can be used as baseline when discussing conditional LTM cell switch delay requirements.

The starting point could be the end of the last TTI containing the RRC command for conditional LTM. The ending point could be same as that defined in existing LTM cell switch delay requirements, i.e. time the UE transmits the first UL message on the target cell, which naturally already cover both RACH based and RACH-less.

Proposal 7: The starting point is the end of the last TTI containing the RRC command for conditional LTM.

Proposal 8: The ending point could be same as that defined in existing LTM cell switch delay requirements, i.e. time the UE transmits the first UL message on the target cell, which already cover both RACH based and RACH-less.

Therefore, the following formular is proposed as conditional LTM cell switch delay:

DCLTM = TRRC + TEvent_DU + Tmeasure + TLTM-interrupt + TCLTM_execution

Where:

TRRC is the RRC procedure delay defined in clause 12 in TS 38.331.

TEvent_DU is the delay uncertainty which is the time from when the UE successfully decodes a conditional LTM cell switch command until a condition exists at the measurement reference point which will trigger the conditional LTM cell switch.

Tmeasure is the measurements time. It is defined from the end of TEvent_DU until UE executes an LTM cell switch to a target cell and interruption time starts:

-	if SSB based L1-RSRP measurement is used in the event, Tmeasure is same as one measurement period of SSB based L1-RSRP measurement as introduced in R18.

-	if CSI-RS based L1-RSRP measurement is used in the event, Tmeasure is same as one measurement period of CSI-RS based L1-RSRP measurement which will be introduced in R19.

-	if L3 measurement is used in the event, Tmeasure is same as that defined in legacy CHO.

TCLTM_execution is the UE execution preparation time for conditional LTM cell switch, and starts after UE realizes the condition of CLTM is met and identity of the target cell is determined. TCLTM_execution can be up to [10ms].

TLTM-interrupt is the interruption time stated in clause 6.3.1.3, same as that defined in R18 LTM cell switch.

Proposal 9: conditional LTM cell switch delay is defined as:

DCLTM = TRRC + TEvent_DU + Tmeasure + TLTM-interrupt + TCLTM_execution

Where:

TRRC is the RRC procedure delay defined in clause 12 in TS 38.331.

TEvent_DU is the delay uncertainty which is the time from when the UE successfully decodes a conditional LTM cell switch command until a condition exists at the measurement reference point which will trigger the conditional LTM cell switch.

Tmeasure is the measurements time. It is defined from the end of TEvent_DU until UE executes a LTM cell switch to a target cell and interruption time starts:

-	if SSB based L1-RSRP measurement is used in the event, Tmeasure is same as one measurement period of SSB based L1-RSRP measurement as introduced in R18.

-	if CSI-RS based L1-RSRP measurement is used in the event, Tmeasure is same as one measurement period of CSI-RS based L1-RSRP measurement which will be introduced in R19.

-	if L3 measurement is used in the event, Tmeasure is same as one measurement period of legacy L3 measurement.

TCLTM_execution is the UE execution preparation time for conditional LTM cell switch, and starts after UE realizes the condition of CLTM is met and identity of the target cell is determined. TCLTM_execution can be up to [10ms].

TLTM-interrupt is the interruption time stated in clause 6.3.1.3, same as that defined in R18 LTM cell switch.

## Early synchronization and TCI state activation

Issue 1-3-1: Early DL sync

< Way Forward > FFS the following options:

- Option 1: R18 LTM early DL-sync can be as baseline.

- Option 2: FFS whether new requirements due to CLTM specific DL/UL sync procedures are needed.

- Option 3: Wait for RAN2 further agreement on the framework.

Issue 1-3-2: Early UL sync

< Way Forward > FFS the following options:

- Option 1: For UL-sync, we can consider some situations as follows,

- Do not need to acquire the TA, i.e., the NTA of candidate cell(s) is/are same as serving cell or NTA = 0.

- RAR-based TA acquisition.

- UE based TA measurement.

- Option 2: How/whether to support early RACH on candidate cells during conditional LTM.

- Option 3: Whether updates are needed to the Rel-18 PDCCH ordered RACH delay requirements.

- Option 4:  If DL TODA-based UE autonomous TA estimation is found beneficial for the conditional LTM cell switch, RAN4 to define the requirements for DL TODA-based UE autonomous TA estimation.

- Option 5: Wait for more progress from other WGs.

Issue 1-3-3: Early TCI state activation

< Way Forward > FFS the following options:

- Option 1: How/whether to support early TCI activation on candidate cells during conditional LTM.

- Option 2: FFS whether updates are needed to the Rel-18 early TCI state activation delay requirements.

- Option 3: R18 LTM early TCI state activation can be as baseline.

- Option 4: Wait for more progress from other WGs.

This part can be postponed until RAN2 design becomes stable. If existing procedure is reused, then existing RAN4 requirements can be reused, since they are defined separately outside LTM cell switch requirements.

## Others

Issue 1-4-1: Whether cover both RACH-based and RACH-less conditional LTM

< Way Forward > FFS the following options:

- Option 1: Cover both RACH-based and RACH-less conditional LTM.

- Option 2: FFS whether both RACH-less and RACH-based CLTM are supported by RAN1/2.

Issue 1-4-2: Whether to cover both known and unknown cell for conditional LTM

< Way Forward > FFS the following options:

- Option 1: The known cell condition for conditional LTM can be revised as, during the last 5 seconds before the reception of the RRC command that contain conditional LTM configuration…….

- Option 2: All the cell(s) configured as candidate cell(s) for conditional LTM is/are known.

RAN2 already confirmed that both RACH-less and RACH-based CLTM are supported:

Agreements on C-LTM - Source cell sends the conditional LTM configuration via RRCReconfiguration to UE, which includes the LTM candidate configurations, and the corresponding execution conditions. - Event LTM3-like and LTM5-like are used as the conditional LTM execution condition. FFS on reuse of CHO conditions. - Source cell and each candidate cell provides its own execution condition for conditional LTM. - It is DU to generate the L1 execution condition. FFS on a case that L3 measurement is used. - RACH-less Conditional intra-CU LTM is supported. - RACH based conditional intra-CU LTM is supported. - UE based TA measurement mechanism is supported for conditional intra-CU LTM. - PDCCH ordered early TA acquisition is supported for conditional LTM. - Rel-18 Early candidate TCI State activation/deactivation is supported for conditional intra-CU LTM. - For RACH-less conditional LTM, CG-based first UL transmission on target cell is supported. FFS on DG-based approach. - The LTM completion defined for Rel-18 intra-CU LTM is reused for conditional LTM.

So issue 1-4-1 can be concluded. Regarding issue 1-4-2, it is straightforward that only known cell can trigger CLTM since it is triggered once UE realizes the events are met after measurement.

Proposal 10: RAN4 requirements for CLTM shall cover both RACH-less and RACH-based cell switch.

Proposal 11: only known cell needs to be covered in CLTM RRM requirements.

# Conclusion

In this contribution, we provide discussion on measurement related enhancements for LTM. After discussion, the following conclusions are provided:

Proposal 1: in the agreed applicable scenario of “PCell switch to an LTM candidate cell that is a serving SCell in MCG”, clarify that if CLTM is triggered by L1 measurement (e.g. event LTM3 and LTM5) the scope is limited to active SCell.

Proposal 2: no new measurement requirements are needed under this agenda.

Proposal 3: since L1-RSRP measurement and LTM cell switch was decoupled in R18, L3 measurement shall also be considered in event configuration for conditional LTM.

Proposal 4: assuming subsequent CLTM is to be supported by RAN2, RAN4 requirements for subsequent CLTM can be defined using CLTM requirements as baseline with the following updates:

- The starting point of subsequent CLTM is the time UE transmits RRCReconfigurationcomplete message for the previous CLTM.

  - Note: RAN2 agreed that the LTM completion defined for Rel-18 intra-CU LTM is reused for conditional LTM.

- TEvent_DU is the delay uncertainty which is the time from when UE transmits RRCReconfigurationcomplete message for the previous CLTM until a condition exists at the measurement reference point which will trigger the subsequent CLTM.

Proposal 5: new requirements for CLTM can be defined in a new sub-section 6.3.1A. New requirements for subsequent CLTM can be defined in a new sub-section 6.3.1B.

Proposal 6: CHO requirements framework can be used as baseline when discussing conditional LTM cell switch delay requirements.

Proposal 7: The starting point is the end of the last TTI containing the RRC command for conditional LTM.

Proposal 8: The ending point could be same as that defined in existing LTM cell switch delay requirements, i.e. time the UE transmits the first UL message on the target cell, which already cover both RACH based and RACH-less.

Proposal 9: conditional LTM cell switch delay is defined as:

DCLTM = TRRC + TEvent_DU + Tmeasure + TLTM-interrupt + TCLTM_execution

Where:

TRRC is the RRC procedure delay defined in clause 12 in TS 38.331.

TEvent_DU is the delay uncertainty which is the time from when the UE successfully decodes a conditional LTM cell switch command until a condition exists at the measurement reference point which will trigger the conditional LTM cell switch.

Tmeasure is the measurements time. It is defined from the end of TEvent_DU until UE executes a LTM cell switch to a target cell and interruption time starts:

-	if SSB based L1-RSRP measurement is used in the event, Tmeasure is same as one measurement period of SSB based L1-RSRP measurement as introduced in R18.

-	if CSI-RS based L1-RSRP measurement is used in the event, Tmeasure is same as one measurement period of CSI-RS based L1-RSRP measurement which will be introduced in R19.

-	if L3 measurement is used in the event, Tmeasure is same as one measurement period of legacy L3 measurement.

TCLTM_execution is the UE execution preparation time for conditional LTM cell switch, and starts after UE realizes the condition of CLTM is met and identity of the target cell is determined. TCLTM_execution can be up to [10ms].

TLTM-interrupt is the interruption time stated in clause 6.3.1.3, same as that defined in R18 LTM cell switch.

Proposal 10: RAN4 requirements for CLTM shall cover both RACH-less and RACH-based cell switch.

Proposal 11: only known cell needs to be covered in CLTM RRM requirements.

# Reference

- RP-242356, Revised Work Item: NR mobility enhancements Phase 4, Apple Inc, China Telecom

- R4-2416863, WF on RRM requirements for NR_Mob_Ph4_Part2, China Telecom

# Annex

3GPP TSG-RAN WG4 Meeting #113						     R4-24xxxxx

Orlando, USA, 18th – 22nd November, 2024

Title:	LS to RAN2 on conditional LTM cell switch

Response to:

Release:	Release - 19

Work Item:	NR_Mob_Ph4-Core

Source:	RAN4

To:	RAN2

Cc:

Contact Person:

#### Name:	Qiming Li

E-mail Address:	< li_qiming@apple.com>

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	none

1. Overall Description:

Conditional LTM cell switch was extensively discussed in RAN4#113. After discussion, RAN4 reached the following agreements:

- It is feasible from RAN4 perspective that L3 measurement can be used to trigger conditional LTM cell switch.

2. Actions:

To RAN2.

ACTION: 	RAN4 kindly asks RAN2 to take above information into consideration in future design check if there is any issue to support conditional LTM based on L3 measurement.

3. Date of Next TSG-RAN WG4 Meetings:

TSG RAN WG4 Meeting #114		              17 - 21 February 2025 			Athens, Greece

TSG RAN WG4 Meeting #114bis		              7 - 1 April 2025				China