3GPP TSG-RAN WG4 Meeting #113	 R4-2418589

Orlando, USA, 18th – 22nd November, 2024

Agenda Item:	7.25.2

Source: 	Apple

Title:	Discussion of RRM impact of XR for NR Phase 3

Document for:	Discussion

# Introduction

RRM impact of XR for NR Phase 3 has been widely discussed in previous RAN4 meetings. The latest agreements can be found in [1], according to which there are quite many open issues. In this contribution, we continue discussing the open issues.

# Discussion

The first issue we would like to discuss is about RAN1 LS on minimum time offset(s) X between indication to skip and skipped measurement occasion:

Working Assumption For solutions based on triggering/enabling by network signaling to enable Tx/Rx in gaps/restrictions that are caused by RRM measurements select the following option: - Alt. 1: Dynamic indication to enable Tx/Rx in particular gap/restriction that are caused by RRM measurements. - Alt 1-1: Explicit indication by DCI to skip a particular gap/restriction; - Indication is included as part of scheduling DCI: - Bit-field size is one bit; - The bit in the DCI is used to indicate whether to skip the first gap/restriction occasion after a minimum time offset required between the last symbol of the PDCCH carrying the DCI format and the start of corresponding skipped gap/restriction occasion indicated by the DCI. Send an LS to RAN4 to inform them of the above working assumption and ask them if there is any issue with it. Agreement If Alt. 1 from RAN1#117 agreement is supported, minimum time offset(s) X between indication to skip and skipped measurement occasion is up to RAN4 to discuss and decide on particular value(s).

RAN1 is asking the minimum time offset X between DCI and the skipped measurement occasion. Take gap cancellation for example:

In above example, the second gap occasion is already planed if there is no DCI. UE already canceled data Rx/Tx for the second gap occasion. Now with the new DCI received, UE needs to cancel the second gap and resume data Rx/Tx. Some lead time is necessary from UE implementation point of view. Regarding the exact value, RAN4 has a similar procedure, Pre-MG deactivation, which was introduced in R17. The deactivation delay is 5ms after the event. Take BWP switching for example, total Pre-MG deactivation delay is 5ms after BWP switching completes. BWP switching delay is 1ms and 3ms for type 1 and type 2 UE, repsectively. Thus total delay of Pre-MG deactivation is 6~8ms. To simplify RAN4 design, here we propose a single value 7ms for all UE.

Observation 1: Pre-MG deactivated delay is 6~8ms per RAN4 requirements.

Proposal 1: as baseline, 7ms minimum time offset is needed between indication to skip and skipped measurement occasion.

Issue 1-1: Deployment scenarios

< Agreement>:

- Frequency ranges

  - Agree on FR1 and FR2-1

- Scenarios

  - Start the discussion for

    - NR-SA

    - Carrier aggregation

<Way forward>:

- Further discussion needed for

  - FR2-2

  - NR-DC, EN-DC, NE-DC

We don’t have strong view on FR2-2 but it would be good to have it in the scope. Regarding DC, if only dynamic solution is supported in this work item, it would be quite challenging to support DC. Note that MG sometimes apply to both MCG and SCG. When MN/SN uses DCI to cancel some MG occasions,  SN/MN cannot realize that timely.

Observation 2: if only dynamic solution is supported, it could be challenging to enable it in DC scenario.

If RRC based solution is supported, then UE under DC scenario can also enjoy this feature since RRC based solution is stable and can be easily exchanged between MN and SN.

Proposal 2: In addition to DCI-based MG skipping control, RRC-based MG skipping control should also be supported, especially for DC scenario.

Issue 1-2: Types of measurements to be considered

<Way forward>:

- Option 1: Agree at least on the following and further discuss other scenario

  - Intra-frequency

  - Inter-Frequency

  - LTE inter-RAT

- Option 2: Agree at least on the following and further discuss other scenario

  - Intra-frequency

  - Inter-Frequency

  - LTE inter-RAT

  - XR requirement do not apply for UE configured to measure UTRAN FDD for SRVCC or 2G/3G for EN-DC/NE-DC

- Option 3: include all related scenarios

It would be easier for UE to handle the gap cancellation if the gap occasion is assigned only for NR measurement. Considering LTE network is still being widely used, it would be good to support this feature when LTE inter-RAT measurement is configured. However,  cancelling measurement gap which has already been assigned for LTE inter-RAT measurement may take longer time. Regarding 2G/3G measurement, we don’t see the strong need to support them in XR scenario.

Proposal 3: the following types of measurement can be considered in this work item:

  - Intra-frequency

  - Inter-Frequency

  - LTE inter-RAT

Issue 1-3: L3 Measurements without gaps with scheduling restrictions

<Way forward>:

- Option 1: Support skipping of measurements without gaps

- Option 2: Measurement skipping does not apply for measurements without gaps

Issue 1-4: L1 measurements with scheduling restrictions and procedures

<Way forward>:

- Option 1: Support skipping of L1 measurements and procedures such as RLM, BFD, L1-RSRP etc

- Option 2: Measurement skipping does not apply for L1 measurements and procedures

We would like to highlight that intra-frequency handover is the most typical handover in real deployment, wherein UE is not configured with measurement gap. Therefore, if we only focus on the case with measurement gap configured, we cannot benefit from this feature in most typical scenario.

Observation 3: intra-frequency handover without gap is the most typical handover in real deployment, wherein UE is not configured with measurement gap.

From feasibility point of view, we already have mechanisms supported in standard to relax RRM measurement, RLM and BFD under certain conditions like power saving. It is straightforward to us that at least we can let NW keep scheduling UE, as necessary for XR traffic, when UE is not actually performing RRM measurement, RLM and BFD due to relaxation. Besides, we can also consider exchanging measurement order information between UE and NW so that NW can know whether there is scheduling restriction on each measurement occasion, when there are multiple carriers to measure. Thereby, NW can perform measurement scheduling restriction cancellation precisely.

Especially when after gap cancellation, there is still scheduling restriction due to SMTC or other L1-RS. Without any optimization, UE is still expected to perform RRM measurement on those SSB(s) within SMTC, which is overlapping with gap that is cancelled due to XR traffic.

However, if we can support measurement cancellation also on SMTC, then legacy scheduling restrictions do NOT apply on the SSB to measure within SMTC and we can enjoy data after gap cancellation.

We understand that some companies may still have concern on this. However, there are UE vendor, NW vendor and operator would like to support skipping of measurements without gaps. It is fair to consider some optional UE capability to support this.

Observation 4: we already have mechanisms supported in standard to relax RRM measurement, RLM and BFD under certain conditions. At least it is feasible to schedule UE on RRM, RLM and BFD occasions which would cause scheduling restriction but UE is not actually performing measurement.

Proposal 4: it is beneficial and feasible to support data Tx/Rx in occasions of scheduling restriction due to RRM measurement outside measurement gap.

Proposal 5: it is beneficial and feasible to support data Tx/Rx in occasions of scheduling restriction due to RLM and BFD.

Proposal 6: introduce an optional UE capability to support skipping of measurements without gaps.

Issue 2-1: General approach for defining requirements for measurements with gaps

<Way forward>:

- Option 1: RAN4 to deprioritize the discussion on applicable type of MGs at early stage of the WI, and focus on the solutions/impacts on RRM requirements. Unless, it is found that a particular type of MG cannot be supported or need special treatment, RAN4 can discuss whether the feature can apply to this MG type.

- Option 2: As a baseline RAN4 to define requirements due to measurement skipping based on Type 1 measurement gap. Other gaps are FFS.

Issue 2-2: Types of measurement gaps

<Way forward>:

- In this issue we discuss the types of measurement gaps to be considered in this WID. In order to help the discussion, we reuse the definition of Type-1 and Type-2 measurement gaps as agreed in Rel-18 NR_MG-enh2:

- Type-1 MG: Gap(s) configured via GapConfig without suffix - Type-2 MG: Gap(s) configured via GapConfig-r17 without preConfigInd-r17 or ncsgInd-r17 - Rel-18 Concurrent Gap: gap with assigned priority which can be also NCSG or Pre-MG

- Companies may bring proposals regarding the priorities of each gap type for RAN4 to work based on the following table

| Cap types | First prio | Second prio | Don’t include |
| --- | --- | --- | --- |
| Type 1 |  |  |  |
| Type 2 |  |  |  |
| NCSG |  |  |  |
| MUSIM |  |  |  |
| Positioning |  |  |  |
| Rel 18 concurrent gap |  |  |  |
| PreConfigured |  |  |  |

We continue supporting the following types:

- Type-1 gap

- Type-2 gap

- NCSG

- concurrent gaps

- pre-MG

Proposal 7: priorities of each gap type:

| Cap types | First prio | Second prio | Don’t include |
| --- | --- | --- | --- |
| Type 1 | √ |  |  |
| Type 2 | √ |  |  |
| NCSG | √ |  |  |
| MUSIM |  | √ |  |
| Positioning |  |  | √ |
| Rel 18 concurrent gap | √ |  |  |
| PreConfigured | √ |  |  |

Issue 3-1: How to capture impact of measurement skipping

<Way forward>:

- Option 1:

  - As minimum requirements, cell detection, index reading, and measurement period is extended.

    - FFS how to extend

  - Reuse existing measurement accuracy requirements.

- Option 2:

  - For the cases where measurement impact is needed

    - As minimum requirements, cell detection, index reading, and measurement period for NR cells is extended.

      - FFS how to extend.

    - Reuse existing measurement accuracy requirements.

  - For the cases where measurement impact is not needed

    - FFS the feasible conditions and necessary UE assistance information.

Issue 3-2: Principles for measurement delay calculation

<Way forward>:

- Option 1: Follow the scaling factor (Ntotal/Navailable) from Concurrent MG spec to extend the total measurement delay.

- Option 2: Follow NR-U like approach to extend the delays by the number of MGs skipped during the measurement period.

- Other options are not precluded

It is straightforward that measurement cancellation has negative impact on measurement. In order to maintain the same measurement performance, UE still needs same number of measurement samples in physical layer. Assuming during the measurement period, network cancels X measurement occasions, then the total measurement delay shall be extended by X measurement occasions. Take inter-frequency measurement with gap for example:

Proposal 8: as minimum requirements, measurement period is extended by number of occasions which are cancelled due to XR traffic.

Note that if too many measurement occasions are cancelled on target carrier (UE cannot measure it for a long time), UE may need to adjust AGC and timing again before it can perform measurement on the carrier successfully. From UE measurement point of view, this is similar to measurement cancellation due to LBT failure. Similarly, max number of occasions shall be defined within a measurement period. If actual number of cancelled occasions exceeds the maximum value, UE shall be allowed to restart a measurement period.

Proposal 9: max number of occasions cancelled during a measurement period shall be defined.

The last thing we would like to discuss is about UE assistance information. UE knows better about measurement performance. For instance, when UE is in low mobility or not-at-cell-edge, UE can tolerate more measurement cancellation. When UE is at cell edge, UE doesn’t expect too many measurements cancellation. A simple solution is to let UE indicates NW a status whether it can tolerate measurement cancellation.

Proposal 10: UE assistance information is helpful for NW to decide whether and how many measurement occasions can be cancelled, e.g. UE indicates NW a status whether it can tolerate measurement cancellation.

# Conclusion

In this contribution, we provide discussion on RRM impact of XR for NR phase 3. After discussion, the following conclusions are provided:

Observation 1: Pre-MG deactivated delay is 6~8ms per RAN4 requirements.

Proposal 1: as baseline, 7ms minimum time offset is needed between indication to skip and skipped measurement occasion.

Observation 2: if only dynamic solution is supported, it could be challenging to enable it in DC scenario.

Proposal 2: In addition to DCI-based MG skipping control, RRC-based MG skipping control should also be supported, especially for DC scenario.

Proposal 3: the following types of measurement can be considered in this work item:

  - Intra-frequency

  - Inter-Frequency

  - LTE inter-RAT

Observation 3: intra-frequency handover without gap is the most typical handover in real deployment, wherein UE is not configured with measurement gap.

Observation 4: we already have mechanisms supported in standard to relax RRM measurement, RLM and BFD under certain conditions. At least it is feasible to schedule UE on RRM, RLM and BFD occasions which would cause scheduling restriction but UE is not actually performing measurement.

Proposal 4: it is beneficial and feasible to support data Tx/Rx in occasions of scheduling restriction due to RRM measurement outside measurement gap.

Proposal 5: it is beneficial and feasible to support data Tx/Rx in occasions of scheduling restriction due to RLM and BFD.

Proposal 6: introduce an optional UE capability to support skipping of measurements without gaps.

Proposal 7: priorities of each gap type:

| Cap types | First prio | Second prio | Don’t include |
| --- | --- | --- | --- |
| Type 1 | √ |  |  |
| Type 2 | √ |  |  |
| NCSG | √ |  |  |
| MUSIM |  | √ |  |
| Positioning |  |  | √ |
| Rel 18 concurrent gap | √ |  |  |
| PreConfigured | √ |  |  |

Proposal 8: as minimum requirements, measurement period is extended by number of occasions which are cancelled due to XR traffic.

Proposal 9: max number of occasions cancelled during a measurement period shall be defined.

Proposal 10: UE assistance information is helpful for NW to decide whether and how many measurement occasions can be cancelled, e.g. UE indicates NW a status whether it can tolerate measurement cancellation.

# Reference

- R4-2416864, WF on RRM requirements for XR_Ph3, Nokia