# 3GPP TSG-RAN WG4 Meeting #113                               	R4-2418596

# Orlando FL, USA, November 18 – 22, 2024

Agenda item:	7.23.4.1

Source:	Apple

Title:	Simulation Assumptions and Results for LP-WUR Core Requirements

Document for:	Discussion

# Introduction

Relevant objectives of the work item to RRM are the following [1]:

- To specify an LP-WUS design commonly applicable to both IDLE/INACTIVE and CONNECTED modes (RAN1, RAN4)

  - Specify OOK (OOK-1 and/or OOK-4) based LP-WUS with overlaid OFDM sequence(s) over OOK symbol

    - The LP-WUS design shall ensure that for IDLE/INACTIVE operation, the same information is delivered irrespective of LP-WUR type. The OFDM sequence can carry information.

  - At least duty-cycled monitoring of LP-WUS is supported

- For IDLE/INACTIVE modes

  - Specify procedure and configuration of LP-WUS indicating paging monitoring triggered by LP-WUS, including at least configuration, sub-grouping and entry/exit condition for LP-WUS monitoring (RAN2, RAN1, RAN3, RAN4)

  - Specify LP-SS with periodicity with Yms for LP-WUR, for synchronization and/or RRM for serving cell. (RAN1, RAN4)

    - LP-SS is based on OOK-1 and/or OOK-4 waveform with or without overlaid OFDM sequences. Further down selection between with and without overlaid OFDM sequences is to be done within WI.

    - Note: For LP-WUR that can receive existing PSS/SSS, existing PSS/SSS can be used for synchronization and RRM instead of LP-SS.

    - Y will be decided within WI. 320ms is the start point.

  - Specify further RRM relaxation of UE MR for both serving and neighbor cell measurements, and UE serving cell RRM measurement offloaded from MR to LP-WUR, including the necessary conditions (RAN4, RAN2)

In last RAN4#112bis meeting, the WF for RRM requirements for LP-WUS/WUR was agreed in [2], and open issues have been summarized in [3] and [4]. In this contribution, we continue the discussion on simulation assumptions, provide preliminary results, and propose a framework for RAN4 RRM to manage the consolidation of companies’ simulation results moving forward.

# Methodology

RAN4 RRM has continued discussing simulation assumptions and parameters with the objective of defining relevant RRM requirements for LP-WUS/WUR once RAN1 design is finished. At the time of this discussion, design on LP-SS sequence design, duration and modulation has not yet been agreed. However, RAN4 has decided to start the RRM requirements discussion for companies to align on the issues that can be discussed in advance, but leaving more detailed-oriented discussions and the exact numerical requirements definitions once RAN1/RAN2 design is fully in place. During the last RAN1 meeting, the following relevant agreements have been made.

RAN1 Agreement

To determine the binary sequences for LP-SS, for each M value, down-select the sequence length L from the corresponding candidate values:

- M=1, L= {4,6,8}

- M=2, L= {8,12,16,24}

- M=4, L= {16,24,32,56}

- FFS whether one or more than one L value (s) applied to each of M values M=1,2,4

- FFS the L values applied to other applicable SCS(s)

- FFS the L values if other M values in addition to M=1,2,4 are supported.

- FFS whether the time estimation averaged across multiple LP-SS occasions is applied

- Note 1: with sequence length L, it includes Manchester coding (if supported for LP-SS)

- Note 2: This doesn’t preclude any of three agreed sequence types

Above applies at least for both 15kHz and 30kHz

RAN1 Agreement

For LP-SS periodicity, support at least 320ms

- FFS: 80ms,160ms, 640ms,1280ms, 2560ms, 5120ms, 10240ms

Binary LP-SS sequences have not been chosen, and modulation and sequence length decisions are not yet firm. This means that several simulation scenarios should be simulated in advance, before selecting the right scenario. This does not mean that RAN4 cannot work independently from RAN1, but rather that RAN4 should find a systematic way to make progress and avoid excessive dependency on RAN1 progress.

Observation 1: Defining RRM requirements for LP-WUS/WUR are simulation-heavy and detailed alignment is needed both to make progress and for requirements to be defined, since LP-SS design is not yet finished.

However, during previous meetings, RAN4#112 and RAN4#112bis, several simulation parameters have been agreed among companies. More specifically, the broad strokes of the scenario considering a serving cell and an interfering cell (not known to the UE) have been agreed. These parameters, some agreed, some still pending, can be inspected in Tables 1, 2 and 3 below.

Table 1: General parameters

| Simulation parameters | Comments/values |
| --- | --- |
| Carrier frequency for Cell 1 and Cell 2 | Agreement: 2.6 GHz initially |
| Prior knowledge of Cell 1 / Cell 2 by the UE | Interfering cell (Cell 2) is not known to UE |
| DRX | No applicable for LP-WUR |
| BS transmit antennas for LP-SS blocks | 1 Tx |
| UE receive antennas | 1 Rx |
| Data and control channel subcarrier spacing | [Data, SSB and LP-SS have the same SCS] |
| Subcarrier spacing | 30KHz initially TBD for 15 KHz |
| Measurement period (in number of measurement samples) | [4, 5, other number could be studied upon a need] |
| LP-SS/SSB measurement interval | LP-SS: 320 ms SSB: 320 ms |
| LP-SS BW | 132 subcarriers for SCS=30kHz for LP-SS initially TBD for 15KHz SCS |
| SSS | 30KHz for SSS, TBD for 15KHz |
| Actual LP-SS transmissions | always transmitted |
| Guard band | 1 RB on each side of LP-SS/LP-WUS signal |

Table 2: Cell-specific parameters

| Parameter | Unit | Cell 1 | Cell 2 |
| --- | --- | --- | --- |
| RF Channel number | - | Channel 1 | Channel 1 |
| NR-PSS, NR-SSS (OFDM based LP-WUR) | - | To be indicated by companies | To be indicated by companies |
| LP-SS |  | OOK-1; or OOK-4 with M = [2,4] Note: M value [2 4] are up to company selection | when Cell 1 uses OOK-1; OOK-1 or NR signal is used for Cell 2 when Cell 1 uses OOK-4, OOK-4 with same M value as cell 1 or NR signal is used for Cell 2 |
| LP-SS pattern |  | [M sequence] [Golden sequence] [Computer search sequence] Note: Company can simulate one or all of them | [M sequence] [Golden sequence] [Computer search sequence] Note: Company can simulate one or all of them |
| PBCH and DMRS power offset with respect to NR-PSS, NR-SSS and LP-SS | dB | 0 | 0 |
| Data and control PSD relative to NR-PSS,NR-SSS and LP-SS | dB | 0 | 0 |
| RB Utilization | % | 100 | 100 |
| Data Modulation | - | QPSK | QPSK |
| Slot length | - | 14 symbols | 14 symbols |
| CP Length | - | Normal | Normal |
| Frequency offset relative to UE frequency reference | Hz | OFDM based receiver [5] ppm OOK based receiver [5 10] ppm | N/A |
| Timing error |  | Residual timing error + timing drift (frequency offset* 320ms (reference signal periodicity) Residual timing error: - T= 5us for OOK-1 - T= 2us for OOK-4 with M=2 - T= 1us for OOK-4 with M=4 |  |
| 1)Relative Delay of 1st Path (synchronous) | µs | 0 | CP/2 |
| 2) Relative Delay of 1st Path (asynchronous): Fixed delay | ms | 0 | 3 ms |
| SNR | dB | SNR setting for serving and interference cell are derived based on agreement of Issue 2-1-1-1 |  |
|  |  | When Ês/Iot = -3 dB - When SNR of cell 2 is 9 dB lower compared with cell 2; SNR = [-2.7] - When SNR of cell 2 is 6 dB lower compared with cell 2 SNR = [-2.4] | When Ês/Iot = -3 dB - When SNR of cell 2 is 9 dB lower compared with cell 2; SNR = [-11.7] - When SNR of cell 2 is 6 dB lower compared with cell 2; SNR = [-8.4] |
|  |  | When Ês/Iot = -0.5 dB Note: Determine the SNR based on based on agreement of Issue 2-1-1-1 | When Ês/Iot = -0.5 dB Note: Determine the SNR based on based on agreement of Issue 2-1-1-1 |
|  |  | When Ês/Iot = 2 dB Note: Determine the SNR based on based on agreement of Issue 2-1-1-1 | When Ês/Iot = 2 dB Note: Determine the SNR based on based on agreement of Issue 2-1-1-1 |
| Ês/Iot | dB | -3; -0.5dB; 2dB | N/A |
| Propagation conditions | - | FR1: AWGN TDL-C 300ns |  |
| UE speed |  | 3 km/h |  |

Table 3: UE-specific parameters

| [Receiver Filter] | [3th/5th Order Butterworth with 3.96MHz bandwidth] |
| --- | --- |
| [Receiver ADC bit width] | [4 or 8-bitADC] |
| [Receiver Sampling Rate for LP-SS only] | [3.84 or 7.68MHz] |

Observation 2: Good progress has been made in defining the serving cell / interfering cell scenario as a baseline for upcoming RRM requirements definition.

We believe that one aspect is key for continue making progress, which is to align on LP-SS simulation parameters (e.g. a hypothetical design) so results brought by companies can be compared against each other. This will be important in order to detect outliers, potential misunderstanding in simulation assumptions / signal processing, and understand whether differences between reported results are legitimate differences on LR / UE implementation, or they are something else.

Observation 3: Even though companies have started submitting LP-WUS/WUR simulation results, these results are not quite comparable since assumed LP-SS sequences, duration and modulation are vastly different.

Hence, we believe that if a systematic approach to align and compile simulation results can be implemented, this will bring great benefits to the discussion taking place, and the final requirement definition.

Proposal 1: RAN4 RRM to compile a simulation results summary spreadsheet to be updated during every RAN4 meeting based on the contributions of the interested companies.

Since there’s a good understanding of the scenarios that should be simulated, we believe that this can be quickly agreed online during the week of the RAN4#113 meeting.

Proposal 2: RAN4 RRM to agree on an initial summary of simulations spreadsheet format, including an interested company to take care of it, by the end of RAN4#113.

At the moment, tracking RSRP results can be enough to align understanding among companies and to review the results. Same framework could be later expanded to RSRQ if needed.

Proposal 3: RAN4 RRM to track RSRP measurement accuracy as a function of number of measurement instances and SNR/SINR values, AWGN and TDLC channels, and other agreed simulation parameters. Limit maximum number of measurement instances to [6 samples]. FFS about tracking RSRQ, others.

This way we can clearly align on reported accuracy and the delay requirement and its implications.

# Simulation Assumptions

As mentioned before during RAN4#112bis, in our view the simulations performed by RAN4 should be as close as possible with respect to simulation assumptions used in RAN1’s evaluation. In general, RAN4 could possibly modify and add necessary improvements tailored to RAN4 requirement design work. Our proposal goes in line to making sure that RAN4 assumptions for requirement design are substantially aligned with the performance expectations of RAN1 WG work and with the RAN4 RF agenda, which continues its parallel work.

Observation 4: For LP-WUR based RRM requirements design, RAN1 link level simulation assumptions and RAN4 RF session’s simulation assumptions should be continuously monitored as the baseline for RAN4 RRM simulation work.

During the past RAN1 meeting there was an update on which SNR values to be considered in their evaluations. See agreement below.

RAN1 Agreement

To determine the binary sequences for LP-SS, consider the following for performance comparison:

- Sync accuracy T’: the achieved sync accuracy T’ for a set of 4 sequences with the same number of occupied OFDM symbols (timing estimation error smaller than ≤T’ us for P=90 % of the time for SNR=-3dB, -6dB (low priority))

  - Assume the sliding window size is at least [-25us, 25us].

- RRM measurement accuracy X: the achieved measurement accuracy within range ± XdB for Q=90% measurements based on Y LP-SS samples within a period comparable to Z=the length of I-DRX cycle that is larger or equal to 1.28s

  - Assume X= 3.5dB shall be satisfied for LP-RSRP and LP-RSRQ, based on Y=1, 4 and Z=1.28s for comparison

- Cross correlation: maximum cross-cor value/ maximum auto-cor value

  - FFS how to use cross-correlation for comparison

- Note: refer to RAN4 RRM assumption for the inter-cell interference

Hence, RAN4 RRM assumptions should be updated accordingly.

Proposal 4: Include SINR = -6dB to the {-3dB; -0.5dB; 2dB} set for simulation assumptions. FFS whether -6dB is a low priority case.

In addition, in a RAN1 agreement shown in the previous section, we can see that M and L parameters for LP-SS have not been agreed. One option is that the number of occupied OFDM symbols for LP-SS scale with respect to M/L. Another option is that the number of occupied OFDM symbols remain the same, so L needs to be adjusted depending on M. During RAN1 discussion the following parameters have been narrowed down.

- M=1, L= {4,6,8}

- M=2, L= {8,12,16,24}

- M=4, L= {16,24,32,56}

Observation 5: It is still unclear the number of OFDM symbols that LP-SS may occupy, since it depends on modulation and M value, and the length L of the binary sequence.

Proposal 5: Define at least {M,L} pairs {2, 8} and {4, 16} for alignment purposes. FFS other pairs, upon need.

Proposal 6: From RAN1/RAN4 proposals, define at least 2 binary sequences, one for L=8 and one for L=16, to be agreed offline during the week of RAN4#113 preferably.

Another aspect that would be good to clarify has to do with the source of interference in the current requirements scenario. A first case, we are defining an OOK interferer colliding the LP-SS of the serving cells. In another scenario, we define an OFDM interferer. It is quite clear that the effect of each interference source has a different effect. See figure below.

Observation 6: Current simulation assumptions leave up to company report the binary sequence to be used for serving cell and the binary sequence use for interference cell.

By the current approach, we cannot really distinguish whether the reported accuracy is primarily based on the choice of sequences, or the assumed implementation.

Proposal 7: Define a specific binary sequence for serving cell and a specific binary sequence for the interference cell. Interfering binary sequence shouldn’t be known to the UE, but also should not be random.

Moving forward, we believe another topic needs to be clarified. For example, with one of the possible ways of estimating RSRP as ON-duration energy minus OFF-duration energy, no potential issue is seen since OFDM interference does not have a concrete pattern in time domain, and we can model it as random QPSK noise. However, in the case of a colliding OOK interferer, we can have the case in which the interference in the ON duration is different than the interference in the OFF duration. It is not clear how to address the implications for LR design, and what are the specific reception assumptions. If we clarify, this can help better understand what are the main implementation aspect that are common to all companies.

Proposal 8: RAN4 RRM to clarify simulation assumptions related to potentially colliding OOK into OOK interference. For OFDM interference, confirm the assumption of a random QPSK source.

Finally, another aspect that needs to be discussed is the use of OFDM overlaid sequences. From past RAN1 agreements, it has been decided that overlaid sequences will be transmitted.

RAN1 Agreement

Support overlaid OFDM sequence(s) for LP-SS:

- LP-SS reuses the overlaid OFDM sequence(s) specified for LP-WUS. The design on overlaid OFDM sequence(s) specified for LP-WUS doesn’t target for sync and RRM measurement performance based on overlaid OFDM sequence for LP-SS.

- Whether to transmit LP-SS by using a specified overlaid OFDM sequence is configurable.

  - Applicable at least for OOK-1 and FFS for OOK-4

- From RAN1 perspective, it is not intended to introduce new RAN4 requirements specific to overlaid sequences

RAN1 agreement

In case of overlaid OFDM sequence not carrying information:

- Option 1: Single overlaid sequence is on each OOK ‘ON’ symbol.

- Note 1: multiple overlaid OFDM sequences are specified.

- Note 2: gNB can configure different overlaid OFDM sequence(s) for different cells.

Agreement

For overlaid OFDM sequence at least for LP-WUS,

- For OOK-4 with M>1, support ZC sequence for overlaid sequence.

  - FFS details

The previous agreement says that LP-SS will reuse the overlaid OFDM sequence(s) specified for LP-WUS. In our interpretation, ZC sequences should be specified, but the details of how many and how one ZC sequence differ from each other is still FFS. For RAN4 purposes, it would be sufficient to consider 1) same ZC sequence/no ZC sequence for both cells, and 2) different ZCs for each cell.

Observation 7: Details of overlaid OFDM sequences in LP-SS have not been addressed yet in RRM simulation assumptions.

Proposal 9: For overlaid sequences, RAN4 to consider two scenario 1) no ZC sequence for both cells, and 2) different ZCs for each cell. FFS details.

In addition, another aspect that is not clearly defined yet is related to the residual timing offset. In our RAN1 contribution we have proposed the use of a preamble in LP-WUS to counteract the case where LP-SS may not be sufficient to reach sufficient time synchronization. Whether a preamble is necessary or not will finally depend on the time synchronization requirement. An OOK-based receiver is in general quite robust against frequency offset. However, the frequency offset affects the achievable time offset, which can have an impact. As previously agreed, we consider a model where the timing error is equal to Residual timing error + timing drift (frequency offset * 320ms (reference signal periodicity). Assuming no frequency synchronization is performed, and 10ppm is used for time tracking, the time drift for 320ms (which is assumed as the initial periodicity of LP-SS) would be too long. This means that other measures may need to be considered, and they would need to be stated explicitly.

Observation 8: Time synchronization may be challenging for OOK-4 receivers and other measures such as 1) using MR to assist LR in frequency synchronization, or 2) use of a preamble in LP-WUS to allow that LP-SS sync performance to be relaxed.

Observation 9: The use of a preamble or MR assistance may allow LP-SS being transmitted with periodicities longer than 320ms, e.g. 80ms,160ms, 640ms,1280ms, 2560ms, 5120ms, 10240ms that are still under RAN1 consideration.

Proposal 10: RAN4 RRM to consider LP-SS periodicities of 320ms and above and to discuss the implications on LP-SS performance the use of a preamble in LP-WUS.

# Simulation Results

In this section we provide an extract of our simulation results.

# Conclusion

In this contribution, we continue the discussion on the simulation assumption.

Observation 1: Defining RRM requirements for LP-WUS/WUR are simulation-heavy and detailed alignment is needed both to make progress and for requirements to be defined, since LP-SS design is not yet finished.

Observation 2: Good progress has been made in defining the serving cell / interfering cell scenario as a baseline for upcoming RRM requirements definition.

Observation 3: Even though companies have started submitting LP-WUS/WUR simulation results, these results are not quite comparable since assumed LP-SS sequences, duration and modulation are vastly different.

Proposal 1: RAN4 RRM to compile a simulation results summary spreadsheet to be updated during every RAN4 meeting based on the contributions of the interested companies.

Proposal 2: RAN4 RRM to agree on an initial summary of simulations spreadsheet format, including an interested company to take care of it, by the end of RAN4#113.

Proposal 3: RAN4 RRM to track RSRP measurement accuracy as a function of number of measurement instances and SNR/SINR values, AWGN and TDLC channels, and other agreed simulation parameters. Limit maximum number of measurement instances to [6 samples]. FFS about tracking RSRQ, others.

Observation 4: For LP-WUR based RRM requirements design, RAN1 link level simulation assumptions and RAN4 RF session’s simulation assumptions should be continuously monitored as the baseline for RAN4 RRM simulation work.

Proposal 4: Include SINR = -6dB to the {-3dB; -0.5dB; 2dB} set for simulation assumptions. FFS whether -6dB is a low priority case.

Observation 5: It is still unclear the number of OFDM symbols that LP-SS may occupy, since it depends on modulation and M value, and the length L of the binary sequence.

Proposal 5: Define at least {M,L} pairs {2, 8} and {4, 16} for alignment purposes. FFS other pairs, upon need.

Proposal 6: From RAN1/RAN4 proposals, define at least 2 binary sequences, one for L=8 and one for L=16, to be agreed offline during the week of RAN4#113 preferably.

Observation 6: Current simulation assumptions leave up to company report the binary sequence to be used for serving cell and the binary sequence use for interference cell.

Proposal 7: Define a specific binary sequence for serving cell and a specific binary sequence for the interference cell. Interfering binary sequence shouldn’t be known to the UE, but also should not be random.

Proposal 8: RAN4 RRM to clarify simulation assumptions related to potentially colliding OOK into OOK interference. For OFDM interference, confirm the assumption of a random QPSK source.

Observation 7: Details of overlaid OFDM sequences in LP-SS have not been addressed yet in RRM simulation assumptions.

Proposal 9: For overlaid sequences, RAN4 to consider two scenario 1) no ZC sequence for both cells, and 2) different ZCs for each cell. FFS details.

Observation 8: Time synchronization may be challenging for OOK-4 receivers and other measures such as 1) using MR to assist LR in frequency synchronization, or 2) use of a preamble in LP-WUS to allow that LP-SS sync performance to be relaxed.

Observation 9: The use of a preamble or MR assistance may allow LP-SS being transmitted with periodicities longer than 320ms, e.g. 80ms,160ms, 640ms,1280ms, 2560ms, 5120ms, 10240ms that are still under RAN1 consideration.

Proposal 10: RAN4 RRM to consider LP-SS periodicities of 320ms and above and to discuss the implications on LP-SS performance the use of a preamble in LP-WUS.

# References

[1] RP-241824, Revised WID: Low-power wake-up signal and receiver for NR (LP-WUS/WUR), vivo / NTT DOCOMO, RAN #105, Melbourne, Australia, September 9-12, 2024

[2] R4-2416861, WF for RRM core requirements for LP-WUS/WUR, Moderator (vivo), RAN4 #112bis

[3] R4-2415664, Topic summary for [112b][229] NR_LPWUS, vivo, RAN4 #112bis

[4] R4-2416883, Coffee break discussion minutes for [112b][223] NR_LPWUS, vivo, RAN4 #112