3GPP TSG-RAN4 Meeting #113	R4-2418598

# Orlando, United States, 18th Nov 2024 - 22nd Nov 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-4 | CR | 0669 | rev | - | Current version: | 16.18.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | (NR_newRAT-Perf) Editorial CR to 38.101-4 on PDCCH requirements to unify table numbering format |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Apple |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Perf |  |  |  |  |  | Date: |  |  | 2024-11-08 |
| Category: | D |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Tables containing PDCCH test cases do not follow the test numbering used for PDSCH and PBCH, where a prefix is added following the number of tables in the subclause. This is, for example, “Table 5.3.2.1.2-1, Test 1” vs “Table 5.3.2.1.2-1, Test 2-1” |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding prefix to PDCCH test case number in the tables where they are defined. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Format would remain inconsistent with the rest of the specification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.3.2.1.1, 5.3.2.1.2, 5.3.2.2.1, 5.3.2.2.2, 5.3.3.1.1, 5.3.3.1.2, 5.3.3.1.1, 5.3.3.1.2. |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.521-4 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

----------------------------------------------------- Beginning of Change ------------------------------------------------------------

5.3	PDCCH demodulation requirements

The receiver characteristics of the PDCCH are determined by the probability of miss-detection of the Downlink Scheduling Grant (Pm-dsg).

The parameters specified in Table 5.3-1 are valid for all PDCCH tests unless otherwise stated.

Table 5.3-1: Common test Parameters

| Parameter |  |  | Unit | Value |
| --- | --- | --- | --- | --- |
| Carrier configuration | Offset between Point A and the lowest usable subcarrier on this carrier (Note 1) |  |  | 0 |
| DL BWP configuration #1 | Cyclic prefix |  |  | Normal |
|  | RB offset |  | RBs | 0 |
| Common serving cell parameters | Physical Cell ID |  |  | 0 |
|  | SSB position in burst |  |  | First SSB in Slot #0 |
|  | SSB periodicity |  | ms | 20 |
| PDCCH configuration | Slots for PDCCH monitoring |  |  | Each slot |
|  | Number of PDCCH candidates |  |  | 1 |
|  | Frequency domain resource allocation for CORESET |  |  | Start from RB = 0 with contiguous RB allocation |
|  | TCI state |  |  | TCI state #1 |
| CSI-RS for tracking | First subcarrier index in the PRB used for CSI-RS (k0) |  |  | 0 |
|  | First OFDM symbol in the PRB used for CSI-RS (l0) |  |  | CSI-RS resource 1: 4 CSI-RS resource 2: 8 CSI-RS resource 3: 4 CSI-RS resource 4: 8 |
|  | Number of CSI-RS ports (X) |  |  | 1 |
|  | CDM Type |  |  | No CDM |
|  | Density (ρ) |  |  | 3 |
|  | CSI-RS periodicity |  | Slots | 15 kHz SCS: 20 30 kHz SCS: 40 |
|  | CSI-RS offset |  | Slots | 15 kHz SCS: 10 for CSI-RS resource 1 and 2 11 for CSI-RS resource 3 and 4 30 kHz SCS: 20 for CSI-RS resource 1 and 2 21 for CSI-RS resource 3 and 4 |
|  | Frequency Occupation |  |  | Start PRB 0 Number of PRB = ceil(BWP size /4)*4 |
|  | QCL info |  |  | TCI state #0 |
| TCI state #0 | Type 1 QCL information | SSB index |  | SSB #0 |
|  |  | QCL Type |  | Type C |
|  | Type 2 QCL information | SSB index |  | SSB #0 |
|  |  | QCL Type |  | Type D |
| TCI state #1 | Type 1 QCL information | CSI-RS resource |  | CSI-RS resource 1 from 'CSI-RS for tracking' configuration |
|  |  | QCL Type |  | Type A |
|  | Type 2 QCL information | CSI-RS resource |  | CSI-RS resource 1 from 'CSI-RS for tracking' configuration |
|  |  | QCL Type |  | Type D |
| PDCCH & PDCCH DMRS Precoding configuration |  |  |  | For number of TX = 1: No precoding; For number of TX > 1: Single Panel Type I, Randomized precoder selection for every REG bundle and updated per slot with equal probability of each applicable i1/i2 combination or codebook index, chosen from section 5.2.2.2.1 of TS 38.214 [12]. |
| Physical signals, channels mapping and precoding |  |  |  | As specified in Annex B.4.1 |
| Symbols for all unused REs |  |  |  | OP.1 FDD as defined in Annex A.5.1.1 OP.1 TDD as defined in Annex A.5.2.1 |
| The number of slots between PDSCH and corresponding HARQ-ACK information |  |  |  | 2 for FDD. For TDD, specific to each TDD UL-DL pattern and as defined in Annex A.1.2. |
| Note 1: Point A coincides with minimum guard band as specified in Table 5.3.3-1 from TS 38.101-1 [6] for tested channel bandwidth and subcarrier spacing. Note 2: The high layer parameter precoderGranularity equals to sameAsREG-bundle as defined in clause 7.4.1.3 of TS 38.211 [9]. |  |  |  |  |

5.3.1	1RX requirements

(Void)

5.3.2	2RX requirements

5.3.2.1	FDD

The parameters specified in Table 5.3.2.1-1 are valid for all FDD tests unless otherwise stated.

Table 5.3.2.1-1: Test Parameters

| Parameter | Unit | 1 Tx Antenna | 2 Tx Antenna |
| --- | --- | --- | --- |
| CCE to REG mapping type |  | nonInterleaved |  |
| REG bundle size |  | 6 |  |
| Shift index |  | 0 |  |

5.3.2.1.1	Minimum requirements with 1TX antenna

For the parameters specified in Table 5.3.2.1-1, the average probability of a missed downlink scheduling grant (Pm-dsg) shall be below the specified value in Table 5.3.2.1.1-1. The downlink physical setup is in accordance with Annex C.3.1.

Table 5.3.2.1.1-1: Minimum performance for PDCCH with 15 kHz SCS

| Test number | Bandwidth (MHz) | CORESET RB | CORESET duration | Aggregation level | Reference Channel | Propagation Condition | Antenna configuration and correlation Matrix | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | Pm-dsg (%) | SNR (dB) |
| 1-1 | 10 | 24 | 2 | 2 | R.PDCCH. 1-2.1 FDD | TDLA30-10 | 1x2 Low | 1 | 8.1 |
| 1-2 | 10 | 24 | 2 | 2 | R.PDCCH. 1-2.3 FDD | TDLC300-100 | 1x2 Low | 1 | 8.2 |
| 1-3 | 10 | 48 | 2 | 4 | R.PDCCH. 1-2.4 FDD | TDLA30-10 | 1x2 Low | 1 | 5.5 |
| 1-4 | 10 | 48 | 1 | 4 | R.PDCCH.1-1.1 FDD | TDLA30-10 | 1x2 Low | 1 | 4.4 |
| 1-5 | 10 | 48 | 2 | 16 | R.PDCCH. 1-2.6 FDD | TDLA30-10 | 1x2 Low | 1 | -2.1 |

5.3.2.1.2	Minimum requirements with 2TX antenna

For the parameters specified in Table 5.3.2.1-1, the average probability of a missed downlink scheduling grant (Pm-dsg) shall be below the specified value in Table 5.3.2.1.2-1. The downlink physical setup is in accordance with Annex C.3.1.

Table 5.3.2.1.2-1: Minimum performance for PDCCH with 15 kHz SCS

| Test number | Bandwidth (MHz) | CORESET RB | CORESET duration | Aggregation level | Reference Channel | Propagation Condition | Antenna configuration and correlation Matrix | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | Pm-dsg (%) | SNR (dB) |
| 2-1 | 10 | 24 | 2 | 4 | R.PDCCH. 1-2.2 FDD | TDLC300-100 | 2x2 Low | 1 | 2.0 |
| 2-2 | 10 | 48 | 2 | 8 | R.PDCCH. 1-2.5 FDD | TDLC300-100 | 2x2 Low | 1 | -1.3 |
| 2-3 | 10 | 48 | 1 | 8 | R.PDCCH.1-1.3 FDD | TDLA30-10 | 2x2 Low | 1 | -0.2 |

5.3.2.1.3	Minimum requirements for power saving

During the test the UE shall monitor the DCI format 2_6 PDCCH in DRX off state and decide whether to receive the following PDCCH in DRX on period.

The parameters specified in Table 5.3.2.1.3-1 are valid for FDD test unless otherwise stated.

Table 5.3.2.1.3-1: Test Parameters

| Parameter |  | Unit | 1 Tx Antenna |
| --- | --- | --- | --- |
| CCE to REG mapping type |  |  | nonInterleaved |
| REG bundle size |  |  | 6 |
| Shift Index |  |  | 0 |
| DRX cycle |  | ms | 10 |
| ps-WakeUp-r16 |  |  | absent |
| Wake-up indication bit in DCI format 2_6 |  |  | 1 |
| PDCCH DCI format 2_6 configuration | PS-offset |  |  |
|  | Number of PDCCH candidates |  | 1 |
|  | Frequency domain resource allocation for CORESET |  | Start from RB = 0 with contiguous RB allocation |
|  | TCI state |  | TCI state #1 |
| PDCCH configuration | Slots for PDCCH monitoring |  | Each slot during DRX-on period |
| Note: TminimumTimeGap is signaled as a part of drx-Adaptation-r16 UE capability. |  |  |  |

For the parameters specified in Table 5.3.2.1.3-1, the average probability of a missed downlink scheduling grant (Pm-dsg) observed on PDCCH during DRX on shall be below the specified value in Table 5.3.2.1.3-2. The downlink physical setup is in accordance with Annex C.3.1.

Table 5.3.2.1.3-2: Minimum performance for PDCCH with 15 kHz SCS

| Test number | Bandwidth (MHz) | CORESET RB | CORESET duration | Aggregation level | Reference Channel | Propagation Condition | Antenna configuration and correlation Matrix | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | Pm-dsg (%) | SNR (dB) |
| 3-1 | 10 | 48 | 2 | 4 | R.PDCCH. 1-2.4 FDD | TDLA30-10 | 1x2 Low | 1 | 5.5 |
|  |  |  | 2 | 8 | R.PDCCH. 1-2.7 FDD |  |  |  |  |

5.3.2.2	TDD

The parameters specified in Table 5.3.2.2-1 are valid for all TDD tests unless otherwise stated.

Table 5.3.2.2-1: Test Parameters

| Parameter | Unit | 1 Tx Antenna |  |  | 2 Tx Antenna |
| --- | --- | --- | --- | --- | --- |
| TDD UL-DL pattern |  | FR1.30-1 |  |  |  |
| CCE to REG mapping type |  | Test 3: non-interleaved Other tests: interleaved |  | interleaved |  |
| Interleaver size |  | 3 |  |  |  |
| REG bundle size |  | Test 3: 6 Other tests: 2 | 6 |  |  |
| Shift Index |  | 0 |  |  |  |

5.3.2.2.1	Minimum requirements with 1TX antenna

For the parameters specified in Table 5.3.2.2-1, the average probability of a missed downlink scheduling grant (Pm-dsg) shall be below the specified value in Table 5.3.2.2.1-1. The downlink physical setup is in accordance with Annex C.3.1.

Table 5.3.2.2.1-1: Minimum performance for PDCCH with 30 kHz SCS

| Test number | Bandwidth (MHz) | CORESET RB | CORESET duration | Aggregation level | Reference Channel | Propagation Condition | Antenna configuration and correlation Matrix | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | Pm-dsg (%) | SNR (dB) |
| 1-1 | 40 | 102 | 1 | 2 | R.PDCCH. 2-1.1 TDD | TDLA30-10 | 1x2 Low | 1 | 7.0 |
| 1-2 | 40 | 102 | 1 | 4 | R.PDCCH. 2-1.2 TDD | TDLC300- 100 | 1x2 Low | 1 | 3.0 |
| 1-3 | 40 | 48 | 2 | 16 | R.PDCCH. 2-2.1 TDD | TDLC300- 100 | 1x2 Low | 1 | -3.8 |

5.3.2.2.2	Minimum requirements with 2TX antenna

For the parameters specified in Table 5.3.2.2-1, the average probability of a missed downlink scheduling grant (Pm-dsg) shall be below the specified value in Table 5.3.2.2.2-1. The downlink physical setup is in accordance with Annex C.3.1.

Table 5.3.2.2.2-1: Minimum performance for PDCCH with 30 kHz SCS

| Test number | Bandwidth (MHz) | CORESET RB | CORESET duration | Aggregation level | Reference Channel | Propagation Condition | Antenna configuration and correlation Matrix | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | Pm-dsg (%) | SNR (dB) |
| 2-1 | 40 | 90 | 1 | 8 | R.PDCCH. 2-1.3 TDD | TDLC300-100 | 2x2 Low | 1 | -1.2 |

5.3.2.2.3	Minimum requirements for power saving

During the test the UE shall monitor the DCI format 2_6 PDCCH in DRX off state and decide whether to receive the following PDCCH in DRX on period.

The parameters specified in Table 5.3.2.2.3-1 are valid for all TDD tests for power saving unless otherwise stated.

Table 5.3.2.2.3-1: Test Parameters

| Parameter |  | Unit | 1 Tx Antenna |
| --- | --- | --- | --- |
| TDD UL-DL pattern |  |  | FR1.30-1 |
| CCE to REG mapping type |  |  | interleaved |
| Interleaver size |  |  | 3 |
| REG bundle size |  |  | 2 |
| Shift Index |  |  | 0 |
| DRX cycle |  | ms | 10 |
| ps-WakeUp-r16 |  |  | absent |
| Wake-up indication bit in DCI format 2_6 |  |  | 1 |
| PDCCH DCI format 2_6 configuration | PS-offset |  | (TminimumTimeGap+1)//0.125 |
|  | Number of PDCCH candidates |  | 1 |
|  | Frequency domain resource allocation for CORESET |  | Start from RB = 0 with contiguous RB allocation |
|  | TCI state |  | TCI state #1 |
| PDCCH configuration | Slots for PDCCH monitoring |  | Each slot during DRX-on period |
| Note: TminimumTimeGap is signaled as a part of drx-Adaptation-r16 UE capability.. |

For the parameters specified in Table 5.3.2.2.3-1, the average probability of a missed downlink scheduling grant (Pm-dsg) observed on PDCCH during DRX on shall be below the specified value in Table 5.3.2.2.3-2. The downlink physical setup is in accordance with Annex C.3.1.

Table 5.3.2.2.3-2: Minimum performance with 30 kHz SCS

| Test number | Bandwidth (MHz) | CORESET RB | CORESET duration | Aggregation level | Reference Channel | Propagation Condition | Antenna configuration and correlation Matrix | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | Pm-dsg (%) | SNR (dB) |
| 3-1 | 40 | 102 | 1 | 4 | R.PDCCH. 2-1.2 TDD | TDLC300- 100 | 1x2 Low | 1 | 3.0 |
|  |  |  |  | 8 | R.PDCCH. 2-1.4 TDD |  |  |  |  |

5.3.3	4RX requirements

5.3.3.1	FDD

The parameters specified in Table 5.3.3.1-1 are valid for all FDD tests unless otherwise stated.

Table 5.3.3.1-1: Test Parameters

| Parameter | Unit | 1 Tx Antenna | 2 Tx Antenna |
| --- | --- | --- | --- |
| CCE to REG mapping type |  | nonInterleaved |  |
| REG bundle size |  | 6 |  |
| Shift index |  | 0 |  |

5.3.3.1.1	Minimum requirements with 1TX antenna

For the parameters specified in Table 5.3.3.1-1, the average probability of a missed downlink scheduling grant (Pm-dsg) shall be below the specified value in Table 5.3.3.1.1-1. The downlink physical setup is in accordance with Annex C.3.1.

Table 5.3.3.1.1-1: Minimum performance for PDCCH with 15 kHz SCS

| Test number | Bandwidth (MHz) | CORESET RB | CORESET duration | Aggregation level | Reference Channel | Propagation Condition | Antenna configuration and correlation Matrix | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | Pm-dsg (%) | SNR (dB) |
| 1-1 | 10 | 24 | 2 | 2 | R.PDCCH. 1-2.1 FDD | TDLA30-10 | 1x4 Low | 1 | 2.2 |
| 1-2 | 10 | 24 | 2 | 2 | R.PDCCH. 1-2.3 FDD | TDLC300- 100 | 1x4 Low | 1 | 2.7 |
| 1-3 | 10 | 48 | 2 | 4 | R.PDCCH. 1-2.4 FDD | TDLA30-10 | 1x4 Low | 1 | 0.2 |
| 1-4 | 10 | 48 | 1 | 4 | R.PDCCH.1-1.1 FDD | TDLA30-10 | 1x4 Low | 1 | -0.4 |
| 1-5 | 10 | 48 | 2 | 16 | R.PDCCH. 1-2.6 FDD | TDLA30-10 | 1x4 Medium A | 1 | -3.2 |

5.3.3.1.2	Minimum requirements with 2TX antenna

For the parameters specified in Table 5.3.3.1-1, the average probability of a missed downlink scheduling grant (Pm-dsg) shall be below the specified value in Table 5.3.3.1.2-1. The downlink physical setup is in accordance with Annex C.3.1.

Table 5.3.3.1.2-1: Minimum performance for PDCCH with 15 kHz SCS

| Test number | Bandwidth (MHz) | CORESET RB | CORESET duration | Aggregation level | Reference Channel | Propagation Condition | Antenna configuration and correlation Matrix | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | Pm-dsg (%) | SNR (dB) |
| 2-1 | 10 | 24 | 2 | 4 | R.PDCCH. 1-2.2 FDD | TDLC300-100 | 2x4 Low | 1 | -1.9 |
| 2-2 | 10 | 48 | 2 | 8 | R.PDCCH. 1-2.5 FDD | TDLC300-100 | 2x4 Low | 1 | -4.5 |
| 2-3 | 10 | 48 | 1 | 4 | R.PDCCH.1-1.2 FDD | TDLA30-10 | 2x4 Low | 1 | -1.0 |

5.3.3.1.3	Minimum requirements for power saving

During the test the UE shall monitor the DCI format 2_6 PDCCH in DRX off state and decide whether to receive the following PDCCH in DRX on period.

The parameters specified in Table 5.3.3.1.3-1 are valid for FDD test unless otherwise stated.

Table 5.3.3.1.3-1: Test Parameters

| Parameter |  | Unit | 1 Tx Antenna |
| --- | --- | --- | --- |
| CCE to REG mapping type |  |  | nonInterleaved |
| REG bundle size |  |  | 6 |
| Shift Index |  |  | 0 |
| DRX cycle |  | ms | 10 |
| ps-WakeUp-r16 |  |  | absent |
| Wake-up indication bit in DCI format 2_6 |  |  | 1 |
| PDCCH DCI format 2_6 configuration | PS-offset |  |  |
|  | Number of PDCCH candidates |  | 1 |
|  | Frequency domain resource allocation for CORESET |  | Start from RB = 0 with contiguous RB allocation |
|  | TCI state |  | TCI state #1 |
| Slots for PDCCH monitoring |  |  | Each slot during DRX-on period |
| Note: TminimumTimeGap is signaled as a part of drx-Adaptation-r16 UE capability. |  |  |  |

For the parameters specified in Table 5.3.3.1.3-1, the average probability of a missed downlink scheduling grant (Pm-dsg) observed on PDCCH during DRX on shall be below the specified value in Table 5.3.3.1.3-2. The downlink physical setup is in accordance with Annex C.3.1.

Table 5.3.3.1.3-2: Minimum performance for PDCCH with 15 kHz SCS

| Test number | Bandwidth (MHz) | CORESET RB | CORESET duration | Aggregation level | Reference Channel | Propagation Condition | Antenna configuration and correlation Matrix | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | Pm-dsg (%) | SNR (dB) |
| 3-1 | 10 | 48 | 2 | 4 | R.PDCCH. 1-2.4 FDD | TDLA30-10 | 1x4 Low | 1 | 0.2 |
|  |  |  | 2 | 8 | R.PDCCH. 1-2.7 FDD |  |  |  |  |

5.3.3.2	TDD

The parameters specified in Table 5.3.3.2-1 are valid for all TDD tests unless otherwise stated.

Table 5.3.3.2-1: Common Test Parameters

| Parameter | Unit | 1 Tx Antenna |  | 2 Tx Antenna |
| --- | --- | --- | --- | --- |
| TDD UL-DL pattern |  | FR1.30-1 |  |  |
| CCE to REG mapping type |  | Test 3: Non-interleaved Other tests: interleaved | interleaved |  |
| Interleaver size |  | 3 |  |  |
| REG bundle size |  | Test 3: 6 Other tests: 2 | 6 |  |
| Shift Index |  | 0 |  |  |

5.3.3.2.1	Minimum requirements with 1TX antenna

For the parameters specified in Table 5.3.3.2-1, the average probability of a missed downlink scheduling grant (Pm-dsg) shall be below the specified value in Table 5.3.3.2.1-1. The downlink physical setup is in accordance with Annex C.3.1.

Table 5.3.3.2.1-1: Minimum performance for PDCCH with 30 kHz SCS

| Test number | Bandwidth (MHz) | CORESET RB | CORESET duration | Aggregation level | Reference Channel | Propagation Condition | Antenna configuration and correlation Matrix | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | Pm-dsg (%) | SNR (dB) |
| 1-1 | 40 | 102 | 1 | 2 | R.PDCCH. 2-1.1 TDD | TDLA30-10 | 1x4 Low | 1 | 2.1 |
| 1-2 | 40 | 102 | 1 | 4 | R.PDCCH. 2-1.2 TDD | TDLC300-100 | 1x4 Low | 1 | -0.9 |
| 1-3 | 40 | 48 | 2 | 16 | R.PDCCH. 2-2.1 TDD | TDLA30-10 | 1x4 Medium A | 1 | -3.6 |

5.3.3.2.2	Minimum requirements with 2TX antenna

For the parameters specified in Table 5.3.3.2-1, the average probability of a missed downlink scheduling grant (Pm-dsg) shall be below the specified value in Table 5.3.3.2.2-1. The downlink physical setup is in accordance with Annex C.3.1.

Table 5.3.3.2.2-1: Minimum performance for PDCCH with 30 kHz SCS

| Test number | Bandwidth (MHz) | CORESET RB | CORESET duration | Aggregation level | Reference Channel | Propagation Condition | Antenna configuration and correlation Matrix | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | Pm-dsg (%) | SNR (dB) |
| 2-1 | 40 | 90 | 1 | 8 | R.PDCCH. 2-1.3 TDD | TDLC300-100 | 2x4 Low | 1 | -4.3 |

5.3.3.2.3	Minimum requirements for power saving

During the test the UE shall monitor the DCI format 2_6 PDCCH in DRX off state and decide whether to receive the following PDCCH in DRX on period.

For the parameters specified in Table 5.3.3.2.3-1, the average probability of a missed downlink scheduling grant (Pm-dsg) observed on PDCCH during DRX on shall be below the specified value in Table 5.3.3.2.3-2. The downlink physical setup is in accordance with Annex C.3.1.

Table 5.3.3.2.3-1: Test Parameters

| Parameter |  | Unit | 1 Tx Antenna |
| --- | --- | --- | --- |
| TDD UL-DL pattern |  |  | FR1.30-1 |
| CCE to REG mapping type |  |  | interleaved |
| Interleaver size |  |  | 3 |
| REG bundle size |  |  | 2 |
| Shift Index |  |  | 0 |
| DRX cycle |  | ms | 10 |
| ps-WakeUp-r16 |  |  | absent |
| Wake-up indication bit in DCI format 2_6 |  |  | 1 |
| PDCCH DCI format 2_6 configuration | PS-offset |  | (TminimumTimeGap+1)//0.125 |
|  | Number of PDCCH candidates |  | 1 |
|  | Frequency domain resource allocation for CORESET |  | Start from RB = 0 with contiguous RB allocation |
|  | TCI state |  | TCI state #1 |
| Slots for PDCCH monitoring |  |  | Each slot during DRX-on period |
| Note: TminimumTimeGap is signaled as a part of drx-Adaptation-r16 UE capability. |  |  |  |

Table 5.3.3.2.3-2: Minimum performance with 30 kHz SCS

| Test number | Bandwidth (MHz) | CORESET RB | CORESET duration | Aggregation level | Reference Channel | Propagation Condition | Antenna configuration and correlation Matrix | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | Pm-dsg (%) | SNR (dB) |
| 3-1 | 40 | 102 | 1 | 4 | R.PDCCH. 2-1.2 TDD | TDLC300- 100 | 1x4 Low | 1 | -0.9 |
|  |  |  |  | 8 | R.PDCCH. 2-1.4 TDD |  |  |  |  |

------------------------------------------------------------- End of change ------------------------------------------------------------