# 3GPP TSG-RAN WG4 Meeting #113	R4-2418601

# Orlando, US, 18th – 22nd November, 2024

Agenda item:	7.20.1

Source: 		Qualcomm Incorporated

Title: 		Views on general aspects for SBFD

Document for:		Approval

# Introduction

In RAN#102, the work item on evolution of NR duplex operation was approved [1]. From RAN4 RF perspective, the objective of the work item is as follows:

The objectives are as follows: - Specify BS RF requirements for SBFD operation at gNB [RAN4]

In RAN4#112bis, a WF was agreed in [2] entailing several aspects regarding SBFD general system design and deployment. In this contribution we share our views on general aspects for Rel-19 SBFD WI within RAN4.

# General aspects

## DL/UL subband configurations support for SBFD

In [2], the following was agreed regarding how to handle guard band and subband configurations in the core specification

- RAN4 to further study Option 2b by considering the below additional restrictions such as: - For DUD case, the UL subband should be placed in the middle of channel bandwidth - FFS the number of DL subband sizes to be declared and tested - Alt-1: For FR1 WA BS, one DL subband size is declared to support SBFD and tested, operation with other DL subband sizes than the tested one is not allowed. - FFS the necessity of this restriction is applicable to other FR1 BS classes and FR2 BS. - Alt-2: For FR1 WA BS, the DL subband sizes declared to support SBFD shall be tested - FFS the necessity of this restriction is applicable to other FR1 BS classes and FR2 BS. - Restriction on UL subband size based on the specific channel bandwidth - FFS to specify a range of ratio for UL subband size over channel bandwidth. - FFS the restriction is different for DU and DUD - FFS the restriction is applied for FR2 case.

Regarding the definition of the UL/DL subband configurations for RF requirements, it is viable to consider a configuration-agnostic approach, such as the RF requirements are defined independent of the subband configuration. The supported configuration will impact the conformance work, which is one of the main driving factors of the above discussion. On one extreme, leaving the DL subband, UL subband, and guardband fully flexible might impose additional testing challenges, as conveyed by some companies within RAN4. On the other hand, specifying only one configuration will limit the feature deployment as operators will be restricted to deploy such a configuration.  Regarding the two alternatives above on top of the agreed option 2b from RAN4#112, it is important to ensure within RAN4 that the above discussion should be restricted to WA FR-1 SBFD-capable gNB. For other base station classes and FR2-1 gNB, from such a restriction might not be needed, since there might be more flexibility in term of switching between different SBFD configurations as different (and more flexible) interference cancellation techniques are feasible, in contrast, to less flexible cancellation for WA BS.

Proposal 1: To limit the discussion of DL/UL subband configurations support for SBFD to FR1 WA BS at this stage, as other BS classes and FR2-1 WA BS have more flexibility in terms of interference cancellation.

Moving to the alternative one vs alternative two, our initial preference was alternative two as it is more aligned with how a SBFD capable BS is expected to be tested and deployed. Restricting the SBFD configuration only to one single configuration imposes unneeded restriction on the feature adoption without clear motivation why is that needed. In order to progress the discussion within RAN4, it is a viable to consider a fixed number of configurations to be supported by FR1 WA BS. It was discussed last meeting that this number can be two. Accordingly, RAN4 can further discuss what would be a reasonable number of configurations supported by the FR1 WA BS.

Proposal 2: RAN4 to consider fixed number of downlink subband configuration supported for FR1 WA BS. FFS what exact number to be agreed.

## BS RF Specification structure for SBFD requirements

In [2], the following was agreed regarding how to include SBFD requirements in the core specifications

- FFS options of how to introduce BS RF new requirements for SBFD-capable BS: - Option-2: Creating new sub-clauses in TS 38.104 for RF requirements that applies for SBFD - Option-3: A new specification for SBFD BS RF requirements

In our view, creating new sub-clauses in TS 38.104 for RF requirements that applies for SBFD is the viable option to consider. It is unclear and unnecessary for us that a new specification for SBFD is needed. An SBFD capable BS is basically a legacy BS with additional capability and design differences (e.g., separate panels for Tx and Rx). Additionally, a SBFD capable can operate simply as a legacy TDD if configured. In this regard, to avoid unnecessary work in RAN4, our preference is to keep the Rel-19 normative work captured in the legacy BS specs (TS 38.104, 38.141-1/-2), preferably via additional sub-clauses.

Proposal 3: RAN4 to include Rel-19 SBFD BS requirements in the legacy BS specs, preferably via additional sub-clauses.

# Conclusion

In this contribution, we provided our views on general aspects for SBFD operation with the following observations and proposals:

Proposal 1: To limit the discussion of DL/UL subband configurations support for SBFD to FR1 WA BS at this stage, as other BS classes and FR2-1 WA BS have more flexibility in terms of interference cancellation.

Proposal 2: RAN4 to consider fixed number of downlink subband configuration supported for FR1 WA BS. FFS what exact number to be agreed.

Proposal 3: RAN4 to include Rel-19 SBFD BS requirements in the legacy BS specs, preferably via additional sub-clauses.

# References

- RP-240789 “Revised WID: Evolution of NR duplex operation: Sub-band full duplex (SBFD)”, Huawei, HiSilicon, 3GPP TSG RAN#103.

- R4-2416584, “Way Forward for [112bis][306] NR_duplex_evo_General”, RAN4#112bis, Samsung.