# 3GPP TSG-RAN WG4 Meeting #113	R4-2418602

Orlando, US, 18th – 22nd November, 2024

Agenda item:	7.20.2.1

Source: 	Qualcomm Incorporated

Title: 	Views on new requirements for SBFD

Document for:	Approval

# Introduction

In RAN#102, the work item on evolution of NR duplex operation was approved [1]. From RAN4 RF perspective, the objective of the work item is as follows:

The objectives are as follows: - Specify BS RF requirements for SBFD operation at gNB [RAN4]

In RAN4#112bis, a WF was agreed in [2]. In this contribution we share our views on existing BS RF requirements for Rel-19 SBFD.

# New BS RF requirements for SBFD operation

In [2], the following was agreed regarding the potentially new in-channel adjacent subband leakage ratio, in-channel adjacent subband selectivity, and in-channel adjacent subband blocking new requirements:

- FFS the necessity of in-channel adjacent subband leakage ratio requirements - Option 1: No necessity to introduce new requirement for in-channel adjacent subband leakage ratio. - Option 2: New requirement for in-channel adjacent subband leakage ratio should be defined. - Define a requirement on TX sub-band ACLR similar to the ACLR requirement and use existing ACLR requirement as baseline. - FFS the necessity of in-channel adjacent subband selectivity requirements - Option 1: No necessity to introduce new requirement for in-channel adjacent subband selectivity. - Option 2: New requirement for in-channel adjacent subband selectivity should be defined. - Define a requirement on RX sub-band ACS similar to the ACS requirement and use existing ACS requirement as baseline. - FFS the necessity of in-channel adjacent subband blocking requirements - Option 1: No necessity to introduce new requirement for in-channel adjacent subband blocking. - Option 2: New requirement for in-channel adjacent subband blocking should be defined.

All the above-mentioned new requirements are related to the in-band operation, i.e., intra-operator, and are different than existing ACLR/ ACS/ blocking requirements, which are aimed to ensure feasible deployment of inter-operator adjacent operation. Historically, RAN4 did not specify any in-band emissions for legacy TDD BS, since it is implementation dependent and can be addressed via proprietary solutions.

Observation 1: RAN4 has not specified before in-band emission requirements for legacy TDD BSs as it is implementation dependent and different solutions might vary between vendors.

Additionally, during the study item phase, it was for further study whether new requirements will be needed or not. The dependency on the self-interference capability yields the definition of a new inter-subband requirement difficult to agree between different companies. As observed in Section 9 in TR 38.853, different implementations can be assumed between vendors, leading to different self-interference, co-sector interference, and inter-site interference mitigation capabilities. Moreover, employed technologies and mitigation techniques in the SBFD-capable gNB is quite difficult within RAN4, and thus, room for implementation differences should be taken into account when defining the new inter-subband leakage, selectivity, and blocking requirements. As a result, it might be difficult to propose technically meaningful requirements taken into consideration the above aspects.

Observation 2: The in-channel subband sub-band Tx leakage, selectivity, and blocking depend on the self-interference, inter-site and inter-sector interference considerations, thus, they are implementation dependent.

Taking into consideration also the techniques introduced by other WGs to address inter-gNB and inter-UE CLI, the definition of new RAN4 requirements should take into consideration such techniques. As a result, we prefer that RAN4 would adopt agree that no new requirements for in-channel leakage, selectivity, and blocking are defined for Rel-19 SBFD-capable gNBs.

Proposal 1: RAN4 to agree that no new requirements for in-channel subband leakage, selectivity, and blocking are defined for Rel-19 SBFD-capable gNBs.

# Conclusion

In this contribution, we provided our views on BS RF requirements for SBFD with the following observations and proposals:

Observation 1: RAN4 has not specified before in-band emission requirements for legacy TDD BSs as it is implementation dependent and different solutions might vary between vendors.

Observation 2: The in-channel subband sub-band Tx leakage, selectivity, and blocking depend on the self-interference, inter-site and inter-sector interference considerations, thus, they are implementation dependent.

Proposal 1: RAN4 to agree that no new requirements for in-channel subband leakage, selectivity, and blocking are defined for Rel-19 SBFD-capable gNBs.

# References

- RP-240789 “Revised WID: Evolution of NR duplex operation: Sub-band full duplex (SBFD)”, Huawei, HiSilicon, 3GPP TSG RAN#103.

- R4-2416584, “Way Forward for [112bis][306] NR_duplex_evo_General”, RAN4#112bis, Samsung.