3GPP TSG-RAN4 Meeting #113	R4-2418606

# Orlando, US, 18th – 22nd November, 2024

Title:	Views on BS demodulation requirements for MMSE-IRC

Source:	Qualcomm Incorporated

Agenda item:	7.16.3

Document for:	Approval

# 1.	Introduction

During RAN#103 plenary a new study item on BS advanced receiver performance was agreed in [1]. One of the objectives of the WI is as follows:

- Define FR1 PUSCH performance requirements with inter-cell interference in homogeneous and heterogeneous deployment based on the following assumptions: - Receiver type: MMSE-IRC receiver - Scenarios: - FDD synchronous deployments - TDD synchronous deployments with aligned UL:DL configurations among cells - Slot-based transmission and aligned SCS among cells - Reuse LTE based interference profile in TR 36.884 as a starting point. Other interference profiles are not precluded.

In [2], a WF was agreed where several topics were discussed and agreed regarding general aspects and interference profiling that would be considered during the normative phase. In this contribution, we provide our views on the different assumptions and potential deltas that RAN4 is expected to consider in Release 19 compared to the work that was carried out in LTE for BS with advanced receiver.

# 2 	Discussion

## 2.1	Interference Profiles

Regarding the interference profiles, it was agreed regarding the test scenarios in [2] the following

- On how to account for HPUE in existing LTE profiles: - Reuse the LTE interference power level as a start point, and check performance gain of MMSE-IRC over baseline MMSE. - It is not precluded for companies to bring system level simulation on other profiles. - Proposals on how to account for HPUE in existing LTE profiles. - Proposal 1: Perform new system-level simulations with the inclusion of HPUEs - Proposal 2: Consider artificially increasing DIP/INR with additional 3 or 7 dB

To account for HPUEs, RAN4 could consider artificial values for DIP/INR, where different interference levels could be simulated to ensure the impact of HPUEs is accounted for in the link level studies. However, increasing DIP/INR by 3 or 7 dB as captured in proposal 2 is not fully correct, because this assumes that any increase in the UE Tx will result in 1:1 increase in the interference, which is not the case, as the signal power also increases. As a result, it is proposed to consider DIP values from [3] at this stage and further investigate how to artificially adapt DIP/ INR values to account for the impact of HPUEs, if applicable.

Proposal 1: RAN4 not to artificially increase DIP/INR with additional 3 or 7 dB, but to consider legacy DIP values from TR 36.884, and further investigate how to artificially adapt DIP/ INR values to account for the impact of HPUEs, if applicable.

## 2.2	Test parameters and simulation assumptions

Propagation condition

In [2], the following options were captured

- Candidate options: - For serving cell: - Option 1: TDLA30-10 and TDLC300-100 for HetNet and HomNet respectively - Option 2: TDLA30-10 and TDLB100-100 for HetNet and HomNet respectively - Option 3: Cover TDLA30-10, TDLB100-400, TDLC300-100 - Option 4: Use TDLB100-400 - For interference cell: - Option 1: Introduce new propagation channel for testing with desired delay spread 1000ns for the interference PUSCH, i.e., TDLC1000 - Option 2: Same propagation condition for serving and interference PUSCH - Option 3: Use TDLC300-100

LTE study [3] considered ETU channel with high delay spread for the interferers and EPA5 and EVA70 were chosen for the serving channel while, ETU5 and ETU70 were considered for the interfering channel. Since different channel model for serving and interfering UEs was assumed and based on initial discussions within RAN4 in [2], we consider the same assumption in this work item (i.e., different channel model for serving and interfering UEs). Additionally, in [3], different channel models for homogenous and heterogeneous deployments were assumed, such that low Doppler models were adopted for heterogenous (EPA5 and ETU5) and high Doppler models for homogenous (EVA70 and ETU70) due to anticipated different cell ranges and typical UE speed.

Observation 1: In LTE work, different channel model was assumed for serving and interfering links as well as between homogenous and heterogeneous deployments.

For Rel-19, it is expected that RAN4 would choose an appropriate propagation conditions that is realistic compared to the envisioned use cases and scenarios. Accordingly, it is proposed to select among legacy TDLA30, TDLB100, and TDLC300 conditions. Specifically, we propose to adopt TDLA30-10 (TDLB100-100) for the serving PUSCH for HetNet (HomNet) while for the interference PUSCH, use TDLC300-100.

Proposal 2: RAN4 to adopt TDLA30-10 (TDLB100-100) for the serving PUSCH for HetNet (HomNet) while for the interference PUSCH, use TDLC300-100.

MCS

In [2], the following options were captured

- Candidate options: - Option 1: Cover QPSK/16QAM/64QAM - Option 1A: Use MCS 5(QPSK, R=379/1024), MCS 13(16QAM, R=490/1024) and MCS 24(64QAM, R=772/1024) - Option 1B: MCS index 2, 12, 20 in MCS table 1 - Option 1C: Cover MCS 2, 16, 20 - Option 2: Choose only one MCS configuration - Option 3: Consider MCS 5(QPSK, R=379/1024) and MCS 13(16QAM, R=490/1024) and FFS 64QAM

To ensure enough test coverage of different conditions, it is a viable option to consider one MCS per modulation level, while taking into considerations QPSK, 16QAM and 64QAM. Regarding UL 256QAM, it is anticipated that with such high modulation order, the UE will experience favourable propagation conditions, and thus, intercell interference could be marginal. The specific MCS could start from typical values used in normal PUSCH demodulation requirements, e.g., MCS index 2, 16, 20 in MCS table 1 for QPSK, 16QAM and 64QAM separately.

Proposal 3 : RAN4 to consider QPSK, 16QAM and 64QAM addressed via MCS index 2, 16, and 20.

DMRS configuration

DMRS configuration - Candidate options on DMRS type: - Option 1: Cover DMRS configuration type 1 and type 2 - Option 2: DMRS configuration type 1

In order not to preclude any configuration at this early stage of the normative work, we propose to adopt option 1, where both type 1 and type 2 DMRS configuration are supported.

Proposal 4: RAN4 to cover DMRS configuration type 1 and type 2.

# Conclusion

In this contribution we provide our views on the impact of multi-user spatial beamforming techniques on the system level simulation. Our observations and proposals can be summarized for that frequency range as follows:

Proposal 1: RAN4 not to artificially increase DIP/INR with additional 3 or 7 dB, but to consider legacy DIP values from TR 36.884, and further investigate how to artificially adapt DIP/ INR values to account for the impact of HPUEs, if applicable.

Observation 1: In LTE work, different channel model was assumed for serving and interfering links as well as between homogenous and heterogeneous deployments.

Proposal 2: RAN4 to adopt TDLA30-10 (TDLB100-100) for the serving PUSCH for HetNet (HomNet) while for the interference PUSCH, use TDLC300-100.

Proposal 3 : RAN4 to consider QPSK, 16QAM and 64QAM addressed via MCS index 2, 16, and 20.

Proposal 4: RAN4 to cover DMRS configuration type 1 and type 2.

# Reference

[1]		RP-240831, “New WID: WI resulting from discussion on Demodulation performance requirements enhancements”, Vivo, RAN#103, Maastricht, Netherlands.

[2] 	R4-2416548, “Way Forward for [112bis][319] NR_demod_Ph5_Part1_General_BS”, RAN4#112bis,  China Telecom.

[3]	TR 36.884, “Performance requirements of MMSE-IRC receiver for LTE BS” 3GPP V13.1.0