3GPP TSG-RAN4 Meeting #113	R4-2418610

Orlando, US, 18th – 22nd November, 2024

Title:	Views on adjacent channel co-existence for 14800 - 15350 MHz

Source:	Qualcomm Incorporated

Agenda item:	7.2.4.1

Document for:	Approval

# 1.	Introduction

During RAN#103 plenary a new study item on IMT parameters for 4400 to 4800 MHz, 7125 to 8400 MHz and 14800 to 15350 MHz was approved [1]. In RAN4#112, a WF was agreed in [2] and [3] where several agreements were captured for the 14800 – 15350 MHz frequency range. In this contribution, we provide our preliminary results and views on the different system aspects, RF parameters for BS and UE items for the 14800 – 15350  MHz frequency range.

# 2 	Discussion

## 2.1	System parameters

## 2.1.1	Channel bandwidth

In general, wider channel bandwidths should be enabled with increasing frequency following same approach as FR1 (3 to 100 MHz) to FR2 (50 to 400 MHz). This comes with increasing SCS and increasing RB width and availability of the spectrum. Channel bandwidth of 100 MHz has been considered as a representative channel bandwidth in TR 38.921 for 6GHz and 10GHz. Accordingly, it is proposed that higher channel bandwidth (e.g., 200 MHz) should be considered as a baseline for this frequency range.

Proposal 1: RAN4 to consider 200 MHz as a baseline in the adjacent channel coexistence framework.

## 2.5 Adjacent channel coexistence results

In this contribution we present our simulation results for Urban macro scenario for both downlink and uplink transmissions based on the set of parameters and models given in [4]. We follow the RAN4 simulation methodology, where the RF parameters (i.e., ACLR/ACS) are determined based on the degradation cause by adjacent channel interference (ACI).

Figure 1 SINR statistics for downlink and uplink (ACIR = 15 dB)

The SINR CDF for the uncoordinated case for both downlink and uplink scenarios is given in Figure 1. Based on the observed performance, we can conclude the following:

- BS antenna parameters agreed in [3] (i.e., 3072 total dual polarized elements) provide sufficient coverage for both DL and UL scenarios.

- Channel bandwidth of 200MHz assumption does not provide observed coverage issues.

Observation 1: No coverage issues (DL and UL) are observed with the agreed BS AAS configuration (3072 antenna elements, 16*24 URA with 4 elements per subarray) and 200 MHz channel bandwidth.

Proposal 2: RAN4 to consider 3072 as total number of antenna elements and 200 MHz channel bandwidth since no coverage issues were observed.

In terms of the impact of the adjacent channel interference, Figure 2 presents the throughput loss% as a function of the ACIR for both downlink and uplink in an UMa scenario, while considering uncoordinated deployment between the victim and aggressor deployments. It can be observed that in order to meet the 5% throughput loss degradation, ACIR values of 22 dB and 13 dB for downlink and uplink, respectively.

Observation 2: To meet the throughout threshold target, ACIR values of 22 dB and 13 dB were observed for downlink and uplink, respectively, based on the latest agreed parameters.

Figure 2  Throughput loss for downlink and uplink as function of ACIR.

In comparison to FR2-1, Table below summarizes the different ACLR for the different frequency ranges based on RAN4 RF requirements. It can be observed that the 15GHz ACIR values are comparable or even lower compared to FR2-1 frequency range. This is a byproduct of the enhanced performance resulting from large antenna arrays deployed at the BS, which leads to lower adjacent channel interference. In comparison to Rel-17 work done on 10GHz, number of antennas at the BS side was assumed to be 16x8, whereas in [2], 3k elements were considered, resulting in around 9 dBi difference in the directivity.

Table 1 ACIR values for different frequency ranges

| Range | Direction | ACLR [dB] | ACS [dB] | ACIR [dB] |
| --- | --- | --- | --- | --- |
| FR1 | UL | UE ACLR: 30 | BS ACS: 46 | ~29 |
|  | DL | BS ACLR: 45 | UE ACS: 33 | ~33 |
| FR2 | UL | UE ACLR: 17 | BS ACS: 23.5 | ~16 |
|  | DL | BS ACLR: 28 | UE ACS: 23 | ~22 |
| 15GHz | UL | UE ACLR:24 | BS ACS:TBD | ~13 |
|  | DL | BS ACLR:27 | UE ACS:24 | ~22 |

Observation 3: Large antenna arrays at the BS enables feasible adjacent channel coexistence via directing the wanted signal to the target destination and reducing the adjacent channel interference.

In order to derive the ACLR/ACS parameters for the BS and UE, the numbers from the adjacent channel coexistence should be considered with care. As an example, focusing on the provided ACIR performance for the uplink scenario given in Figure 2, the cell edge curve results in around uplink ACIR = 13 dB, which represents the combined effects of the UE ACLR and the gNB ACS. Additionally, UEs in the 14800 – 15350 MHz frequency range will anyway need to comply with the regulatory occupied bandwidth requirement, that specifies the maximum bandwidth within which emissions do not exceed a certain percentage of the total emissions. For NR, it is equal to the channel bandwidth with a maximum of 1% of emissions allowed outside of it (0.5% on each side), which result in around 24 dB UE ACLR requirement.

Observation 4: Due to the regulatory occupied bandwidth requirement, required UE ACLR is technically equal to 24 dB.

Given the above, it is preferable to derive the ACLR/ACS parameters based on a combination of results from the adjacent channel coexistence and existing other RF parameters (e.g., occupied BW) as the adjacent channel coexistence results might be too optimistic. Thus, it is a viable proposal to consider BS (UE) ACLR as 27 dB (24 dB), while the BS UE ACS as a 24 dB.

Proposal 4: RAN4 to consider BS (UE) ACLR as 27 dB (24 dB), while UE ACS as 24 dB.

Finally, it should be clarified that the above numbers and the outcome of the adjacent channel coexistence are based on the underlying assumptions. During the normative phase of defining the requirements later in RAN4, different assumptions can be agreed and thus, the above ACIR numbers can be revisited.

Observation 5: During the normative work, different assumptions and accompanying ACIR values can be discussed within RAN4.

# Conclusion

In this paper we have shared our views on the RF parameters for 14800 - 15350 MHz frequency range. Our observations and proposals can be summarized for that frequency range as follows:

Proposal 1: RAN4 to consider 200 MHz as a baseline in the adjacent channel coexistence framework.

Observation 1: No coverage issues (DL and UL) are observed with the agreed BS AAS configuration (3072 antenna elements, 16*24 URA with 4 elements per subarray) and 200 MHz channel bandwidth.

Proposal 2: RAN4 to consider 3072 as total number of antenna elements and 200 MHz channel bandwidth since no coverage issues were observed.

Observation 2: To meet the throughout threshold target, ACIR values of 22 dB and 13 dB were observed for downlink and uplink, respectively, based on the latest agreed parameters.

Observation 3: Large antenna arrays at the BS enables feasible adjacent channel coexistence via directing the wanted signal to the target destination and reducing the adjacent channel interference.

Observation 4: Due to the regulatory occupied bandwidth requirement, required UE ACLR is technically equal to 24 dBc.

Proposal 4: RAN4 to consider BS (UE) ACLR as 27 dB (24 dB), while UE ACS as 24 dB.

Observation 5: During the normative work, different assumptions and accompanying ACIR values can be discussed within RAN4.

# Reference

[1]	RP-240787, “New SI proposal: Study on IMT parameters for 4400 to 4800 MHz, 7125 to 8400 MHz and 14800 to 15350 MHz”, Ericsson, RAN #103, Maastricht, Netherlands.

[2] R4-2417107, “WF on 15GHz BS parameters”, ZTE, RAN4#112bis, Hefei, China.

[3] R4-2417108, “WF on 15GHz UE parameters”, Qualcomm, RAN4#112bis, Hefei, China.

[4] R4-2417105, “TP to TR 38.922: Revisions of system level simulation assumptions for study on IMT parameters for 14800 to 15350 MHz frequency range”, Nokia, RAN4#112bis, Hefei, China.