3GPP TSG-RAN WG4 Meeting#113		R4-2418615

Orlando, United States, Nov 18th -22nd, 2024

Agenda item:	7.6.2

Source: 	Spreadtrum Communications

Title: 	Discussion on methods for reducing the number of UE RX chains

Document for:	Discussion

Introduction

In RAN4#112bis meeting, a WF [1] was approved. In this contribution, we continue to provide some initial views on methods for reducing the number of UE RX chains.

#### ssue 3-1: Discussion on assumptions on test configurations for evaluation Agreement: (a) RAN4 agree on the general assumptions that are same for legacy NR RF performance evaluation: o UE self-band uplink Tx re-use same assumptions for NR PC3 PA i.e., ACLR=30dB, with MPR=1dB, Full RB allocation. Tx LO leakage and image rejection ratio are 28dB o Reuse the Rx chain image rejection, which is 25dB, as the starting point. Agreement: - The configuration defined in Table 7.3A.2.2-1 and Table 7.3A.2.2-2 of TS 38.101-1 could be re-used for performance evaluation, for ΔRIBNC, REFSENS, both out-of-gap and in-gap ACS, IBB and NBB requirements for selected example combos Agreement: - Priority to one uplink carrier Agreement: - Evaluation may consider impairment factors such as ACLR, phase noise or IMDx of aggressor ability, duplexer isolation…etc - The example bands should be in the scope of existing intra-band non-contiguous CA operating bands - No relaxation on any Tx RF requirements should be assumed in this study item - Both PC3 and PC2 should be considered - Prioritize PC3 - The minimum Wgap for ACS, narrow band blocking and in-band blocking verification should meet: - Wgap ≥ 2∙¦FInterferer (offset),j¦ – BWChannel(j) - It is the condition to apply the requirements - Both the symmetric CC scenario and the asymmetric CC scenario should be considered in the FC SI, and companies are encouraged, but not limited, to study the CC scenarios shared by the operators - [Adjusted requirements for fragmented carriers, if any, shall be separated from the existing requirements] - RAN4 shall not re-discuss the power imbalance assumption of the wanted carriers further in relation to the SI on fragmented carriers #### Issue 3-2: Example band consideration for evaluation Agreement: - RAN4 doesn’t consider 3MHz channel bandwidth for fragmented carriers - The example band(s) should already have NC DLCA specified - Prioritize the frequency bands of ≤2700MHz - Remove n39 and include n26 in the example bands: Consider n2/n25, n3, n7, n66, n41, n39, n26 as the example band - Ad hoc meeting agreements #### Merge issue 2-0/2-1 discussion: Agreement: The terminology of “One Rx RF Chain” is from single antenna to the end of ADC (before digital BB filter) with a single down-converter - To reduce the number of Rx RF chains, a fully shared RF chain (one Rx RF chain) is the reference architecture for fragmented carriers - A partially shared RF chain architecture is the reference architecture for existing DL NCCA requirements (refer to TR 36.823) #### Issue 2-2: Study on power spectral density difference between carriers of co-located adjacent channel operators Agreement: To evaluate acceptable [PSD] difference from UE side between wanted signal and un-wanted signal based on the deployment where BSs from different operators are collocated but their radio units (RUs) are not shared. - The [PSD] difference means the difference between unwanted signal and the wanted carrier with the lower PSD

Discussion

UE reference architecture

In the last meeting, fully shared architecture from single antenna to ADC was agreed to be the reference architecture for fragmented carriers. However, there is no conclusion about shared digital BB or separate digital BB. In our understanding, shared digital BB includes that shared digital filter and a single FFT. If shared digital BB filter is to be considered, out of the boundary of two carriers rejection can’t be achieved. In our understanding, shared digital BB filter needs to be careful consideration. Besides, a single FFT is able to process when two CCs are aligned with SCS. However, a single FFT can’t process OFDM data when two CCs with different SCSs are used. A single FFT could be used just when SCS are aligned. Additionally, two non-contiguous carriers to be received by a single digital BB together, the cost and complexity of digital BB with the Gap bandwidth between two non-contiguous carriers increase compared to separate digital BB. On the whole, shared digital BB or separate digital BB is up to implementation. We can take separate digital BB to be priority.

Proposal 1: Shared digital BB or separate digital BB up to implementation. Separate digital BB are priority to be considered.

Study on power spectral density difference between carriers of co-located adjacent channel operators

According to the last meeting agreement, the example bands consider n2/n25, n3, n7, n66, n41, n39, n26 as the example band. We can take n25 as an example in Fgiure1. The configuration that CA_n25 (2A), PCC+SCC=40M+5MHz, Wgap=20MHz.  It is observed that IMD5 and IMD7 may fall into the gap, making the in-gap interference increase. Hence, in our understanding, it is necessary to consider the in-gap interferer caused by IMD5 and IMD7.

Figure1: IMD5 and IMD7 interfere n25 DL NCCA

Proposal 2: The influence of IMD5 and IMD7 on in-gap interference needs to be considered.

Regarding performance evaluation based on existing ACS requirements, due to the limited time, we didn’t do simulation and testing evaluation. For ACS DL NCCA requirements, which are defined with the uplink configuration in accordance with Table 7.3A.2.2-1.For this uplink configuration, the UE shall meet the requirements for each sub-block are specified in clauses 7.5 for one component carrier. According to the Table 7.5-3 for case1 and Table 7.5-4 for case2, it is observed that the power of interferer is higher than 31.5dB compared to wanted signals. In our understanding, power spectral density (PSD) difference,  as we assumed here, is confined by the ADC dynamic range, since there will be nothing helped from the filter when we use a wide filter covering both carrier components. It is a challenge to control the AGC table, which will cause performance degradation. Depending on the waveforms of the in-gap interference, the peak to average ratio has to be reserved. If we use partially shared architecture, we can suppress the in-gap interference by 30dB. However, now we need to use fully shared architecture, we can bear the in-gap interference by 31.5-30=1.5dB. Considering the margin, our ADC resolution will reserve two bits of accuracy, it can bear an additional 12dB. And we need to consider the margin from 3dB to 6dB. Hence, 1.5dB+12dB-6dB=7.5dB. In addition, based on the above analysis, we need to keep some margin to consider the interference caused by IMD5 and IMD7, except the interference caused by adjacent channel operators. Generally speaking, in our understanding, we can use 6dB as PSD difference between carriers of co-located adjacent channel operators to be a starting point. If the operators cannot guarantee or control the power level of the interference by 6dB, we can judge by UE measurement and reporting.

Proposal 3: Use 6dB as PSD difference between carriers of co-located adjacent channel operators to be a starting point.

Semi-statically switch hardware resources

The SID [2] mentioned that the ability to semi-statically switch hardware resources means that UE is needed to switch between fully shared RX chain and separate RX chains for DL NCCA( two intra-band non-contiguous carriers). It is observed that the performance (i.e., ACS, In-band blocking requirements, SNR) of fully shared RX chain is degraded compared to partially shared RX chain. From our perspective, it is hardly possible to maintain the ACS and in-band blocking requirements between fully shared RX chain and separate RX chain. Therefore, it is necessary to define the switching criteria between separate RX chains and fully shared RX chain for DL NCCA. The switching criteria can be SNR, power level of in-gap interference or others. With regard to how to get these results, gNB could trigger or indicate UE to measure when gNB needs to configure the UE with more multiple CCs or UE could do some measurements periodically autonomously.

Proposal 4: To define the switching criteria between separate RX chains and fully shared RX chain for DL NCCA should be considered.

Proposal 5: gNB trigger or indicate UE or UE do some measurements autonomously could be considered.

In addition, the potential impact of switching hardware resources needs to be considered, which maybe interruption, the degradation, and so on.

Proposal 6: Potential impact of switching hardware resources needs to be considered.

Impacts on DL RX requirements

In the last meeting, some companies discussed the impacts on DL Rx requirements. In our understanding, the purpose of reducing the number of RX chains is to enable more number of CCs. SNR dynamic range of fully shared architecture compared to partially shared architecture can be decreased. However, there should be no performance degradation. Otherwise, it can’t be called DL NCCA. Which means that the coverage range can be reduced, such as REFESENS and Max input level, but performance is retained. At the same time, the interferer for ACS and IBB/NBB requirements can be reduced but throughput can be retained. If the interference is too high, we can switch from fully shared architecture to partially shared architecture or switch from 2CC to single CC.

Proposal 7: Tolerable interference reduced may be considered. No performance degradation between fully shared architecture and partially shared architecture for DL NCCA should be considered.

Conclusions

In this paper, we provide our views on methods for reducing the number of UE RX chains, we have made the following proposals.

Proposal 1: Shared digital BB or separate digital BB up to implementation. Separate digital BB are priority to be considered.

Proposal 2: The influence of IMD5 and IMD7 on in-gap interference needs to be considered.

Proposal 3: Use 6dB as PSD difference between carriers of co-located adjacent channel operators to be a starting point.

Proposal 4: To define the switching criteria between separate RX chains and fully shared RX chain for DL NCCA should be considered.

Proposal 5: gNB trigger or indicate UE or UE do some measurements autonomously could be considered.

Proposal 6: Potential impact of switching hardware resources needs to be considered.

Proposal 7: Tolerable interference reduced may be considered. No performance degradation between fully shared architecture and partially shared architecture for DL NCCA should be considered.

- Reference

- R4-2417201,WF on fragmented DL carriers study, Mediatek Inc. TSG-RAN WG4 112bis meeting