3GPP TSG-RAN WG4 Meeting #113															 R4-2418617

Orlando, US, 18th – 22nd November, 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.133 | CR | 7357 | rev | - | Current version: | 18.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR on RLM test cases for NB-IoT over NTN |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MediaTek inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN4 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_NBIOT_eMTC_NTN_req-Core |  |  |  |  |  | Date: |  |  | 2024-11-8 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The following agreement was reached in RAN4#105 R4-2220362 regarding the timing alignment validation for PUR: Agreement - Do not define requirements for the legacy RSRP-based TA validation for PUR for both GEO and LEO Besides, according to cl. 5.3.3.19 in 36.331, since there is no defined requirement for the legacy RSRP-based TA validation for NTN, the subsequent condition for timing alignment validation will rely on pur-TimeAlignmentTimer: #### 5.3.3.19 Timing alignment validation for transmission using PUR The UE shall consider the timing alignment value for transmission using PUR to be valid when the following conditions are fulfilled: 1> either pur-TimeAlignmentTimer is not configured or pur-TimeAlignmentTimer is running as confirmed by lower layers; and 1> either pur-RSRP-ChangeThreshold (pur-NRSRP-ChangeThreshold in NB-IoT) is not configured or the following conditions are fulfilled: |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarify the TA validation conditions for PUR in IoT NTN. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | FFS remains |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.6A.3.3, 4.7A.4.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## <<< START OF CHANGES 1>>>

#### 4.6A.3.3	Requirements on TA validation for transmission using PUR

The UE is allowed to transmit using PUR provided that TA is valid when the following conditions are fulfilled:

- either pur-TimeAlignmentTimer is not configured or pur-TimeAlignmentTimer is running as confirmed by lower layers, as defined in clause 5.3.3.19 in TS 36.331 [2].

The UE shall determine the uplink timing,  TTA, for transmitting on PUR using the valid values of NTA, NTA,offset, NTA,adjcommon and   NTA,adjUE where TTA is defined in [16] which are the latest acquired values before PUR transmission.

## <<< END OF CHANGES 1>>>

## <<< START OF CHANGES 2>>>

#### 4.7A.4.3	Requirements on TA validation for transmission using PUR

The UE is allowed to transmit using PUR provided that TA is valid when the following conditions are fulfilled:

- either pur-TimeAlignmentTimer is not configured or pur-TimeAlignmentTimer is running as confirmed by lower layers, as defined in clause 5.3.3.19 in TS 36.331 [2].

The UE shall determine the uplink timing,  TTA, for transmitting on PUR using the valid values of NTA, NTA,offset, NTA,adjcommon and   NTA,adjUE where TTA is defined in [16] which are the latest acquired values before PUR transmission.

## <<< END OF CHANGES 2>>>