3GPP TSG-RAN WG4 Meeting #113	    R4- 2418624

Orlando, US, 18th – 22nd November 2024

Agenda item:	7.4.3

Source:	Intel Corporation

Title:	Discussion on the RRM requirements for Rel-19 less than 5MHz UE

Document for:	Discussion

# Introduction

The Rel-19 NR channel BW less than 5MHz for FR1 Phase 2 work item was updated in June 2024 in [1]. The work item Core part objectives are provided below.

- Define common co-located and non-co-located inter-band NR CA/DC UE RF requirements with 3MHz CBW in the one band and 5MHz or 10MHz CBW in the other band - Example band combination: CA/DC of 3MHz in band n100 and 5MHz or 10MHz in band n101 - Define RRM requirements for inter-band CA and DC for combinations introduced in RF part Note: other band combinations than example band combinations can be specified in basket WIs after the above generic requirements are specified.

In the last meeting MCC decided to postpone the publication of TS 38.133 Rel-19 first version until Q1 2025, and the draft Big CR in the last work group meeting for RRM requirements in this item [2] was then also postponed accordingly. In this meeting, we target to endorse the draft Big CR for the RRM requirements in this work item scope.

In this paper we provide our analysis on the RRM requirements for the work item.

# RRM requirements

## Carrier aggregation with 12 PRB SSB bandwidth SCell

Regarding supporting carrier aggregation when one of the bands in the combination is configured with less than 5MHz channel bandwidth, we need to specify new UE requirements for SCell operations targeting an SCell with smaller SSB bandwidth than those in the legacy specifications.

In Rel-18, consensus was that the measurement performance is not impacted when the target SSB has smaller BW than 5MHz, but only the PBCH bandwidth was reduced. However, the detection delay of a cell with 12 PRB or 15 PRB bandwidth was relaxed, when SSB index detection is necessary in the process.

Time period requirements for time index detection on deactivated SCell with 12PRB SSB bandwidth

In the Rel-18 specification, the SSB index detection delay was relaxed in the below manner – comparison is shown in the below box between T_SSB_time_index_intra and T_SSB_time_index_intra_less_than_5Mhz.

Table 9.2.6.2-3: Time period for time index detection (Frequency range FR1) DRX cycle; TSSB_time_index_intra. No DRX; max(120ms, ceil(3 x Kgap ) x max(MGRP, SMTC period)) x CSSFintra. DRX cycle≤ 320ms; max(120ms, ceil(M2Note 1x 3 x Kgap) x max(MGRP, SMTC period,DRX cycle) x CSSFintra). DRX cycle>320ms; Ceil(3 x Kgap )x max(MGRP, DRX cycle) x CSSFintra. NOTE 1: When highSpeedMeasFlag-r16 is not configured, M2 = 1.5; When highSpeedMeasFlag-r16 is configured, M2 = 1.5 if SMTC periodicity > 40 ms, otherwise M2=1. NOTE 2: When highSpeedMeasFlag-r16 is configured, the requirements apply only to UE supporting either measurementEnhancement-r16 or intraNR-MeasurementEnhancement-r16 on measurements of the primary component carrier and do not apply to measurements of a secondary component carrier with active SCell. NOTE 3: For a UE supporting concurrent GAPs, if multiple concurrent GAPs are configured, the MGRP is the periodicity of the activated Pre-MG or the MG pattern associated to the intra-frequency layer. NOTE 4: When highSpeedMeasCA-Scell-r17 is configured, the requirements apply to UE on measurements of secondary component carrier with active SCell.. Table 9.2.6.1-12: Time period for time index detection for a UE operating on a target cell with 12 PRB SSB (Frequency range FR1) (Frequency range FR1) DRX cycle; TSSB_time_index_intra_less_than_5Mhz. No DRX; max(120ms, 7 x max(MGRP, SMTC period)) x CSSFintra. DRX cycle≤ 320ms; max(120ms, ceil(1.5 x 7) x max(MGRP, SMTC period,DRX cycle) x CSSFintra). DRX cycle>320ms; 7 x max(MGRP, DRX cycle) x CSSFintra. Note 1: Void NOTE 2: FFS When highSpeedMeasInterFreq-r17.

Similarly, we need to specify the time period requirements for time index detection for UE operating on a target cell with less than 5MHz SSB for deactivated SCell measurements in FR1.

Note that for the corresponding requirements for intra-frequency measurement with NCSG, since there were no requirements specified in Rel-18 for target SSB with 12PRB bandwidth, we propose not to introduce the Rel-19 requirements for these scenarios. Regarding measurements with CCA and/or for a ATG UE, same situation applies.

Proposal 1: Specify time period for time index detection for a UE operating on a target cell with 12PRB SSB, deactivated SCell in FR1 as in the following table.

Table 9.2.5.1-x: Time period for time index detection for a UE operating on a target cell with 12PRB SSB, deactivated SCell (FR1)

| DRX cycle | TSSB_time_index_intra |
| --- | --- |
| No DRX | Ceil(7 x Kp) x measCycleSCell x CSSFintra |
| DRX cycle≤ 320ms | Ceil(7 x Kp) x max(measCycleSCell, 1.5xDRX cycle) x CSSFintra |
| DRX cycle> 320ms | Ceil(7 x Kp) x max(measCycleSCell, DRX cycle) x CSSFintra |
| NOTE 1: The requirements also apply to deactivated SCG SCell. |  |

Draft Big CR submitted to this meeting reflecting the changes is in [3].

Fine time tracking time on 12PRB SSB BW when UE carries out SCell activation

In the SCell activation delay requirements, fine time tracking after search of the target cell is considered and when the target SSB BW is smaller than 5MHz the legacy requirement does not apply. This is to say that we need to specify new requirements for fine time tracking time, which is included in the SCell activation delay requirements.

In the last meeting we had the agreement that for both known and unknown SCell activations, extended fine time tracking time applies.

To help understand, fine time tracking time for less than 5MHz target HO requirements is copied in the below box.

When intra-frequency or inter-frequency handover is commanded, the interruption time shall be less than Tinterrupt Tinterrupt = Tsearch + TIU + Tprocessing + T∆ + Tmargin ms Where: - If the target cell is an unknown inter-frequency cell and Tsearch = 3* Trs ms if the target cell Es/Iot≥-2 dB. T∆ is time for fine time tracking and acquiring full timing information of the target cell. T∆ = Trs for both known and unknown target cells operating with 20 PRB SSB BW. T∆ = 3*Trs for both known and unknown target cells operating with 12 PRB SSB BW.

Proposal 2: Specify fine time tracking time equals 3 SSB periods instead of 1 when the target cell is with 12PRB SSB bandwidth, as is included in the SCell activation delay requirements in FR1 in Rel-19.

Tactivation_time is the SCell activation delay in millisecond. If the SCell is known and belongs to FR1, Tactivation_time is: - TFirstSSB_MAX + 3 * Trs + 5ms, if the UE supporting [12PRB SSB bandwidth UE feature] is configured with a target SCell that is with 12PRB SSB bandwidth.

If the SCell is unknown and belongs to FR1, and if one of the following conditions is met - ‘ssb-PositionInBurst’ indicates only one SSB is being actually transmitted, or - ‘ssb-PositionInBurst’ indicates multiple SSBs and TCI indication is provided in same MAC PDU with SCell activation, - TFirstSSB_MAX + TSMTC_MAX + 4*Trs + 5ms, if the UE supporting [12PRB SSB bandwidth UE feature] is configured with a target SCell that is with 12PRB SSB bandwidth.

- Otherwise - 6ms + TFirstSSB_MAX + TSMTC_MAX + 3 * Trs + TL1-RSRP, measure + TL1-RSRP,report + THARQ + max(Tuncertainty_MAC + TFineTiming + 2ms, Tuncertainty_SP), if semi-persistent CSI-RS is used for CSI reporting, and the UE supporting [12PRB SSB bandwidth UE feature] is configured with a target SCell that is with 12PRB SSB bandwidth, - 3ms + TFirstSSB_MAX + TSMTC_MAX + 3 * Trs + TL1-RSRP, measure + TL1-RSRP,report + max(THARQ + Tuncertainty_MAC + 5ms + TFineTiming, Tuncertainty_RRC + TRRC_delay), if periodic CSI-RS is used for CSI reporting, and the UE supporting [12PRB SSB bandwidth UE feature] is configured with a target SCell that is with 12PRB SSB bandwidth.

Draft Big CR submitted to this meeting reflecting the changes is in [3].

## NR-DC with 12PRB SSB bandwidth PSCell

Time period requirements for time index detection on deactivated PSCell with less than 5MHz SSB bandwidth

The similar requirements as in the NR CA discussed in the above section are expected to be also introduced for NR-DC with a PSCell that has 12PRB SSB bandwidth.

Proposal 3: Specify time period for time index detection for a UE operating on a target cell with 12PRB SSB, deactivated PSCell in FR1 as in the following table.

Table 9.2.5.1-y: Time period for time index detection for a UE operating on a target cell with 12PRB SSB, deactivated PSCell (FR1)

| DRX cycle | TSSB_time_index_intra |
| --- | --- |
| No DRX | Ceil(7 x Kp) x measCyclePSCell x CSSFintra |
| DRX cycle≤ 320ms | Ceil(7 x Kp) x max(measCyclePSCell, 1.5xDRX cycle) x CSSFintra |
| DRX cycle> 320ms | Ceil(7 x Kp) x max(measCyclePSCell, DRX cycle) x CSSFintra |

PSCell addition delay and (subsequent) conditional PSCell addition delay where PSCell SSB is 12PRB bandwidth

In the last working group meeting it is agreed that when PSCell is configured with less than 5MHz channel bandwidth, RAN4 needs to specify new PSCell addition and (subsequent) conditional PSCell addition delay requirements. Similar to handover and SCell activation, the fine time tracking delay needs to be relaxed.

Proposal 4: Specify fine time tracking time equals 3 SSB periods instead of 1 when the target cell is with 12PRB SSB bandwidth, as is included in the PSCell addition and (subsequent) conditional PSCell addition delay requirements.

### 8.9.2 PSCell Addition Delay Requirement Upon receiving PSCell addition in subframe n, the UE shall be capable to transmit PRACH preamble towards PSCell in FR1 or FR2 no later than in slot n+Tconfig_PSCellNR slot length: where: T∆ is time for fine time tracking and acquiring full timing information of the target cell. T∆ = 1*Trs ms for a known or unknown PSCell operating with 20 PRB SSB BW. T∆ = 3*Trs ms for either known or unknown target PSCell operating with 12 PRB SSB BW.

### 8.9A.2 Conditional PSCell Addition Delay Requirement The requirements in this clause shall apply for the UE configured with only PCell in FR1. Upon receiving conditional PSCell addition in subframe n, the UE shall be capable to transmit PRACH preamble towards PSCell no later than in subframe n + Tconfig_PSCell_Addition_Conditional: where: T∆ is time for fine time tracking and acquiring full timing information of the target cell. T∆ = 1*Trs ms for PSCell operating with 20 PRB SSB BW. T∆ = 3*Trs ms for either known or unknown target PSCell operating with 12 PRB SSB BW.

### 8.9C.2 Subsequent Conditional PSCell Addition Delay Requirement The requirements in this clause shall apply for the UE configured with only PCell in FR1. Upon receiving RRC Command containing MR-DCRelease configuration and sending an RRCReconfigurationComplete message confirming the release in subframe n, the UE shall be capable to transmit PRACH preamble towards PSCell no later than in subframe n + Tconfig_PSCell_Addition_Conditional: where: T∆ is time for fine time tracking and acquiring full timing information of the target cell. T∆ = 1*Trs ms for PSCell operating with 20 PRB SSB BW. T∆ = 3*Trs ms for either known or unknown target PSCell operating with 12 PRB SSB BW.

PSCell change and (subsequent) conditional PSCell change delay where PSCell SSB is 12PRB bandwidth

Automatically, the PSCell change delay requirements follow the PSCell addition delay requirements.

Proposal 5: Specify fine time tracking time equals 3 SSB periods instead of 1 when the target cell is with 12PRB SSB bandwidth, as is included in the PSCell change and (subsequent) conditional PSCell change delay requirements.

## 8.11 PSCell Change This clause defines requirements for the delay within which the UE shall be able to change PSCell to other cell in EN-DC or NR-DC. The requirements in this clause are applicable to EN-DC and NR-DC. In the case of NR-DC PSCell change in FR1, when the UE supporting [12PRB SSB bandwidth UE feature] is configured with a target PSCell that is with 12PRB SSB bandwidth, the UE shall transmit PRACH preamble towards the target PSCell no later than the delay requirements specified in clause 8.9.2.

### 8.11B.2 Conditoinal PSCell Change delay The requirements in this clause shall apply for the UE configured with only PCell in FR1. The UE shall be capable to transmit PRACH preamble towards the new target PSCell no later than in slot n + Tconfig_PSCell_Conditional: where: T∆ is time for fine time tracking and acquiring full timing information of the target cell. T∆ = 1*Trs ms for PSCell operating with 20 PRB SSB BW. T∆ = 3*Trs ms for either known or unknown target PSCell operating with 12 PRB SSB BW.

### 8.11E.2 Subsequent Conditoinal PSCell Change delay The requirements in this clause shall apply for the UE configured with only PCell in FR1. The UE shall be capable to transmit PRACH preamble towards the new target PSCell no later than in slot n + Tconfig_PSCell_Subsequent_Change_Conditional: where: T∆ is time for fine time tracking and acquiring full timing information of the target cell. T∆ = 1*Trs ms for PSCell operating with 20 PRB SSB BW. T∆ = 3*Trs ms for either known or unknown target PSCell operating with 12 PRB SSB BW.

Draft Big CR submitted to this meeting reflecting the changes is in [3].

Handover with PSCell and CHO with PSCell – PSCell change delay

In the last meeting we agreed that for conditional handover including target MCG and target SCG, we also need to extend the fine time tracking time.

##### 6.1.6.1.2 CHO with PSCell – PSCell change delay The requirements in this section shall apply for PSCell change during conditional handover with PSCell from NR DC to NR-DC. When conditional handover with PSCell from NR-DC to NR-DC is commanded, the PSCell change time shall be less than DCHOwithPSCell_PSCell: - DCHOwithPSCell_PSCell = TRRC + TEvent_DU + Tmeasure + TCHO_execution + Tprocessing + Tsearch_PCell_Conditional + Tsearch_PSCell + T∆_PSCell + TPSCell_DU + 2 ms Where: - T∆_PSCell is time for fine time tracking and acquiring full timing information of the target cell. T∆_PSCell = 1*Trs ms for a known or unknown PSCell operating with 20 PRB SSB BW. T∆_PSCell = 3*Trs ms for a known or unknown PSCell operating with 20 PRB SSB BW.

Proposal 6: Specify fine time tracking time equals 3 SSB periods instead of 1 when the target cell is with 12PRB SSB bandwidth, as is included in the CHO with target MCG and target SCG delay requirements.

# Conclusion

Proposal 1: Specify time period for time index detection for a UE operating on a target cell with 12PRB SSB, deactivated SCell in FR1 as in the following table.

Table 9.2.5.1-x: Time period for time index detection for a UE operating on a target cell with 12PRB SSB, deactivated SCell (FR1)

| DRX cycle | TSSB_time_index_intra |
| --- | --- |
| No DRX | Ceil(7 x Kp) x measCycleSCell x CSSFintra |
| DRX cycle≤ 320ms | Ceil(7 x Kp) x max(measCycleSCell, 1.5xDRX cycle) x CSSFintra |
| DRX cycle> 320ms | Ceil(7 x Kp) x max(measCycleSCell, DRX cycle) x CSSFintra |
| NOTE 1: The requirements also apply to deactivated SCG SCell. |  |

Draft Big CR submitted to this meeting reflecting the changes is in [3].

Proposal 2: Specify fine time tracking time equals 3 SSB periods instead of 1 when the target cell is with 12PRB SSB bandwidth, as is included in the SCell activation delay requirements in FR1 in Rel-19.

Proposal 3: Specify time period for time index detection for a UE operating on a target cell with 12PRB SSB, deactivated PSCell in FR1 as in the following table.

Table 9.2.5.1-y: Time period for time index detection for a UE operating on a target cell with 12PRB SSB, deactivated PSCell (FR1)

| DRX cycle | TSSB_time_index_intra |
| --- | --- |
| No DRX | Ceil(7 x Kp) x measCyclePSCell x CSSFintra |
| DRX cycle≤ 320ms | Ceil(7 x Kp) x max(measCyclePSCell, 1.5xDRX cycle) x CSSFintra |
| DRX cycle> 320ms | Ceil(7 x Kp) x max(measCyclePSCell, DRX cycle) x CSSFintra |

Proposal 4: Specify fine time tracking time equals 3 SSB periods instead of 1 when the target cell is with 12PRB SSB bandwidth, as is included in the PSCell addition and (subsequent) conditional PSCell addition delay requirements.

Proposal 5: Specify fine time tracking time equals 3 SSB periods instead of 1 when the target cell is with 12PRB SSB bandwidth, as is included in the PSCell change and (subsequent) conditional PSCell change delay requirements.

Proposal 6: Specify fine time tracking time equals 3 SSB periods instead of 1 when the target cell is with 12PRB SSB bandwidth, as is included in the CHO with target MCG and target SCG delay requirements.

# References

- RP-241622, Revised WID: NR channel BW less than 5MHz for FR1 Phase 2, Intel Corporation

- R4-2416086, Draft Big CR for less than 5MHz RRM requirements in Rel-19, Intel Corporation

- R4-2418625, Draft Big CR for less than 5MHz RRM requirements in Rel-19, Intel Corporation