3GPP TSG-RAN WG4 Meeting # 113	R4-2418655

# Orlando, US, 18th – 22nd November, 2024

Agenda item:	7.19.3.1

Source: 	Huawei, HiSilicon

Title: 	Discussion on RRM impact for Rel-19 MIMO on UE initiated BM

Document for:	Discussion

# Introduction

The Rel-19 MIMO WI is approved in [1] and further updated in [2], RAN4 has initiate the discussion in lasting. In this contribution, we further provide our views regarding the RAN4 impacts for the WI.

# Discussion

Based on the discussion in last meeting, the agreements reached are summarized as follows:

Issue 1-1: How to define Event triggered measurement reporting delay? < Agreement> - Start point of Event triggered measurement reporting delay: reuse the same definition in L3 event trigger report which is the event is met over the air. - Ending point is the point when the UE transmits a [first PUCCH] triggered by the event. - “[first PUCCH]” can be updated based on further RAN1/2 agreement. Issue 1-3: Applicability of Event triggered measurement reporting delay requirements? < Agreement> - Applicable for all events agreed in RAN1/2 - Applicable to both Mode A and B - Note: applicable to the event/mode supported by UE Issue 1-7: Whether to support CSI-RS based L1-RSRP for aperiodic? <Agreement > - RAN4 to define CSI-RS based UEIBM L1-RSRP requirements for periodic CSI-RS. CSI-RS based UEIBM L1-RSRP requirements for semi-persistent CSI-RS are FFS pending on RAN1 further agreement to support semi-persistent CSI-RS.

Based on the discussion in last meeting, RAN4 spent quite a lot time slot to discuss the ending point of the Event triggered measurement reporting, e.g. whether it should be the PUCCH or reporting considering different Mode. As commented during last meeting, the essential part from RAN4 perspective is the measurement behaviour/performance. The reporting procedure will only be reflected as an uncertainty in RAN4 requirements.

Observation 1: RAN4 shall focus the measurement behaviour/performance of UE-initiated/event-driven beam management.

Another issue is about how to consider the window of triggering event determination in Event triggered measurement reporting. The status are summarized as follows:

Issue 1-4: How to consider the time window of triggering event determination in Event triggered measurement reporting? <Way forward > - Option 1: - RAN4 needs to consider the impact of triggering window defined in RAN1. - Option 2: - It is proposed to define requirements for both the case without filtering and the case with filtering (i.e. timer and counter M). - It is proposed that same reporting requirements are applied to the case without filtering and the case with filtering (i.e. timer and counter M) - Option 3: - No additional time-window-based filtering or other filtering should be applied for the L1-RSRP measurements - Option 4: - Discuss the necessity of filtering based on the two trigger-event detection solutions approved in RAN1. - For the basic solution, one-shot measurement is defined by RAN1. No room to consider filtering. - For the optional solution, multi-shot measurement with at least M instance is defined by RAN1. Filtering is feasible. - Option 5: - RAN4 to discuss how to define one Event-2 instance for following Event-2 case: - If within a time window (which is configurable), the number of Event-2 instance(s) for at least one same new beam is greater than or equal to a configurable number M, UE initiated beam report occurs. - Note: Event-2 instance for a new beam is determined if the L1-RSRP of the new beam becomes a threshold value better than the current beam - Option 6: - Wait further RAN1 progress. - Other options are not precluded.

Based on the RAN1 discussion in last meeting, there is relevant RAN1 discussion about the evaluation period with following agreements.

Agreement Regarding the triggering event determination for Event 2, down-select among the following alternatives for the evaluation periodicity for determining Event-2 instance [at least when DRX is not configured] - Alt-1: The periodicity of the current beam RS should be the same as that of the new beam RS(s). - The evaluation periodicity is the same as the periodicity of the current and new beam RS(s) - Alt-2: The periodicity of the current beam RS can be different from that of the new beam RS(s) - Alt-2_1: The evaluation periodicity is the same as the periodicity of the current beam RS; - Alt-2_2: The evaluation periodicity is the same as periodicity of the new beam RS; - Alt-2_3: The evaluation periodicity is the same as shortest periodicity of the current beam RS and new beam RS(s): - Alt-2_4: The evaluation periodicity is the maximum of {X ms, shortest periodicity of the current beam RS and new beam RS(s)}: - Alt-2_5: The evaluation periodicity is the same as largest periodicity of the current beam RS and new beam RS(s): Note: There is the same periodicity for the new beam RS(s).

Though RAN1 will further discuss the issue, we would like to provide our views on potential impacts on UE measurement behaviour and performance. Take following figure as example. From our understanding, the issue mentioned in RAN1 is for the case when current beam and new beam has different periodicities. For instance, the periodicity of current beam is 80 ms, and that of new beam is 20 ms. If the evaluation periodicity is determined based on the periodicity of one of the RS, for instance, the periodicity of the new beam which is 20 ms. Then UE shall evaluate whether the event is triggered every 20 ms, in T2/T3/T4, actually there is no new results from current beam. Is UE still required to count number of instances? If UE obtains a low result of current beam in T1 due to channel fading, and if UE takes this result to evaluate the event in T1/2/3/4, it is possible that UE mistakenly counts 4 instances and triggering the report to NW, which will lead to wrong NW configuration (e.g. switch the TCI though the current beam is still good.)

Fig. 1 Event instance wrongly counted

Observation 2: When the periodicity of current beam and new beam are different, one over/under-estimated result due to instant channel fading may leads to several wrongly counted Event instance, which will mislead NW configuration.

Thus, one reasonable approach is that the event instance evaluation shall be based on new results from both current and new beam, which means one measurement result shall not be used for multiple Event instance counting.

Proposal 1: For event instance determination for event 2, event instance determination shall be based on new results from both current and new beam, which means one measurement result shall not be used for multiple Event instance counting.

Regarding the evaluation periodicity, which is under discussion in RAN1, RAN4 shall wait for further RAN1 agreement. From our understanding, it may also impact UE measurement behaviour since RAN4 may further discussion whether UE is allowed for relaxed measurement considering the length of evaluation window.

Proposal 2: RAN4 to wait for further RAN1 agreement on evaluation periodicity and FFS whether to consider relaxed measurement consider the length of evaluation window.

Another issue is about CSI-RS based L1-RSRP for inter-Cell BM, the status is summarized as follows:

Issue 1-8: CSI-RS based L1-RSRP for inter-cell BM? <Way forward > - Option 1: - RAN4 should not introduce in Rel-19 CSI-RS based L1-RSRP measurements for inter-cell beam management as long as RAN1 has not identified a clear use case - Option 2: - For CSI-RS based inter-cell L1-RSRP measurement, since no specific discussion before, RAN4 needs to identify whether additional clarifications on the following points are necessary: - Known condition for the NSC, especially for the time offset assumption between serving cell and NSC; - The prerequisite on L3 measurement on NSC. - Option 3: - RAN4 to define CSI-RS based L1-RSRP measurements requirements for inter-cell mTRP scenario in FR2. - RAN4 to use principles of sharing agreed for SSB based L1-RSRP for inter-cell CSI-RS based L1-RSRP measurements. - Other options are not precluded.

From our understanding, the inter-cell/intra-cell is the definition only differentiated w.r.t SSB since UE needs detect SSB with same SSB index from different TRP/Cell with different PCI. However, for CSI-RS there is no such differentiations. Different CSI-RS resources from different TRP/Cells can QCLed to SSB with different PCI.

Observation 3: From UE perspective, for CSI-RS based BM, there is no differentiation w.r.t. intra-cell or inter-cell.

Proposal 3: CSI-RS based L1-RSRP measurement requirements applies to both intra-cell and inter-cell without differentiation.

Another issue is about whether there is impacts on known conditions for TCI state switching. Take known conditions as example, which is shown as follows, the known conditions are determined by whether there is L1-RSRP reporting before TCI state switching. For UEIBM, the reporting content is also L1-RSRP. Thus, the existing known conditions can already cover the case where the L1-RSRP is triggered by UEIBM.

### 8.15.2 Known conditions for downlink TCI state The downlink TCI state is known if the following conditions are met: - During the period from the last transmission of the RS resource used for the L1-RSRP measurement reporting for the target downlink TCI state to the completion of active downlink TCI state switch, where the RS resource for L1-RSRP measurement is the RS in target downlink TCI state or QCLed to the target downlink TCI state - Downlink TCI state switch command is received within 1280 ms upon the last transmission of the RS resource for beam reporting or measurement - The UE has sent at least 1 L1-RSRP report for the target downlink TCI state before the downlink TCI state switch command - The target downlink TCI state remains detectable during the downlink TCI state switching period - The SSB associated with the downlink TCI state remain detectable during the downlink TCI switching period - SNR of the downlink TCI state ≥ -3dB - The SSB can be associated with either the serving cell PCI or a PCI different from serving cell PCI. Otherwise, the downlink TCI state is unknown.

Proposal 4: The existing known conditions for TCI state switching can cover the case that L1-RSRP reporting triggered by UEIBM.

# Conclusions

Observation 1: RAN4 shall focus the measurement behaviour/performance of UE-initiated/event-driven beam management.

Observation 2: When the periodicity of current beam and new beam are different, one over/under estimated result due to instant channel fading may leads to several wrongly counted Event instance, which will mislead NW configuration.

Proposal 1: For event instance determination for event 2, event instance determination shall be based on new results from both current and new beam, which means one measurement result shall not be used for multiple Event instance counting.

Proposal 2: RAN4 to wait for further RAN1 agreement on evaluation periodicity and FFS whether to consider relaxed measurement consider the length of evaluation window.

Observation 3: From UE perspective, for CSI-RS based BM, there is no differentiation w.r.t. intra-cell or inter-cell.

Proposal 3: CSI-RS based L1-RSRP measurement requirements applies to both intra-cell and inter-cell without differentiation.

Proposal 4: The existing known conditions for TCI state switching can cover the case that L1-RSRP reporting triggered by UEIBM.

# References

[1] RP-234007 New WID: NR MIMO Phase 5

[2] RP-242394 WID revision: NR MIMO Phase 5

[3] R4-2414035 WF on RRM requirements for NR_MIMO_Ph5