3GPP TSG-RAN WG4 Meeting # 113	R4-2418656

# Orlando, US, 18th – 22nd November, 2024

Agenda item:	7.19.3.2

Source: 	Huawei, HiSilicon

Title: 	Discussion on RRM impact for Rel-19 MIMO on other aspect

Document for:	Discussion

# Introduction

The Rel-19 MIMO WI is approved in [1] and further updated in [2], RAN4 has discussed the RRM impacts of the objectives with agreements and issues captured in [3][4]. In this contribution, we further provide our views regarding the RAN4 impacts for the WI.

# Discussion

## CSI enhancement to support up to 128 ports

For CSI enhancement to support up to 128 ports, the status in last RAN4 meeting is summarized as follows:

Issue 1-1: Whether RRM impacts exist by the added SRS port grouping for CSI enhancement? In last RAN plenary meeting RAN#105, another sub-bullet was approved for CSI enhancement (Objective#2): - SRS port grouping and its association to the two codewords for the 6/8Rx low complexity receiver supporting more than 4 layers, with legacy codebook - No enhancement on codeword-to-layer mapping, DL resource allocation, CSI feedback, and DCI format - Note: Whether to support 6Rx with more than 4 layers is to be decided in RAN4 Rel-19 RF enhancements WI. Way forward: - Option 1: Previous RAN4 agreement still apply “No RRM impact by CSI enhancement to support up to 128 ports”. - Option 2: Wait for RAN1.

Based on the discussion in last meeting, companies want to wait for further RAN1 progress to check whether there is RRM impacts for the new added objective about SRS porting grouping. Per latest RAN1 agreements, the discussion is only about mapping between SRS ports mapping. For SRS AS RRM requirements, the requirements are defined based on number of SRS resource per slot regardless of xTxR or specific ports. Thus, previous RAN4 agreements can still apply.

Proposal 1: No RRM impact for objective#2.

## UE reporting enhancement for CJT calibration

In the last meeting, we have an online understanding type, which is shown below:

Agreement: - RAN4 to simulate the performance of offsets for phase, time delay and frequency, based on one-shot measurement and multiple-sample measurement, and discuss the impact on RRM core requirement later. - Further discuss the simulation assumptions offline during this meeting. Way forward: - Companies are encouraged to bring simulation assumption/results in the next meeting. Simulation assumption in Appendix A can be used as a starting point for information only.

We have some considerations regarding the phase offset value that the UE will measure and report. According to the agreement from the RAN1 #116 meeting report, the phase offset in the system is modelled as a uniform random variable within the range [-y:y], where y is π for TDD. We propose adopting this value in the RAN4 simulation assumptions. The relevant agreement from the RAN1 #116 meeting is shown below.

Agreement For the Rel-19 NR CJT calibration reporting, reuse the EVM agreed for Rel-18 NR Type-II CJT codebooks (cf. R1-2205289) for SLS with the following refinement: - Adding ISD=500m as an option for inter-site CJT - Delay offsets caused by propagation delays from the UE to the TRPs and, whenever applicable, the ISI incurred by the composite (summed across selected TRP(s)) channel impulse response should be modelled/accounted - In addition, initial timing offsets among TRPs can be included, e.g. from TR 38.855 for positioning or uniformly distributed [-0.5CP:0.5CP] - Per-TRP frequency offset (relative to the UE) is modelled as a uniform random variable with [-x:x] - Companies should state the assumed value of x, e.g. 0.05ppm, 0.1ppm (per Table 6.5.1.2-1 of TS.38.104) - For TDD, per-TRP DL/UL phase misalignment is modelled as a uniform random variable with [-y:y] where y is {} - For TDD, per-TRP DL/UL timing misalignment is modelled as a uniform random variable with [-z:z] - Companies should state the assumed value of z, e.g. 65ns(from RAN4) - For FDD scenarios, Rel-18 Type-II CJT codebook is assumed - For TDD scenario, SRS-based (e.g. SVD) precoding is assumed - Companies should state the assumed sub-carrier spacing, e.g. 15kHz, 30kHz

Proposal 2:	Phase difference between TRP (only for phase offset simulations) are -π,-π/2,π/2,π as the simulation assumption.

| Parameter | Value |
| --- | --- |
| Duplex, Waveform | TDD, OFDM |
| Carrier Frequency | 3.5 GHz |
| Channel Model | TDL-C |
| Delay Spread | 300ns |
| antenna configuration | 1X2 |
| TRP number | 2 |
| TRS Bandwidth | 48RB, 106 RB |
| Numerology | 14 OFDM symbol per slot, 30kHz SCS |
| UE speed | 3km/h, 30kmph, 72kmph, 108kmph |
| SNR | -3dB, 0dB, 3dB |
| Frequency offset between TRP (only for freq. offset simulations) | 0.1ppm. |
| Delay difference between TRP (only for delay offset simulations) | 0.5CP, CP |
| Phase difference between TRP (only for phase offset simulations) | -π, -π2,π2,π |
| Number of samples for measurement | 1, 2, 3, 4, 5, 10 |
| CSI-RS periodicity | 20ms / 40 slots |
| CSI-RS offset | Slot 20, 21 |

## Enhancement for asymmetric DL sTRP/UL mTRP scenarios

For the objective for asymmetric DL sTRP/UL mTRP, RAN4 only discussed the impact to supporting two TA in last meeting, with agreements captured as follows:

Issue 4-1: Whether RRM impacts exist by 2TA enhancement? Agreement: - RAN4 to update the RRM requirements for 2TA enhancement [for asymmetric DL sTRP/ UL mTRP] - As baseline, the Rel-18 framework can be considered as starting point, and the update is needed to accommodate the new scenario in this WID. - Further discuss the details in the next meeting.

Per Rel-18 MIMO discussion, for mTCI with two TAG, the DL reference timing is determined by the association to CoresetPoolIndex. For Rel-19 asymmetric DL sTRP/UL mTRP, RAN1 achieves following agreements in last meeting. It could be observed that the restriction on corsetPoolInex is removed. Besides, how to determine the downlink reference timing is still not very clear at current stage. RAN4 shall wait further RAN1 agreements on 2TA for asymmetric DL sTRP/UL mTRP to determine the DL reference timing for potential RRM requirements updating.

Agreement Support 2TA for the asymmetric DL sTRP/UL mTRP deployment scenarios: - Remove the restriction that coresetPoolIndex needs to be configured for the 2TA feature. - One downlink reference timing is supported and applied to both TAGs. - (FFS) Note: UE autonomous TA adjustment is only applicable to the first TAG - One single n-TimingAdvanceoffset is configured and applied to both TAGs. - Any of the TCI states can be associated with any one of the two TAGs. - The RAR carrying TA adjustment for those 2 TAGs is reused for Rel-19 2TA - The MAC CE based TA adjustment for 2 TAGs is reused for Rel-19 2TA. - Introduce the optional UE capability of “Overlapping UL transmission reduction” for Rel-19 2TA - If UE does not report this UE capability, UE does not expect two UL transmissions associated with different TAGs are overlapped. - FFS: UE does not expect that in intra-slot TDM PUSCH type-B repetition transmission, two consecutive repetitions associated with different TAGs are overlapped.

Proposal 3: RAN4 shall wait further RAN1 agreements on 2TA for asymmetric DL sTRP/UL mTRP to determine the DL reference timing for potential RRM requirements updating.

Another aspect which was not discussed in last meeting is about the requirements impacts for unified TCI state switching. The status is summarized as follows:

Issue 4-2: RRM core impacts of Active uplink TCI state switching delay for unified TCI? Way forward: - Option 1: No RRM impacts on UL TCI state switching delay requirement for unified TCI states. - Option 2: RAN4 to specify UL TCI state switching requirements. - Option 3: FFS. Issue 4-3: Whether to define RRM core requirements of pathloss offset update? Agreement: - Previous RAN4 agreement captured below still apply and close the issue. Issue 2-5-5: Whether to define RRM core requirements of pathloss offset update? Agreement: Whether to define RRM core requirements of pathloss offset update? - Not define RRM requirement for MAC CE decoding delay. - The agreement can be revisited depending on further RAN1 progress..

There is a LS to RAN1 from RF session to further clarify the scenario [5]. RAN1 is expected to clarify whether UL TRP can transmit SSB. Per WID description, there is no limitation since it is mentioned that DL transmission could be reduced or turned off to reduce energy consumption mean while to maximize UL throughput. Thus, legacy TCI state switching requirements can still apply. In addition, when the QCL source is the SRS, per legacy principles, no RRM requirements when the UL TCI states is associated with SRS.

Based on analysis above. There is no impact on TCI state switching requirements.

Proposal 4: No RRM impacts on TCI state switching delay requirement for unified TCI states as legacy and legacy requirements can apply.

# Conclusions

Proposal 1: No RRM impact for objective#2.

Proposal 2: Phase difference between TRP (only for phase offset simulations) are -π,-π/2,π/2,π as the simulation assumption.

Proposal 3: RAN4 shall wait further RAN1 agreements on 2TA for asymmetric DL sTRP/UL mTRP to determine the DL reference timing for potential RRM requirements updating.

Proposal 4: No RRM impacts on TCI state switching delay requirement for unified TCI states as legacy and legacy requirements can apply.

# References

[1] RP-234007 New WID: NR MIMO Phase 5

[2] RP-242394 WID revision: NR MIMO Phase 5

[3] R4-2416856 WF on RRM requirements for NR_MIMO_Ph5_Part1

[4] R4-2416921 WF on RRM requirements for NR_MIMO_Ph5_Part2