3GPP TSG-RAN WG4 Meeting # 113						                                               R4-2418668

Orlando, US, 18th – 22nd November, 2024

Title: 					Discussion on SSB-less SCell operation

Source: 	Huawei, HiSilicon

Agenda item:	5.27.1

Document for:	Discussion

Introduction

The core requirements on network energy savings have been completed at RAN4#109 meeting. This contribution provides analysis on the maintenance in core requirements for network energy saving.

Discussion

Inter-band SSB-Less operation

Neighbour cells on carrier of SSB-less SCell

The only open issue in inter-band SSB-less operation is regarding the intra-frequency and inter-frequency measurement definition for neighbor cell on carrier of SSB-less SCell [1].

Issue 1-1-4: Neighbour cells on carrier of SSB-less SCell FFS:  Intra-frequency/Inter-frequency definition: o Inter-frequency based on existing definition. No impact to spec about the intra-f and inter-f definition.  No spec impact in terms of inter- or intra- frequency, and with or without gap.

In legacy RAN2 spec, servingCellMO is one MeasObjectNR associated to the serving cell. Each serving cell can be configured with a servingCellMO and the ssbFrequency is the same with absoluteFrequencySSB [2].

ServingCellConfig ::=               SEQUENCE {

tdd-UL-DL-ConfigurationDedicated    TDD-UL-DL-ConfigDedicated                                                OPTIONAL,   -- Cond TDD

initialDownlinkBWP                  BWP-DownlinkDedicated                                                    OPTIONAL,   -- Need M

downlinkBWP-ToReleaseList           SEQUENCE (SIZE (1..maxNrofBWPs)) OF BWP-Id                               OPTIONAL,   -- Need N

…

servingCellMO                       MeasObjectId                                                            OPTIONAL,   -- Cond MeasObject

servingCellMO measObjectId of the MeasObjectNR in MeasConfig which is associated to the serving cell. For this MeasObjectNR, the following relationship applies between this MeasObjectNR and frequencyInfoDL in ServingCellConfigCommon/ServingCellConfigCommonSIB of the serving cell: if ssbFrequency is configured, its value is the same as the absoluteFrequencySSB and if csi-rs-ResourceConfigMobility is configured, the value of its subcarrierSpacing is present in one entry of the scs-SpecificCarrierList, csi-RS-CellListMobility includes an entry corresponding to the serving cell (with cellId equal to physCellId in ServingCellConfigCommon) and the frequency range indicated by the csi-rs-MeasurementBW of the entry in csi-RS-CellListMobility is included in the frequency range indicated by in the entry of the scs-SpecificCarrierList.

ssbFrequency is the frequency of the SSB associated to this MeasObjectNR. ssb-ConfigMobility is SSB configuration for mobility [2].

MeasObjectNR ::=                    SEQUENCE {

ssbFrequency                        ARFCN-ValueNR                                                   OPTIONAL,   -- Cond SSBorAssociatedSSB

ssbSubcarrierSpacing                SubcarrierSpacing                                               OPTIONAL,   -- Cond SSBorAssociatedSSB

smtc1                               SSB-MTC                                                         OPTIONAL,   -- Cond SSBorAssociatedSSB

smtc2                               SSB-MTC2                                                        OPTIONAL,   -- Cond IntraFreqConnected

refFreqCSI-RS                       ARFCN-ValueNR                                                   OPTIONAL,   -- Cond CSI-RS

referenceSignalConfig               ReferenceSignalConfig,

…

ReferenceSignalConfig::=            SEQUENCE {

ssb-ConfigMobility                  SSB-ConfigMobility                                              OPTIONAL,   -- Need M

csi-rs-ResourceConfigMobility       SetupRelease { CSI-RS-ResourceConfigMobility }                  OPTIONAL    -- Need M

}

ssbFrequency Indicates the frequency of the SS associated to this MeasObjectNR. For operation with shared spectrum channel access, this field is a k*30 kHz shift from the sync raster where k = 0,1,2, and so on if the reportType within the corresponding ReportConfigNR is set to reportCGI (see TS 38.211 [16], clause 7.4.3.1). Frequencies are considered to be on the sync raster if they are also identifiable with a GSCN value (see TS 38.101-1 [15], or TS 38.101-5 [75]).

As per existing RAN4 spec, the SSB based intra-frequency measurement definition is the center frequency of servingCellMO comparing to the centre frequency of the SSB of the neighbour cell [3].

A measurement is defined as a SSB based intra-frequency measurement provided the centre frequency of the SSB of the serving cell indicated for measurement and the centre frequency of the SSB of the neighbour cell are the same, and the subcarrier spacing of the two SSBs are also the same. <omit not related text> A measurement is defined as an SSB based inter-frequency measurement provided it is not defined as an intra-frequency measurement according to clause 9.2.

In R18 inter-band SSB-less SCell WI, since the absoluteFrequencySSB is absent for an SSB-less SCell, SSB based measurements cannot be performed on this SSB-less SCell. However for the neighbor cell on the carrier of SSB-less SCell, as there is SSB on this neighbor cell, SSB based measurement can be performed on neighbor cells. whether neighbor cell measurements on the carrier of SSB-less SCell is considered as intra-frequency or inter-frequency measurements depends on RAN2 signalling structure. In last meeting RAN2 agreed that servingCellMO is not configured for inter-band SSB-less SCell.

As per existing intra-frequency/inter-frequency measurement definition, as no servingCellMO is indicated, the SSB measurement on neighbor cell on the SSB-less carrier can’t be regarded as “intra-frequency measurement”, then go to “otherwise” branch, it is regarded as “inter-frequency measurement”. As the definition follows the legacy rule of intra-f/inter-f measurement definition, no specification impact is needed.

Proposal 1: Measurement on neighbour cells on carrier of SSB-less SCell is regarded as“inter-frequency measurement”. There is no spec impact about the intra-f and inter-f definition.

As per existing inter-frequency measurement requirements, if UE supports interFrequencyMeas-Nogap-r16, and the  target SSB on neighbor cell is completely contained in the active BWP, no measurement gap is needed. Otherwise measurement gap is needed.

A measurement is defined as an inter-frequency SSB based measurements without measurement gaps (either legacy measurement gap or NCSG) in active BWP, for UE capable of interFrequencyMeas-NoGap provided that - the UE supports interFrequencyMeas-Nogap-r16 [15], and - the SSB is completely contained in the active BWP of the UE. - For inter-frequency SSB based measurements without measurement gaps, UE may cause scheduling restriction as specified in clause 9.3.9.3. - Note: Non-CA capable UE is not expected to indicate support of interFrequencyMeas-Nogap-r16 [15].

Therefore the legacy principle on the need of gap can be applied for SSB-less SCell operation.

Proposal 2: Measurement on neighbour cells on carrier of SSB-less SCell with or without gap can follow the legacy principle and there is no spec impact.

Conclusions

This contribution provides analysis on maintenance for network energy saving. According to the analysis, we propose the following proposals:

Proposal 1: Measurement on neighbour cells on carrier of SSB-less SCell is regarded as“inter-frequency measurement”. There is no spec impact about the intra-f and inter-f definition.

Proposal 2: Measurement on neighbour cells on carrier of SSB-less SCell with or without gap can follow the legacy principle and there is no spec impact.

References

- R4-2414102, WF on RRM requirements for NR network energy saving, Huawei

- TS 38.331

- TS 38.133