3GPP TSG-RAN WG4 Meeting # 113			                                          R4-2418677

Orlando, Florida, USA, 18 – 22 November, 2024

Title: 					Discussion on FR2-1 L3 measurement delay by optimising CSSF outside gap in CA/DC

Source: 	Huawei, HiSilicon

Agenda item:	7.15.2.2

Document for:	Discussion

Introduction

The RRM Phase 5 FR2-1 L3 measurement delay by optimising CSSF outside gap in CA/DC discussion, as outlined in WF[1], was one of the topics during RAN4 #112bis. Key contributions from various companies are detailed in WF[1]. In this paper, we present our perspectives and contributions regarding FR2-1 L3 measurement delay, focusing on optimising the CSSF outside gap in CA/DC scenarios.

Discussion

UE measurement procedure

There was an agreement in the WF[1] captured in the following. This conclusion provided three aspects, each of these aspects is one vital component of the CSSF outside gap spec.

Agreement in online session: - The following aspects in CA/DC to use L3 measurement delay reduction by optimizing CSSF shall be discussed, and further prioritization among the 3 aspects can be discussed in future meeting: - Aspect 1: SSB based Intra-frequency measurement without MG, including: - TPSS/SSS_sync_intra and TSSB_measurement_period_intra - CSSFintra for intra-frequency measurement without gap which is defined since Rel-15 - Aspect 2: SSB based Inter-frequency measurement without MG, including: - TPSS/SSS_sync_inter, TSSB_time_index_inter and TSSB_measurement_period_inter - CSSFinter for inter-frequency measurement without gap. - Aspect 3: Inter-RAT SSB measurement without MG, including: - CSSFinterRAT for inter-RAT measurement without gap if the UE indicates ‘nogap-noncsg’ via NeedForGapNCSG-InfoEUTRA for the inter-RAT measurement. - MG related features to be considered in aspect 1/2/3 including: - R16 Inter-frequency measurement without gap where SSB is completely contained in active BWP - R17 NCSG measurement with ‘nogap-noncsg’ - R18 NeedForGaps measurement with ‘no-gap-no-interruption’ or with “no-gap-with-interruption”

For the enhancement of CSSF, all the aforementioned aspects are integral and equally crucial components. There are two points for this is detailed as follows:

- Prioritising any single part would necessitate the creation of individual specification content, which could complicate future maintenance and expansion.

- Given that the solution has not yet been fully discussed or finalised, isolating and prioritising one aspect may not align with potential solutions that could arise.

Proposal 1: The optimisation of CSSF for L3 measurement delay reduction in CA/DC should take into account all three aspects.

Candidate Solutions

In this section, the approach for utilising the CSSF outside the gap will be presented from two perspectives: the 2-searcher solution and the 3-searcher solution.

2-searcher solution

The following content has been extracted from the last meeting’s WF[1].

Candidate options for further discussion: - Option 1: UE can measure one serving CC per band if multiple serving CCs are in the same band. For the CCs to be measurement, further discuss option 1A and option 1B - Option 1A: Reuse the legacy principle for neighboring cell measurement. - Option 1B: Based on network indication - The other CCs (not measured based on network indication) need not to be measured and reported. Network assume the same measurement results on the measured CCs and unmeasured CCs in the same band. - Option 2: UE can change the searcher occupancy ratio of measurement among the measured CCs - Option 2A: UE can change the searcher occupancy ratio of PCC or PSCC measurement to speed up SCC measurement for some conditions

In Option 1A, the principle of neighboring cell measurement enables the UE to measure one serving CC per band. However, this does not align with the measurement behaviour intended by the NW. Because, if NW configures an SCC on another serving cell as a MO and servingCellMO, it means that NW is willing to know the measurement results of the indicated frequency layer.

Observation 1: RAN4 need to further discuss how to align with NW configuration and corresponding UE behaviour.

In our view, NW determines which CC need to configure MO is aligned with the legacy, allowing the NW to flexibly configure the MO quantity. For instance, if servingCellMO is assigned a value as an MO index and servingCellMO is indicated for an SCell, the SCC should be first configured as an MO. If the target is planning not to perform on other serving cells in the same band, NW can directly not configure MO associated with the SCC.

Additionally, the NW can decide whether CCs within the same band share measurement results, based on its implementation. If NW wants to know the information of serving CCs, it is able to configure servingCellMO on one CC and no MO configured on the other CCs in the same band. Then other serving CCs’ measurement results can be shared with the serving cell with MO.

For Option 1B, we would like to know the indication’s details and what new behaviour UE needs.

Proposal 2: Further discussion is needed to justify the introduction of a new NW indication for Option 1B.

For Option 2, we propose a method to optimise the CSSF outside the gap by having the PCell share its searcher with only one SCell. Essentially, this approach enables the searcher occupied by the PCell to handle a portion of the Scell MOs to reduce the measurement delay for the SCell slightly.

Allowing the PCell to share its searcher with one SCell MO may result in doubled measurement delay. However, under certain conditions—such as when PCell service quality is stable or the UE is in a low-mobility state—this may not cause significant issues.

Proposal 3: To facilitate the reduction of FR2-1 L3 measurement delay, we propose that the PCell share its occupied searcher with one SCell MO. The specific conditions under which this solution can be implemented can be FFS.

3-searcher solution

Secondly, we will address the solution of 3-searcher assumption and its performance.

In the last meeting, the following online agreement is captured in WF[1]:

Agreement in online session: On top of the previous agreement on the baseline assumption of 2 searchers, for 3 searchers: - Discuss the applicable scenario(s) and/or solution(s) based on 3 searchers. Decide whether or not to define the requirement later.

Based on this agreement, we would like to propose a clear and feasible 3-searcher solution.

The following table is an example of the solution. We used the CSSF spec in version Rel-15 to discuss and explain the improvement.

Proposal 4: To facilitate the discussion of this issue, we recommend utilising the Rel-15 version of the CSSF as a baseline for solution discussion. Once the solution is settled, we can then explore the incorporation of the features introduced in Rel-16, Rel-17 and Rel-18 into CSSF table.

Table 3. Example of CSSFoutside_gap,i scaling factor for SA mode 3-searcher solution

| Scenario | CSSFoutside_gap,i for FR1 PCC | CSSFoutside_gap,i for FR1 SCC | CSSFoutside_gap,i for FR2 PCC | CSSFoutside_gap,i for FR2 SCC where neighbour cell measurement is required | CSSFoutside_gap,i for FR2 SCC where neighbour cell measurement is not required |
| --- | --- | --- | --- | --- | --- |
| FR1 +FR2 CA (FR1 PCell) | 1 | Number of configured SCell(s)-1 | N/A | 1 | Number of configured SCell(s)-1 |
| FR1 +FR2 CA (FR2 PCell) | N/A | Number of configured SCell(s) / 2 | 1 | N/A | Number of configured SCell(s) / 2 Note 1 |
| Note 1: When the number of configured SCells is odd and larger than 1 this value is equal to (Number of configured SCells - 1) / 2 + 1. If only one SCell is configured this value is equal to 1. |  |  |  |  |  |

This example illustrates the scenario involving combined CA with FR1 and FR2. In this case, when all MOs are outside the gap, the measurement delay for the FR2 SCell is reduced by 50%. The PCell utilises one searcher, while the remaining searcher and the third searcher are shared between the FR1 and FR2 SCells.

For the 3-searcher scenario, the condition is when all MOs are situated outside the gap and CA is inter-band CA in FR1 + FR2. In this instance, the third searcher can be activated, as previously discussed.

Proposal 5: The 3-searcher solution can be utilised under the following conditions

- When all MOs need to be measured outside the gap.

- Only for the FR1 +FR2 CA scenario.

- Any other conditions can be FFS.

Feature capability

Since the solutions are still under discussion, the new capability will need to be defined at a later stage.

Proposal 6: Discussion on the new UE capability is supposed to be addressed later.

Conclusions

Proposal 1: The optimisation of CSSF for L3 measurement delay reduction in CA/DC should take into account all three aspects.

Observation 1: RAN4 need to further discuss how to align with NW configuration and corresponding UE behaviour.

Proposal 2: Further discussion is needed to justify the introduction of a new NW indication for Option 1B.

Proposal 3: To facilitate the reduction of FR2-1 L3 measurement delay, we propose that the PCell share its occupied searcher with one SCell MO. The specific conditions under which this solution can be implemented can be FFS.

Proposal 4: To facilitate the discussion of this issue, we recommend utilising the Rel-15 version of the CSSF as a baseline for solution discussion. Once the solution is settled, we can then explore the incorporation of the features introduced in Rel-16, Rel-17 and Rel-18 into CSSF table.

Proposal 5: The 3-searcher solution can be utilised under the following conditions

- When all MOs need to be measured outside the gap.

- Only for the FR1 +FR2 CA scenario.

- Any other conditions can be FFS.

Proposal 6: Discussion on the new UE capability is supposed to be addressed later.

References

- R4-2416853 WF on RRM requirements for NR_RRM_Ph5_Part1