3GPP TSG-RAN WG4 #113	R4-2418679

Orlando, Florida, USA, 18 – 22 November, 2024

Source: 	Huawei, HiSilicon

Title:  	Discussions on RRM requirements for Rel-19 ATG

Agenda Item:	7.10.4

Document for:	Discussion

Introduction

In the last meeting, we started the discussion of the Rel-19 ATG CA enhancement. The points and views of companies are detailed in the WF [1]. This paper presents our observations and views on RRM impacts on ATG CA.

RRM core requirement analysis

Compared with the requirements defined in Rel-18 ATG for single-Cell, we provide our analysis on the potential RRM impacts to support CA in case by case manner in the following part.

UE antenna type for inter-band CA

In the last meeting, we have an online understanding type, which is shown below:

Further check which understanding is correct: - Understanding #1: Case 4 is excluded in RF session. - Understanding #2: Case 4 is not explicitly discussed in RF session, since no case 4 specific RF requirement. Case 4 UE implementation is possible.

In our understanding, this type of UE antenna type is not discussed in the current band combination in WID, so all the RF discussions on the n3 and n39 combination inter-band CA assume that the antenna of UE is omnidirectional. If the RF can discuss case 4 further, RRM can keep discussing the requirements of case 4.

Proposal 1: Wait for further RF conclusion/clarification on antenna type for inter-band CA.

MRTD

The reuse of legacy intra-band MRTD requirements represents a prudent choice for ATG CA. For inter-band CA, UE has separate RF chains. We see no implementation issue to maintain the same MRTD value for inter-band CA, which is 33 us. This requirement supports that UE has separate antennas on each band. Whether to define two MTRD requirements or redefine the MRTD requirement is up to the discussion conclusion of the UE antenna type.

Proposal 2: Reused the legacy MRTD requirement 33 us for ATG inter-band CA separate antenna UE.

Proposal 3: The MRTD requirement for sharing panel UE needs to be FFS.

Interruption

For UE supporting CA, it is straightforward that there could be an interruption in one Cell caused by certain procedures from the other Cells. Based on the current interruption requirements in chapter 8.2 of 38.133, it is evident that there is no distinction made between different scenarios or UE types. Following the same logic, we think current interruption requirements can also apply to ATG-supporting CA without the need for any additional specifications. If there are ATG new procedures that are not covered in current interruption requirements, RAN4 could discuss whether to introduce new interruption requirements.

Proposal 4: Existing interruption requirements can also apply to ATG supporting CA. RAN4 could discuss whether to introduce new interruption requirements if there are ATG new procedures that are not covered in current interruption requirements.

SCell Activation and Deactivation Delay

In the last meeting, we already discussed the SCell activation well. The agreement is shown below:

Agreement: - SCell Activation and Deactivation Delay requirements should be introduced for ATG, reuse the current FR1 requirements as the starting point - SCell Activation Delay Requirement for Deactivated SCell - SCell Deactivation Delay Requirement for Activated SCell - Direct SCell Activation at SCell addition - Direct SCell Activation at Handover - Direct SCell Activation at RRC Resume - Fast SCell Activation Delay Requirement for Deactivated SCell - SCell Activation Delay Requirement for Deactivated SCell with the L3 reporting during activation - SSB-less SCell related requirement for intra-band CA and inter-band CA - Note: The appplicability of the requirement is up to UE capability supporting - Further discuss whether enhancement/update is needed considering band combinations, antenna types, and co-located deployment.

We have some concerns regarding the SSB-less requirements related to SCells. The below text is the Rel-18 38.133[2] SSB-less requirement definition. The highlighted section indicates that only type-A and type-C QCL relationships are applicable under the SSB-less SCell activation conditions. This indicated that the legacy requirement can not provide the beam-related information QCL.

For a UE supporting scellWithoutSSB-InterBandCA-r18 if the SCell being activated belongs to FR1 and if the UE is not provided with SSB configuration (absoluteFrequencySSB) in the target SCell (FrequencyInfoDL) nor SMTC configuration for the target SCell, and if there is one collocated active reference serving cell on different FR1 band, when the following conditions are fulfilled, - The RTD between the target SCell and the collocated reference serving cell is within CP where CP is corresponding to the SCS of SSB-less SCell, and - The EPRE difference at the UE is smaller than or equal to [30] dB, where EPRE difference is the power difference between TRS/A-TRS symbol on the SSB-less SCell and SSB symbol on the reference serving cell normalized by SCSs of SSB of reference Cell and A-TRS/P-TRS of SSB-less Cell., and - The RS(s) of the SSB-less SCell being activated is (are) QCL-TypeA with TRS(s) of the SSB-less SCell being activated, and the TRS(s) of the SSB-less SCell being activated is (are) further QCL-TypeC with SSB(s) of an inter-band active serving cell, and the inter-band active serving cell shall be same as the reference serving cell.

Given that the ATG UE might employ an array-type antenna in FR1, and the legacy SSB-less requirement does not address beam-related issues. Consequently, we believe that the SSB-less requirement is only applicable to omni-directional antenna ATG UEs.

Proposal 5: Inter-band SSB-less SCell activation requirements can apply to omnidirectional antenna type.

Deactivated SCell measurement

The following is the conclusion in the WF[1]:

Agreement: - Define intra-frequency measurement requirements for deactivated SCell by reusing the principle from the existing cell identification and measurement requirements with measCycleScell as baseline - Further discuss whether beam sweeping factor N1 and L3/L1 measurement sharing factor Klayer1_measurement is needed or not

There are 5 types of UE antenna in ATG Rel-19 discussion. Due to variations in UE types, including omni-directional and array antennas, the factor N1 should be considered. For instance, for deactivated SCell measurement UE also needs to measure neighbor cells on the same frequency layer, which may have different locations than the serving Cell, thus beam sweeping is needed.

Proposal 6: The beam sweeping factor N1 is needed in ATG SCell deactivation measurement.

The deactivated state does not have the L1 measurement, so we think the L3/L1 measurement sharing factor is not needed.

Proposal 7: For the L3/L1 measurement sharing factor Klayer1_measuremen is not needed.

Conclusion

Proposal 1: Wait for further RF conclusion/clarification on antenna type for inter-band CA.

Proposal 2: Reused the legacy MRTD requirement 33 us for ATG inter-band CA separate antenna UE.

Proposal 3: The MRTD requirement for sharing panel UE needs to be FFS.

Proposal 4: Existing interruption requirements can also apply to ATG supporting CA. RAN4 could discuss whether to introduce new interruption requirements if there are ATG new procedures that are not covered in current interruption requirements.

Proposal 5: Inter-band SSB-less SCell activation requirements can apply to omnidirectional antenna type.

Proposal 6: The beam sweeping factor N1 is needed in ATG SCell deactivation measurement.

Proposal 7: For the L3/L1 measurement sharing factor Klayer1_measuremen is not needed.

References

[1] R4- 2416852 WF on RRM requirements for [112bis][208] NR_ATG_enh

[2] 3GPP TS 38.133 V18.7.0