3GPP TSG-RAN4 Meeting # 113	R4-2418691

Orlando, USA, Nov.18th – 22th, 2024

# Source: 	ZTE Corporation, Sanechips

# Title: 	Discussion on R19 HPUE duty cycle and MSD rule

# Agenda item:	7.1.1.1.1

# Document for:	Approval

# 1 Introduction

In last meeting, some agreements/open issues are included in the WF[3], in which the SAR solution and MSD rule are still open. In this contribution, we give some further discussions on SAR solution and MSD rule for PC1.5 intra-band contiguous and non-contiguous UL CA, and 2Tx/3Tx inter-band UL CA.

# 2	Discussion

- 2.1 Duty Cycle solution

- For the duty cycle solution for SAR compliance, the agreements in the past meetings were captured in the WFs[1][2][3]. In the latest WF[3], it is:

Agreement: - For intra-band CA and inter-band CA PC1.5, duty cycle solution for SAR will be discussed. - For EN-DC PC1.5 and PC2, duty cycle solution for SAR will discussed for the scenario without new IE introduction.

PC1.5 intra-band CA

Although some companies mentioned that the duty cycle solution for SAR compliance is never used in the filed, we think it is still needed to be considered for PC1.5 intra-band CA from the completeness of specification due to duty cycle solution is already defined for PC2 intra-band CA in the spec. Otherwise, it may cause specification improvements or maintenance problem in the future.

According to the following general descriptions for intra-band contiguous and non-contiguous CA in sub-clause 4.2:

d)	All the requirements for intra-band contiguous and non-contiguous CA apply under the assumption of the same slot format indicated by UL-DL-configuration-common in the PCell and SCells for NR SA.

The above text are described in power class agnostic manner, so it is nature to be applicable for PC1.5 intra-band contiguous and non-contiguous CA. Due to the component carries in the UL intra-band CA are synchronized, and similar with PC2 intra-band UL CA, it is feasible for PC1.5 TDD UL intra-band CA to reuse the capabilities for single carrier case (i.e. maxUplinkDutyCycle-PC2-FR1, maxUplinkDutyCycle-PC1dot5-MPE-FR1).

- Proposal 1: For duty cycle based SAR solution, reuse the PC1.5 single CC capabilities (i.e. maxUplinkDutyCycle-PC2-FR1, maxUplinkDutyCycle-PC1dot5-MPE-FR1) for PC1.5 TDD UL intra-band contiguous and non-contiguous CA.

2Tx/3Tx PC2/PC1.5 inter-band CA

- Currently, there are no example band combinations in this WID, and there were no discussions on how to select the example band combinations. So it seems the band combination scenarios in this WID should totally depend on the requested band combinations in the corresponding HPUE basket WID.

- For the duty cycle based SAR solution, we think it should be based on the scenarios. It would be foreseen that the duty cycle scheme is different for the band combination between PC2 FDD+PC2/1.5 TDD and PC2 TDD+PC2/1.5 TDD, since there is no duty cycle scheme introduced for PC2 FDD single band, instead P-MPR scheme is used to meet the SAR requirements. Therefore, we think there is no need to define duty cycle scheme for the 2Tx PC1.5 NR inter-band UL CA including PC2 FDD band since it would not proper to assume the 100% duty cycle for PC2 FDD band, unlike PC3 FDD band. In the other words, duty-cycle solution should be considered for limited scenarios.

Proposal 2: Only consider P-MPR scheme for the HPUE 2Tx/3Tx band combination including PC2 FDD band, i.e. FDD+FDD, FDD+TDD

For other configurations, except P-MPR, duty cycle scheme should be applied.

For 2Tx PC1.5 inter-band UL CA, it seems it can only be achieved via PC2 TDD+ PC2 TDD, and PC2 FDD+ PC2 TDD. As we propose above, we should only consider the duty cycle for PC2 TDD+ PC2 TDD, and the duty cycle approach have already been included for the existing configurations in the specification.

Proposal 3. For 2Tx PC1.5 inter-band UL CA existing configurations, the existing duty cycle approach can be reused, and need to be revised to apply for 3UL CC in both UL bands if needed.

For 3Tx PC1.5 inter-band UL CA, the duty cycle approach have already been included for the following existing configurations:

1: 3Tx PC2 = FDD(PC3)+TDD(PC3/PC2) with single carrier in each UL band

2: 3Tx PC2 = TDD(PC2)+TDD(PC2) with single carrier in each UL band

3: 3Tx PC1.5 = FDD/TDD(PC3) +TDD(PC1.5) with single carrier in each UL band

Proposal 4. For 3Tx PC1.5 inter-band UL CA existing configurations, the existing duty cycle approach can be reused, and need to be revised to apply for 3UL CC case if needed.

For 3Tx PC1.5 inter-band UL CA new configuration, such as 3Tx PC1.5 = TDD(PC2) +TDD(PC1.5), new duty cycle approach should be defined, including single carrier in each UL band and 3UL CC in both UL bands.

Proposal 5. For 3Tx PC1.5 inter-band UL CA new configurations of TDD(PC2) +TDD(PC1.5), new duty cycle approach should be defined, including single carrier in each UL band and 3UL CC in both UL bands

- 2.2 MSD rule

- In terms of the previous discussion, it can be seen that companies have strong will to simplify the existing MSD table for HPUE band combination considering the huge work for HPUE artificial MSD calculation based on the worse case or different assumptions which cause high MSD value. In general, the high MSD value derived from the worse case is hardly happened in the practical.

- Observation: Companies have strong will to simplify the existing MSD table for HPUE band combination.

- In last RAN4 meeting, there are several options listed in the WF and there are no conclusions:

Option 1: - RAN4 agrees on the following directions in this meeting and continues on details in subsequent meetings. - RAN 4 defines the default power class inter-band CA/DC and intra-band CA/DC MSD requirements, and RAN4 specifies equations/simplified guidelines/lookup tables “LUT” that define the relationship between the default power class MSD and the HPUE MSD requirements to replace the existing HPUE MSD tables: - Equations/LUT/Simplified guidelines identifying the HPUE MSD requirements are expected to be captured in RAN4 spec as normative requirements, in Rel-19 if possible, the details are FFS - FFS the expected spec changes and procedure changes (i.e., continue/stop providing TP’s for each combination, remove/keep all existing HPUE notes/ HPUE MSD tables etc) are recommended to be applicable from Rel-20, or the late stage of Rel-19 - Decision can be deferred until the Equations/LUT/Simplified guidelines are getting done - Inform RAN5 after all the discussion are finalized, and it is up to RAN5 to decide how to simplify HPUE CA/DC MSD conformance testing, [and RAN4 can make recommendation if any consensus] - [Agree to capture just single worst case MSD in TS for HPUE PC which is agnostic to number of TX, for each kind of MSD] NOTE 1: RAN4 continues normal procedure specifying MSD for HPUE band combination supports at least until down-selection is done. NOTE 2: RAN4 discuss/consider all HPUE scenarios/configurations as a package for sake of consistence. For the scenarios out of this WI scope, the agreements (if any) are recommended for band combination HPUE basket WIs to follow. Option 2: - Current working procedure of HPUE CA/DC introduction are kept unchanged - HPUE MSD tables are kept - Unified Excel tool of calculating HPUE MSD are being developed with following details to be addressed - Companies to evaluate if the interferer level for PC2/PC1.5 can be scaled by less than current 100% to achieve more meaningful PC2/PC1.5 IMD4 and IMD5. - [Agree to capture just single worst case MSD value in TS for each PC which is agnostic to number of TX]. - Agree to use the following rules in deriving PC2/PC1.5 MSD for cases where PC3 MSD is 0dB: - If one of the rules above is met, PC2 1TX MSD=0.4dB, PC2 2TX MSD=0.5dB, PC1.5 2TX MSD=0.8dB. - If both rules above are met, PC2 1TX MSD=0.8dB, PC2 2TX MSD=0.9dB, PC1.5 2TX MSD=1.5dB. - In case it is discovered that the PC3 MSD is omitted or is illogical compared to respective similar band combinations, then naturally the PC3 MSD can be further evaluated before PC2/PC1.5 assessment. - Agree that the assumed PRX/DRX difference is 6dB, and additional noise into PRX is 1dB due to 2 TX case for PC2/PC1.5 NR CA and EN-DC. - The tool is published in 3GPP in Excel format if all proposals are agreeable. NOTE: Freedom for any company to provide PC3 MSD evaluations using companies’ calculation methods and other assumptions fully remains because PC3 is the baseline to derive PC2/PC1.5 MSD. It is not precluded to provide evaluations for PC2/PC1.5. Option 3: - PC2/PC1.5 MSD requirements are not specified. UE only need to be verified under the Rx requirements of default power class, with default power class transmission - Add the Equations/LUT/Simplified guidelines into the Annex of TS to derive the HPUE MSD for information, in Rel-19 if possible, the details are FFS - FFS the expected spec changes and procedure changes (i.e., continue/stop providing TP’s for each combination, remove/keep all existing HPUE notes/ HPUE MSD tables etc) are recommended to be applicable from Rel-20, or the late stage of Rel-19 -Decision can be deferred until the Equations/LUT/Simplified guidelines are getting done - Inform RAN5 after all the discussion are finalized, and it is up to RAN5 to decide how to simplify HPUE CA/DC MSD conformance testing, [and RAN4 can make recommendation if any consensus] NOTE 1: RAN4 continues normal procedure specifying MSD for HPUE band combination supports at least until down-selection is done. NOTE 2: RAN4 discuss/consider all HPUE scenarios/configurations as a package for sake of consistence. For the scenarios out of this WI scope, the agreements (if any) are recommended for band combination HPUE basket WIs to follow. Option 4: - PC2/PC1.5 MSD requirements are not specified, MSD for PC2 and PC1.5 are only specified in TR or Annes of TS which is not mandate, UE shall meet the Rx requirements of default power class, with default power class transmission Agreement: - To clarify the Equations/LUT/Simplified guidelines mentioned in Option 1 and Option 3 - Some examples are provided in the Annex of this WF - Other options such as Option2 are not precluded

- First, we think it is feasible to adopt some equations or look-up tables to derive PC2/PC1.5 MSD from respective PC3 MSD;

- Second, we think the HPUE MSD table should be kept in R19 specification, and no changes for the existing HPUE basket WID procedure. FFS on removing or keeping it in R20 after RAN4 has the agreements on the MSD rule in R19.

- Third, we think it does not make sense to only capture the MSD value for HPUE combs in the TR. If the MSD value is not reflected in the TS in the end, then the MSD value should not be studied in the TP to TR. In this case, no need for the TR at all.

- Last, we think the key point for the MSD rule is how to simplify the HPUE MSD work with the so-called Equations/LUT/Simplified guidelines. For the time being RAN4 should focus on what the Equations/LUT/Simplified guidelines look like, instead how to capture it in the TS.

- Considering the above, we think option 2 can be used as starting point.

- Proposal 6: to use option 2 as staring point.

- Moreover, the MSD rule should be applied to all 2Tx/3Tx HPUE band combinations.

- For 3Tx HPUE band combination, PC3 3Tx band combinations were supported by default. But there were no discussions how to derive PC3 3Tx MSD, instead the MSD requirements for HPUE 3Tx band combination were reused from the corresponding HPUE 2Tx band combination, we think this rule shall also be applied for all the applicable 3Tx HPUE band combination in R19.

- Proposal 7: For 3Tx PC2/PC1.5 inter-band UL CA, the MSD requirements are reused from the corresponding HPUE 2Tx band combination.

- Once the MSD rule for all HPUE band combination, more discussion should be needed for how to treat the parallel work in the corresponding basket WID, and which release should be reflected for the existing MSD requirements with the new MSD rule.

- Proposal 8: How/When to reflect the new agreed MSD for the existing MSD requirements are FFS.

# 3 Conclusion

In this contribution, we give further discussions on PC1.5 Intra-band contiguous and non-contiguous UL CA. The proposals and conclusions are:

duty cycle:

- Proposal 1: For duty cycle based SAR solution, reuse the PC1.5 single CC capabilities (i.e. maxUplinkDutyCycle-PC2-FR1, maxUplinkDutyCycle-PC1dot5-MPE-FR1) for PC1.5 TDD UL intra-band contiguous and non-contiguous CA.

Proposal 2: Only consider P-MPR scheme for the HPUE 2Tx/3Tx band combination including PC2 FDD band, i.e. FDD+FDD, FDD+TDD

Proposal 3. For 2Tx PC1.5 inter-band UL CA existing configurations, the existing duty cycle approach can be reused, and need to be revised to apply for 3UL CC in both UL bands if needed.

Proposal 4. For 3Tx PC1.5 inter-band UL CA existing configurations, the existing duty cycle approach can be reused, and need to be revised to apply for 3UL CC case if needed.

Proposal 5. For 3Tx PC1.5 inter-band UL CA new configurations of TDD(PC2) +TDD(PC1.5), new duty cycle approach should be defined, including single carrier in each UL band and 3UL CC in both UL bands

- MSD rule:

- Observation: Companies have strong will to simplify the existing MSD table for HPUE band combination.

- Proposal 6: To use option 2 as staring point.

- Proposal 7: For 3Tx PC2/PC1.5 inter-band UL CA, the MSD requirements are reused from the corresponding HPUE 2Tx band combination.

- Proposal 8: How/When to reflect the new agreed MSD for the existing MSD requirements are FFS.

# 4	Reference

[1] R4-2410750, WF on high power UE, Samsung

[2] R4-2414277, WF on HPUE for CA in TN, Samsung

[3]R4-2417098, WF on HPUE RF requirements, Samsung

[4] R4-2415904 Discussion on R19 HPUE duty cycle and MSD rule, ZTE Corporation, Sanechips