3GPP TSG-RAN4 Meeting # 113	R4-2418693

Orlando, USA, Nov.18th – 22th, 2024

# Title: 	On SRS antenna switching requirements

# Source: 	ZTE Corporation, Sanechips

# Agenda item:	7.1.1.3.3

# Document for:	Approval

# 1 Introduction

In last meeting, some agreements are included in the approved WF[1], where the ΔTRxSRS requirements for 2T6R SRS antenna switching configurations of t1r6, t2r6 and t1r6-t2r6 are approved.

In addition, two companies provided the ΔTRxSRS requirements for 3T6R SRS antenna switching configurations. Due to Oct. Meeting is the first RAN4 meeting to discuss the 3T6R SRS switching cases, consequently, moderator postpone the discussion and come back to Nov. Meeting.

Agreement: - As this is the first RAN4 meeting since the confirmation that RAN1 will consider t3r6, companies are encouraged to provide ∆TRxSRS values for t3r6, t1r6-t3r6, t2r6-t3r6, and t1r6-t2r6-t3r6 at RAN4#113 for further discussion with the Apple and Qualcomm proposals.

In this contribution, we give some initial discussions on the ΔTRxSRS requirements for the 3T6R SRS antenna switching.

# 2	Discussion

For 2T6R ΔTRxSRS requirements, the agreements in the WF [1] are:

Agreement: RAN4 agrees to consider additional breakpoint for bands whose FUL_high is higher than the FUL_low of n104. Agreement: RAN4 agrees to adopt the values for Bands n79 and n104 as specified below. Table: ΔTRxSRS values Operating Bands; ΔTRxSRS t1r6 (dB); ΔTRxSRS t2r6 (dB); ΔTRxSRS t1r6-t2r6 (dB). Band n79; 5.5; 5.0; 6.0. Band n104; 6.5; 5.5; 7.0.

The above values are for PC3 or PC2 in the band and ΔPPowerClass = 3 dB, or when UE indicating Tx diversity capability.

Foe PC2 and ΔPPowerClass = 0 dB, and UE not indicating Tx diversity capability, additional 3dB shall be considered. This is also aligned with the following agreements:

Issue 2-1-2: Whether to consider separate ∆TRxSRS values when the device is power class 2 in the band and ΔPPowerClass = 0dB and not indicating Tx diversity capability Agreement: Have additional 3dB for power class 2 when ΔPPowerClass = 0dB and not indicating Tx diversity capability.

The above agreement can also be applied to 3T6R ΔTRxSRS requirements, and also 1dB delta value between n104 and n79 is adopted.

For 3T6R, in terms of the agreements, the ΔTRxSRS requirements should be defined for the SRS antenna switch scenarios t3r6, t1r6-t3r6, t2r6-t3r6, and t1r6-t2r6-t3r6 for the bands n41, n77, n78, n79 and n104.

For the ∆TRxSRS values, different RFFE architecture would cause different values since the IL depends on which types of and how many switchers (such as SPDT, DPDT, SP4T, and etc) would be used.

1. t3r6

For t3r6 SRS antenna switching, the example RFRE architectures could be:

Figure 1. Example RFFE architectures for t3r6 (Main. Ant #1/#3/#5, Div. Ant #2/#4/#6)

We think the situation is the same for t4r8 and t2r4, Therefore, it is propose to use the current ∆TRxSRS requirements of t2r4 or trt8 for t3r6, which are 3dB for n41/n77/n78 and 4.5dB for n79.

2. t1r6-t3r6

For t1r6-t3r6 SRS antenna switching, the example RFRE architectures could be:

Figure 2. Example RFFE architectures for t1r6-t3r6 (Main. Ant #1/#3/#5, Div. Ant #2/#4/#6)

The additional IL value can be calculated between Ant #1 and Ant #6, which is SP4T IL+DPDT IL +SPDT IL +PCB IL:

- For n41/n77/n78, it is 3.9dB (=0.8+0.8+0.5+1.8)

- For n79, it is 5.1dB (=1.1+1.0+0.6+2.4)

By considering additional implementation margin due to UE supports multiple band which will make REFE architecture more complicated in reality, ∆TRxSRS requirements of 4.5dB and 6.0dB are applied for bands n41/n77/n78 and band n79, respectively.

3. t2r6-t3r6

For t2r6-t3r6 SRS antenna switching, the example RFRE architectures could be:

Figure 3. Example RFFE architectures for t2r6-t3r6 (Main. Ant #1/#3/#5, Div. Ant #2/#4/#6)

The additional IL value can be calculated between Ant #1 and Ant #6, which is SP3T IL+SP4T IL +PCB IL:

- For n41/n77/n78, it is 3.3dB (=0.7+0.8+1.8)

- For n79, it is 4.5dB (=1.0+1.1+2.4)

By considering additional implementation margin due to UE supports multiple band which will make REFE architecture more complicated in reality, ∆TRxSRS requirements of 4.0dB and 5.5dB are applied for bands n41/n77/n78 and band n79, respectively.

4.t1r6-t2r6-t3r6

For t1r6-t2r6-t3r6 SRS antenna switching, the example RFRE architectures could be:

Figure 4. Example RFFE architectures for t1r6-t2r6-t3r6 (Main. Ant #1/#3/#5, Div. Ant #2/#4/#6)

The additional IL value can be calculated between Ant #1 and Ant #6, which is SP3T IL+DP3T IL + DP3T IL + SP3T IL +PCB IL:

- For n41/n77/n78, it is 5.0dB (=0.7+0.9+0.9+0.7+1.8)

- For n79, it is 6.8dB (=1.0+1.2+1.2+1.0+2.4)

By considering additional implementation margin due to UE supports multiple band which will make REFE architecture more complicated in reality, ∆TRxSRS requirements of 5.5dB and 7.5dB are applied for bands n41/n77/n78 and band n79, respectively.

Therefore, we have the following proposal:

Proposal: Use the following 3T6R numbers as a reference when the device is power class 3 or power class 5 or power class 1.5 in the band, or when the device is power class 2 in the band and ΔPPowerClass = 3 dB, or when UE indicating Tx diversity capability.

| Operating Bands | ΔTRxSRS t3r6 (dB) | ΔTRxSRS t1r6-t3r6 (dB) | ΔTRxSRS t2r6-t3r6 (dB) | ΔTRxSRS t1r6-t2r6-t3r6 (dB) |
| --- | --- | --- | --- | --- |
| Band n41, n77, n78 | 3 | 4.5 | 4.0 | 5.5 |
| Band n79 | 4.5 | 6 | 5.5 | 7.5 |
| Band n104 | 5.5 | 7 | 6.5 | 8.5 |

# 3 Conclusion

In this contribution, we give some initial discussion on the ∆TRxSRS requirements for 3T6R SRS antenna switching. The conclusions are:

Proposal: Use the following 3T6R numbers as a reference when the device is power class 3 or power class 5 or power class 1.5 in the band, or when the device is power class 2 in the band and ΔPPowerClass = 3 dB, or when UE indicating Tx diversity capability.

| Operating Bands | ΔTRxSRS t3r6 (dB) | ΔTRxSRS t1r6-t3r6 (dB) | ΔTRxSRS t2r6-t3r6 (dB) | ΔTRxSRS t1r6-t2r6-t3r6 (dB) |
| --- | --- | --- | --- | --- |
| Band n41, n77, n78 | 3 | 4.5 | 4.0 | 5.5 |
| Band n79 | 4.5 | 6 | 5.5 | 7.5 |
| Band n104 | 5.5 | 7 | 6.5 | 8.5 |

# 4	Reference

[1] R4-2417209, WF on 6Rx UE RF requirements, AT&T