3GPP TSG-RAN4 Meeting # 113	R4-2418695

# Orlando, USA, Nov.18th – 22th, 2024

# Title: 	View on Fragmented carrier RF requirements

# Source: 	ZTE Corporation, Sanechips

# Agenda item:	7.6.3

# Document for:	Approval

# 1 Introduction

In last meeting, some further discussions on the fragment carrier were carried out [1, 2], and there are several agreements are included in the WF[3]. Meanwhile, there still some open issues which was not discussed due to lack of time.

In this contribution, we give some views on some left open issue for fragment carriers.

# 2	Discussion

There are some general assumptions agreed in the WF[3], which are:

- Issue 3-1: Discussion on assumptions on test configurations for evaluation Agreement: (a) RAN4 agree on the general assumptions that are same for legacy NR RF performance evaluation: o UE self-band uplink Tx re-use same assumptions for NR PC3 PA i.e., ACLR=30dB, with MPR=1dB, Full RB allocation. Tx LO leakage and image rejection ratio are 28dB o Reuse the Rx chain image rejection, which is 25dB, as the starting point. Agreement: - The configuration defined in Table 7.3A.2.2-1 and Table 7.3A.2.2-2 of TS 38.101-1 could be re-used for performance evaluation, for ΔRIBNC, REFSENS, both out-of-gap and in-gap ACS, IBB and NBB requirements for selected example combos Agreement: - Priority to one uplink carrier Agreement: - Evaluation may consider impairment factors such as ACLR, phase noise or IMDx of aggressor ability, duplexer isolation…etc - The example bands should be in the scope of existing intra-band non-contiguous CA operating bands - No relaxation on any Tx RF requirements should be assumed in this study item - Both PC3 and PC2 should be considered - Prioritize PC3 - The minimum Wgap for ACS, narrow band blocking and in-band blocking verification should meet: - Wgap ≥ 2∙¦FInterferer (offset),j¦ – BWChannel(j) - It is the condition to apply the requirements - Both the symmetric CC scenario and the asymmetric CC scenario should be considered in the FC SI, and companies are encouraged, but not limited, to study the CC scenarios shared by the operators - [Adjusted requirements for fragmented carriers, if any, shall be separated from the existing requirements] - RAN4 shall not re-discuss the power imbalance assumption of the wanted carriers further in relation to the SI on fragmented carriers - Issue 3-2: Example band consideration for evaluation Agreement: - RAN4 doesn’t consider 3MHz channel bandwidth for fragmented carriers - The example band(s) should already have NC DLCA specified - Prioritize the frequency bands of ≤2700MHz - Remove n39 and include n26 in the example bands: Consider n2/n25, n3, n7, n66, n41, n39, n26 as the example band

In addition to the above agreements, a fully shared RF chain (one Rx RF chain) is the reference architecture for fragmented carriers, which is:

However, the study on [PSD] difference between carriers of co-located adjacent channel operators are still FFS.

- Issue 2-2: Study on power spectral density difference between carriers of co-located adjacent channel operators Agreement: To evaluate acceptable [PSD] difference from UE side between wanted signal and un-wanted signal based on the deployment where BSs from different operators are collocated but their radio units (RUs) are not shared. - The [PSD] difference means the difference between unwanted signal and the wanted carrier with the lower PSD - Start evaluation with wanted signal level currently existing ACS requirements as baseline and to see the performance difference

## 2.1 PSD different

The discussion on the [PSD] different can be simply illustrated in the Fig 1.

Fig 1. Simply illustration for the PSD different between carriers

In Fig 1., there are three PSD differences, where X and Y are the UE received PSD difference between carriers of co-located adjacent channel operators in inter-operator deployment scenario, while Z is the UE received PSD difference between the two non-contiguous CCs of the same operators, which is up to 6dB according to the objectives, i.e. Z<=6dB.

The discussion point is the only Y value since Y is the difference between unwanted signal and the wanted carrier with the lower PSD. It is clearly that the value of Y is larger than 6dB.

In the contribution [4], we observed that the power difference, PSD difference and the carrier channel bandwidths are dependent each other, since PSD difference = Power different - 10log(BWinterference/BWwanted).

In last meeting, it was confirmed that the [PSD] difference should be from UE side. In current ACS/IBB test, the BWinterferer =5MHz is adopted, we think this test configuration should be reused for fragment carriers in a single Rx RF chain. Then look at the existing intra-band DL NC CA case, the largest PCC channel bandwidth is 40MHz (for CA_n25(2A) where the aggregated channel bandwidth (PCC+SCC)= 40MHz +5MHz), which is the wanted carrier with the lower PSD (assume the power for 40MHz PCC and 20MHZ SCC are the same).

In our understanding, the UE received power could be calculated as: Ptx+ Gtx- loss +Grx, where Ptx is the gNB maximum transmission power, and Gtx and Grx are the antenna gain of BS Tx antenna gain and UE Rx antenna gain, respectively. Loss is mainly the path loss.

For co-located adjacent channel operators, our understanding is that the co-located gNBs in inter-operator deployment scenario means sharing the same site/tower for the co-location BS from the different operators. The geographic distance between the two co-located gNBs can be ignored, which means the path loss for different UE received carriers are the same. And usually omnidirectional antenna is used in UE side which means the Grx could be ignored. Then UE received power depends on Ptx+ Gtx. However, due to antenna pattern for different gNB may be different for different operators, and usually 3D antenna element pattern (dB) is used in gNB. In 3GPP, for example TR38.803, there are BS antenna element pattern defined for different scenarios. If only consider the same panel configurations for different BS antenna, then the delta received power from UE can be roughly derived from antenna element gain delta, similar as [7].

However, if consider different panel configurations for different BS antenna, then the delta received power from UE would be different, and also it may not practical to compare with 0 degree and 60 degree since the interference between the two gNBs should be considered.

- Proposal 1. The [PSD] difference should assume >6dB PSD imbalance, considering different panel configurations for different BS antenna, and different channel bandwidths of wanted carrier and interfering carrier.

## 2.2 RF requirements

In the WF, it shows:

- Issue 3-3: Observations of impacts on DL Rx requirements RAN4 discuss if following general aspects can be agreeable: - The largest interferer power in-gap shouldn’t larger than -25dBm for fragmented carriers using a single Rx RF chain - It is proposed to consider only same SCS for both CCs to ease receiver design for the common RF chain - ΔRIBNC for SCC may need to be updated to reflect the filter rejection reduction of common RF chain compared to separate RF chains for each intra-band non-contiguous CA band combination. Furthermore, ΔRIBNC needs to apply to PCC too, although the value of ΔRIBNC can be different for PCC and SCC - RAN4 shall not consider requirements for fragmented carriers which will result in performance degradation as compared to legacy NC IB CA or – only in case of a strong in-gap interferer – single CC operation

For the largest interfer power in-gap, maximum -25dBm is aligned with the existing intra-band non-contiguous CA ACS, where the UE shall fulfil the minimum requirements all values of a single adjacent channel interferer in-gap and out-of-gap up to a –25 dBm interferer power while all downlink carriers are active, where the -25dBm is the same as the maximum input level requirements. We didn’t see there are reasons to change it when the Rx chain is changed to fully shared RF chain for fragment carriers.

Proposal 2: The largest interferer power in-gap shouldn’t larger than -25dBm for fragmented carriers using a single Rx RF chain.

For the SCS, we think it related to whether to use a single FFT size which depends on the SCS for each CC. As we discussed in [5], it may be difficult and impractical to use a common FFT to handle different SCS due to the OFDM length/CP length in time domain, sampling rates per symbol are different for different SCS. But of course, a single FFT size is feasible when the SCS for different carriers/fragments are aligned but it is up to UE implementation. Therefore, we think there is no need to restrict the only same SCS for both CCs.

Due to it was already agreed that the configuration defined in Table 7.3A.2.2-1 and Table 7.3A.2.2-2 of TS 38.101-1 could be re-used for performance evaluation, and the existing configuration are for the same SCS for both carriers, so for ΔRIBNC re-evaluation, same SCS for both CC should be reused, and the results should be applied to the others SCS configurations for the both CC.

- Proposal 3: For ΔRIBNC re-evaluation, only the configuration of the same SCS for both CC is enough, and the results should be applied to the others SCS configurations for the both CC.

For the filter rejection reduction, it will need higher attenuation at the own Rx PCC/SCC range since the roll-off is changed (roughly by BWPCC/(BWPCC+Wgap+BWSCC)). Higher attenuation in near field means the sharper S21 curve and higher order of the filter implementation, also a larger Q value (The Q value can be calculated as f0/BW, where f0 is the centre frequency of band pass filter, and equals to (fh+ fl)/2,  and the BW is the 3-dB bandwidth, and equals to (fh- fl)). Consequently the filter size will become larger and the filter implementation would become more challenging.

As proposed in [6], interested companies should provide the feasibility and performance of wider low pass filter attenuation.

Proposal 4: The assumption for the feasible wider low pass filter attenuation should be discussed for ΔRIBNC re-evaluation.

# 3 Conclusion

In this contribution, we give some discussions on some left open issue related to RF requirements for fragment carriers

- Proposal 1. The [PSD] difference should assume >6dB PSD imbalance, considering different panel configurations for different BS antenna, and different channel bandwidths of wanted carrier and interfering carrier.

Proposal 2: The largest interferer power in-gap shouldn’t larger than -25dBm for fragmented carriers using a single Rx RF chain.

Proposal 3: For ΔRIBNC re-evaluation, only the configuration of the same SCS for both CC is enough, and the results should be applied to the others SCS configurations for the both CC.

Proposal 4: The assumption for the feasible wider low pass filter attenuation should be discussed for ΔRIBNC re-evaluation.

# 4	Reference

[1] R4-2415226, Topic summary for [112bis][122] FS_NR_DL_Frag_Carrier, Mediatek

[2] R4-2417202, Ad hoc minutes on fragemented DL, Mediatek

[3] R4-2417201, WF on fragmented DL, Mediatek

[4] R4-2415911,View on Fragmented carrier general part and RF requirements, ZTE Corporation, Sanechips

[5] R4-2415910, View on methods for reducing the number of UE Rx chains, ZTE Corporation, Sanechips

[6] R4-2415360, Discussion on UE RF requirement impact for DL fragmented carriers, Murata Manufacturing Co Ltd.

[7] R4-2415804, On methods for reducing the number of UE Rx chain, Huawei