3GPP TSG-RAN4 Meeting # 113	R4-2418706

Orlando, USA, Nov.18th – 22th, 2024

# Source: 	ZTE Corporation, Sanechips

# Title: 	On EESS mmWave UE spurious emission

# Agenda item:	6.21.2

# Document for:	Approval

# 1 Introduction

In last RAN #105 meeting, a new work item for mmWave UE spurious emission was approved in [1], the objectives  are captured below:

The objective of this WI includes the following aspects: - For UE, specify additional spurious emission requirements of -5dBm/200Mz for 23.6-24 GHz frequency range for n257 and n258, where - Introduce new corresponding Network Signalling(s) and associated A-MPR(s) if needed by taking into the consideration of the relationship with the existing NS_200, NS_202, NS_203, CA_NS_200, CA_NS_202 and CA_NS_203 - All the power classes are considered in the above work - For BS, conduct necessary modifications on protection of Earth Exploration Satellite Service requirements if any

The work aims to study and define new NS_XX and the corresponding A-MPR requirements for the more stringent UE spurious emission requirement of -35dBW/200MHz or -5dBm/200MHz in EESS(Earth Exploration Satellite Service) frequency range of 23.6GHz-24GHz which shall be satisfied and required in China and EU at an earlier stage.

In this contribution, we give some further discussions on the NS flags and the corresponding A-MPR requirements for the new limits -35dBW/200MHz or -5dBm/200MHz in EESS frequency range of 23.6GHz-24GHz.

# 2	Discussion

## 2.1 NS flags and A-MPR

In last meeting, some agreements on UE RF requirements for EESS protection are approved in the WF [2]. For the NS signalling:

<Way forward for NS signalling > Further analyse whether NS_202 can be updated by just replacing +1 dBm/200 MHz with -5dBm/200MHz EESS passive services protection in the 23.6GHz-24GHz for band n257 and n258.

As we discussed in our contribution in [3], there are three following regulatory limits need to be considered :

1:  -10 dBm/100 MHz (Harmonic) in 7.25 GHz ≤ f ≤ 2nd

2:  +1 dBm/200 MHz in 23.6 - 24.0 GHz

3:  -5dBm/200 MHz in 23.6 - 24.0 GHz

For the existing NS_202, it defines the A-MPR requirements to meet the emissions for both -10 dBm/100 MHz (Harmonic) in 7.25 GHz ≤ f ≤ 2nd and +1 dBm/200 MHz in 23.6 - 24.0 GHz, and NS_202 was introduced in Rel-16.

In [4], it mentioned that ‘..potential EU emission limits of +1dBm/200MHz discussed in RSCOM but, as far as we investigated, no EC Decision was published based on such discussion. Rather, as we mentioned previously, the amended EU Decision specified a limit of -5dBm/200 MHz which was not considered in TS 38.101-2.’.

As discussed in our controbution, the existing NS_202/CA_NS_202 A-MPR requirements are derived based on the regulatory requirements of -10dBm/100MHz of RC REC 74-01, and it can met the -5dBm/200MHz (or -35 dBW/200 MHz) requirements in 23.6 - 24.0 GHz ranges as well. So update the NS_202 by just replacing +1 dBm/200 MHz with -5dBm/200MHz EESS will not introduce new A-MPR requirements.

Observation 1: The existing NS_202/CA_NS_202 A-MPR requirements can met the -5dBm/200MHz (or -35 dBW/200 MHz) requirements in 23.6 - 24.0 GHz ranges.

Although company investigated that +1dBm/200MHz is not published by EC Decision, we are not sure if there are commercial UE designed to report NS_202 to meet the 3GPP emission requirements. If there are not, then it seems it is feasible to update NS_202 by just replacing +1 dBm/200 MHz with -5dBm/200MHz EESS passive services protection. Otherwise, NBC issue would be existed.

Proposal 1. If there are no NBC issue, it is feasible to update NS_202 by just replacing +1 dBm/200 MHz with -5dBm/200MHz EESS passive services protection. Otherwise, new NS values are needed.

In addition to that, there were some agreements on the necessity of New NS signalling captured in the Chairman’s note:

Necessity of New NS signalling Agreement: - Defining new NS values (i.e. NS_205) considering all the power classes to meet -5 dBm/200 MHz (or -35 dBW/200 MHz) EESS passive services protection in the 23.6GHz-24GHz for band n258. - Define the A-MPR requirements considering all the power classes to meet -5 dBm/200 MHz (or -35 dBW/200 MHz) EESS passive services protection in the 23.6GHz-24GHz for band n257 - FFS to define the EESS protection requirements as general requirements for n257.

This is for the updated limits -5dBm/200MHz (or -35 dBW/200 MHz) for EESS 23.6~24GHz range, where A-MPR requirements are needed to meet the new emission requirements.

There are some proposed A-MPR requirements for the new emission requirements in the TR38.871-01 [6], we copy and paste them in the table 1 below:

Table 1. NS value for band n257 and n258 meet the regulatory requirements (extracted from Table 7.5.4.1-2[6])

| Band | Requirement | A-MPR for PC1 | A-MPR for PC2 | A-MPR for PC3 | A-MPR for PC4 |
| --- | --- | --- | --- | --- | --- |
| n258 | (Case A) Protected range: 23.6 - 24.0 GHz Limit: +1 dBm/200 MHz | 3.0 if Offset frequency < BWchannel, 0.0 otherwise | 0 | 0 | 0 |
| n257 |  | 0 (absorb as general requirement) | 0 (absorb as general requirement) | 0 (absorb as general requirement) | 0 (absorb as general requirement) |
| n258 | (Case B) Protected range: 23.6 - 24.0 GHz Limit: -5 dBm/200 MHz | 7.0 if Offset frequency < BWchannel, 6.0 otherwise | 1.0 if Offset frequency < BWchannel, 0.0 otherwise | 1.0 if Offset frequency < BWchannel, 0.0 otherwise | 1.0 if Offset frequency < BWchannel, 0.0 otherwise |
| n257 |  |  |  |  |  |

For the sake of handling the above A-MPR requirements for different limits, new NS flags, i.e. NS_205 for single CC, and CA_NS_205 for UL CA, corresponding to an 6dB lower requirement, i.e. -5 dBm/200 MHz (or -35 dBW/200 MHz).

It is proposed to adopt the above A-MPR requirements for the Protected range: 23.6 - 24.0 GHz with -5 dBm/200 MHz  (or -35 dBW/200 MHz) emission limit.

Proposal 2. To consider the A-MPR requirements defined in TS38.817-01 to meet the -5 dBm/200 MHz (or -35 dBW/200 MHz) emission limit in 23.6 - 24.0 GHz, which are:

- PC1: 7.0 if Offset frequency < BWchannel, 6.0 otherwise

- PC2~7: 1.0 if Offset frequency < BWchannel, 0.0 otherwise

- The Offset frequency is defined as the frequency from 24.25 GHz to the lower edge of the channel bandwidth.

## 2.2 ModifiedMPRbehavior bits

In terms of the TR38.817-01, adopting the modifiedMPR bits mainly to solve the issue that BS cannot know which NS values each of UEs can support in the case of two types of UE existing in a real environment: one is UE working with an existing NS and the other is UE working with a new NS.

In the current TS38.101-2, the modifiedMPRbehavior indication for band n258 NS_203 and CA_NS_203 have already been defined. We think the same approach shall be adopted for the new introduced NS flags and CA_NS flags, which is also mentioned in TR38.817-01:  ‘Note that NS_20X and NS_20Y described in Table 7.5.4.1-3 have not yet been specified in TS 38.101-2 v15.14.0.The same approach with introduction of NS_203 should apply to these NSs according to approved WF [R4-2009141], i.e., Alt 1-2(Explicit signaling for a UE to report newly supported NS value(s) for a legacy band to the network (reuse modifiedMPR bits)) ...’

Proposal 3: The same approach with introduction of NS_203, i.e. modifiedMPRbehavior indication, should apply to the new NS values for band n257 and n258, respectively.

## 2.3 Spurious emission band UE co-existence

Due to this WID include both n257 and n258, so the protection for the frequency range 23.6GHz to 24 GHz should be introduced in the spurious emission band UE co-existence table to include the -5dBm/200MHz limits. So it is proposed to introduce EESS protection of -5dBm/200MHz for n257 and n258 into general spurious emission band UE co-existence requirements. Otherwise, UE may violate regulatory requirements in some regions who need regulation requires EESS protection in 23.6GHz-24GHz.

Similar with BS [6], to adopt different changeover dates for different countries/regions for the requirement applying UE brought into use for -5 dBm/200 MHz (or -35 dBW/200 MHz) limits.

Proposal 4: It is proposed to introduce 23.6GHz-24GHz protection of -5dBm/200MHz for n257 and n258 into general spurious emission band UE co-existence requirements with the following different dates:

-  +1dBm/200MHz limit applies to UE brought into use on or before 1 September 2027 in countries not adopting EU Decision 2020/590.

-  -5dBm/200MHz This limit applies to UE brought into use after 1 September 2027 or to UE in countries adopting EU Decision 2020/590.

# 3 Conclusion

In this contribution, we give some further discussions on the NS flags and the corresponding A-MPR requirements for the new limits -35dBW/200MHz or -5dBm/200MHz in EESS frequency range of 23.6GHz-24GHz based on the the RAN4 previous discussion and agreements, also referring to the TR38.817-01.

The observations, proposals and conclusions are:

Observation 1: The existing NS_202/CA_NS_202 A-MPR requirements can met the -5dBm/200MHz (or -35 dBW/200 MHz) requirements in 23.6 - 24.0 GHz ranges.

Proposal 1. If there are no NBC issue, it is feasible to update NS_202 by just replacing +1 dBm/200 MHz with -5dBm/200MHz EESS passive services protection. Otherwise, new NS values are needed.

Proposal 2. To consider the A-MPR requirements defined in TS38.817-01 to meet the -5 dBm/200 MHz (or -35 dBW/200 MHz) emission limit in 23.6 - 24.0 GHz, which are:

- PC1: 7.0 if Offset frequency < BWchannel, 6.0 otherwise

- PC2~7: 1.0 if Offset frequency < BWchannel, 0.0 otherwise

- The Offset frequency is defined as the frequency from 24.25 GHz to the lower edge of the channel bandwidth.

Proposal 3: The same approach with introduction of NS_203, i.e. modifiedMPRbehavior indication, should apply to the new NS values for band n257 and n258, respectively.

Proposal 4: It is proposed to introduce 23.6GHz-24GHz protection of -5dBm/200MHz for n257 and n258 into general spurious emission band UE co-existence requirements with the following different dates:

-  +1dBm/200MHz limit applies to UE brought into use on or before 1 September 2027 in countries not adopting EU Decision 2020/590.

-  -5dBm/200MHz This limit applies to UE brought into use after 1 September 2027 or to UE in countries adopting EU Decision 2020/590.

# 4	Reference

[1] RP-242371, New WID on mmWave UE spurious emission, China Unicom, and etc.

[2] R4-2417083, WF on UE RF requirements for EESS protection, Huawei, China Unicom

[3] R4-2415908, On EESS mmWave UE spurious emission, ZTE Corporation, Sanechips

[4] R4-2416161, EESS protection - UE RF aspects, Ericsson

[5] R4-2008410, WF on WRC-19 outcome and impact on RAN4 specifications, NTT DOCOMO, INC.