3GPP TSG-RAN4 Meeting # 113	R4-2418707

# Orlando, USA, Nov.18th – 22th, 2024

# Source:	ZTE Corporation

# Title:	TP for TR38.749 BS requirements(Clause 9)

# Agenda Item:	6.21.3

# Document for:	Approval

- Introduction

In this contribution, a TP to TR38.749[3] is provided for BS requirement(Clause 9) based on the WF[1] and the endorsed draft CR to TS38.104[2].

- Reference

- [1] R4-2417084	WF on BS RF requirements for EESS protection, China Unicom, Huawei, HiSilicon, ZTE, CATT

- [2] R4-2417085	Draft CR to TS 38104 - EESS protection - BS RF aspects,Ericsson, Nokia, Huawei, ZTE, CATT

- [3] R4-2417199	TR skeleton for Rel-19 EESS,  Huawei, HiSilicon

- Text Proposal

----- Start of TP -----

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	RP-240787: “Study on IMT parameters for 4400 to 4800 MHz, 7125 to 8400 MHz and 14800 to 15350 MHz”

…

[x]	Commission Implementing Decision (EU) 2020/590 of 24 April 2020 amending Decision (EU) 2019/784 as regards an update of relevant technical conditions applicable to the 24,25-27,5 GHz frequency band

<Next changes>

- 9	BS RF requirements

- The study for BS RF requirements are to specify additional emission requirements of -9dBm/200Mz for 23.6-24 GHz frequency range for NR bands n257 and n258.  Except additional OTA OBUE limits and additional OTA transmiiter spurious emissions requirements for protection of Earth Exploration Satellite Service, there are no other BS RF requirements impacted.

For the BS additional OTA OBUE limits and additional OTA transmitter spurious emissions requirements for protection of Earth Exploration Satellite Service:

-The emission level of -3dBm/200Mz applies to BS brought into use on or before 1 September 2027 in countries not adopting EU Decision 2020/590 [x], while

- The emission level of -9dBm/200Mz applied to BS brought into use after 1 September 2027 or to BS in countries adopting EU Decision 2020/590 [x].

In addition, when new regional regulations on EESS protection become available, relevant updates need to be made to relevant EESS protection tables in the BS specifications if needed.

----- End of TP -----