3GPP TSG-RAN WG4 Meeting #113				                      R4-2418721

Orlando, US, 18th – 22nd November, 2024

Agenda item:	6.20.2

Source: 	CMCC, CBN, China Broadnet

Title: 	UE RF requirements for n28

Document for:	Discussion

# 1. Introduction

In last RAN4 meeting, the UE requirements for n28 PC2 and n40 had been discussed and way forward had been agreed in [1].

In this contribution, we discuss the left open issues for n28 requirements.

- Discussion

In last RAN4 meeting, most open issues had been addressed. The only left issue is the NS_18 values to be further confirmed, and the channel location and channel raster related issues.

Agreement in RAN4#112bis: Issue 1-2-6 channel location Agreement: - Update the NOTE 7 as follows: - NOTE 7: For UEs supporting 30MHz max bandwidth, the minimum requirements are specified for NR UL channel bandwidths confined to [either] 703-733MHz or 718-748MHz [for the 20, 25 and 30MHz bandwidth]. - Introduce the new note as follows: - For UEs supporting 40MHz max bandwidth, the minimum requirements are specified for NR UL channel bandwidths confined to [either] [703-743.04MHz] or 718-748MHz [for the [20, 25, and] 30MHz bandwidth. And for the 40MHz bandwidth, the minimum requirements are specified for NR UL channel bandwidths confined to either 703-743.04MHz.] - FFS the content in [] Issue 1-2-7 channel raster Agreement: - Alt1: Add the exceptional channel raster point of n28 (UL: 723.04MHz, DL: 778.04MHz) to UE RF specification TS 38.101-1 for UE CBW 40MHz. - Alt2: Cover all the channel raster points of n28 based on the enhanced channel raster to UE RF specification TS 38.101-1 for UE CBW 40MHz. Issue 1-2-8 Release independence Agreement: - Option 1: Rel-16 - Option 2: Rel-15

For channel location, it was agreed to change the existing NOTE7 to “UL channel bandwidth” during online discussion. Then during the offline discussion, companies commented that “[either]” and “[for the 20, 25 and 30MHz bandwidth]” can be removed to simply the sentence. In our view, it is preferable to remove the words in words in [] to simply the NOTE, since the NOTE7 is for UE supporting 30MHz max bandwidth.

It is proposed to update NOTE7 as follows:

  - NOTE 7:	For UEs supporting 30MHz max bandwidth, the minimum requirements are specified for NR UL channel bandwidths confined to [either] 703-733MHz or 718-748MHz [for the 20, 25 and 30MHz bandwidth].

Regarding the new NOTE for UEs supporting 40MHz max bandwidth, the similar approach can be used to simply the NOTE.

It is proposed to update the new note as follows:

  - NOTE X: For UEs supporting 40MHz max bandwidth, the minimum requirements are specified for NR UL channel bandwidths confined to [either] [703-743.04MHz] or 718-748MHz [for the 20, 25, and 30MHz bandwidth. And for the 40MHz bandwidth, the minimum requirements are specified for NR UL channel bandwidths confined to either 703-743.04MHz.]

Proposal 1: It is proposed the apply the following note for n28:

  - NOTE 7:	For UEs supporting 30MHz max bandwidth, the minimum requirements are specified for NR UL channel bandwidths confined to 703-733MHz or 718-748MHz.

  - NOTE X: For UEs supporting 40MHz max bandwidth, the minimum requirements are specified for NR UL channel bandwidths confined to 703-743.04MHz or 718-748MHz.

Regarding the two alternatives for channel raster of n28, since it was agreed that enhanced channel raster is mandatory support for n28. Following the release independent manner, for n28 UE which is release independent from earlier release, the enhanced channel raster should also be supported. Hence, it seems that alt1 is more straightforward from specification perspective.

- Alt1: Add the exceptional channel raster point of n28 (UL: 723.04MHz, DL: 778.04MHz) to UE RF specification TS 38.101-1 for UE CBW 40MHz.

- Alt2: Cover all the channel raster points of n28 based on the enhanced channel raster to UE RF specification TS 38.101-1 for UE CBW 40MHz.

Proposal 2: It is proposed to add the exceptional channel raster point of n28 (UL: 723.04MHz, DL: 778.04MHz) to UE RF specification TS 38.101-1 for UE CBW 40MHz.

# 3. Conclusions

In this contribution, we discuss the left open issues for n28 requirements. The proposals are:

Proposal 1: It is proposed the apply the following note for n28:

  - NOTE 7:	For UEs supporting 30MHz max bandwidth, the minimum requirements are specified for NR UL channel bandwidths confined to 703-733MHz or 718-748MHz.

  - NOTE X: For UEs supporting 40MHz max bandwidth, the minimum requirements are specified for NR UL channel bandwidths confined to 703-743.04MHz or 718-748MHz.

Proposal 2: It is proposed to add the exceptional channel raster point of n28 (UL: 723.04MHz, DL: 778.04MHz) to UE RF specification TS 38.101-1 for UE CBW 40MHz.

# 4. Reference

- R4-2417082, WF on PC2 and 40MHz CBW for n28, CMCC

- R4-2414274, WF on introduction of PC2 and 40MHz CBW in NR band n28, CMCC, CBN, China Broadnet

- RP 241660, New WID: Introduction of Power Class 2 and UE 40MHz Channel Bandwidth in NR band n28, CMCC CBN.

- R4-2410558, WF on A-MPR for n28, Apple (approved).

- R4-2406708, WF on band n28 full band duplexer, Apple.

- R4-2403629, WF on HPUE for FDD bands, China Unicom, Skyworks, RAN4#110 (approved).