3GPP TSG-RAN WG4 Meeting # 113                                                             R4-2418732

Orlando, US, 18th – 22nd November, 2024

Agenda item:	7.10.2.3

Source: 	CMCC

Title: 	Discussion on UE RF requirements for ATG with UL-MIMO

Document for:	Discussion

# 1. Introduction

RAN#105 meeting approved revised WID on Enhancements for Air-to-ground network for NR in Rel-19 [1]. After discussions of several meetings, we have made good progress on ATG with UL-MIMO. This paper will focus on the remaining issues.

# 2. Discussion

2.1. ULFPTx mode for UL-MIMO

Issue 3-1-1: ULFPTx mode for UL-MIMO WF: Consider whether and how to introduce this requirement. - Option 1: ULFPTx mode 1/2/Full in the following table can be supported depending on UE PA implementation - Table 2: Example of ATG ULFPTx (R-MOP = x dBm) ATG ULFPTx mode; PA1(dBm); PA2(dBm). Mode2; x; (x-3)~(x-1). Mode-Full; x; x. Mode1; (x-3)~(x-1); (x-3)~(x-1). - Option 2: Current ULFPTx mode requirements for legacy UE in TS38.101-1 can be reused for ATG UE, and there is no other specification affect. - Option 3: No need to extend ULFPTx mode to ATG UE.

After last meeting discussion, companies still have different understand on the ULFPTx mode. Uplink full power transmission is a method for UE transmission enhancement. Be limited by power control, UE could not reach full power transmission in some cases, and it will influence the uplink coverage and transmission rate. To ensure PA could reach the max output power for the power class, different ULFPTx Modes are designed for UEs with different capabilities. Since ULFPTx mode has been supported for legacy, we think ULFPTx mode should be supported for ATG UL-MIMO.

Proposal 1: Support ULFPTx mode for ATG UL-MIMO.

The subsequent challenge is to determine how to incorporate this mode within our specification. From our perspective, Mode 1, Mode 2, and Mode Full correspond to UE capability levels 2 (non-full rated PAs), UE capability level 3 (partially full rated PAs), and UE capability level 4 (all full rated PAs), respectively. The specification does not currently define the output power per PA or per Tx port. When ATG UL-MIMO operates in the ULFPTx mode, it is essential to specify the maximum output power for ATG UEs in the section "UE Maximum Output Power for UL MIMO".

Proposal 2: ATG UE maximum output power should be clarified in the section “UE maximum output power for UL MIMO”.

2.2. Single antenna port and coherent requirement for UL-MIMO

Issue 3-1-2: whether preclude the single antenna port related requirement WF: - Option 1: Yes. - Option 2: No.

In a similar situation regarding whether to support ULFPTx mode for UL-MIMO, we have not yet reached a consensus on supporting a single antenna port. Some companies believe that the benefits of implementing these capabilities are minimal, and they are not widely used in legacy UE. However, in our view, these concerns should not be used as reasons to exclude these capabilities, as they can make the ATG use scenarios more flexible. Moreover, no additional special requirements need to be specified for ATG UE, and the existing requirements for TN UE can be reused.

Proposal 3: Apply the single antenna port related requirement for ATG UE, and the legacy requirements for TN UE could be reused.

# 3. Conclusions

In this contribution, UE RF requirements for ATG with UL-MIMO are discussed with the following proposals:

Proposal 1: Support ULFPTx mode for ATG UL-MIMO.

Proposal 2: ATG UE maximum output power should be clarified in the section “UE maximum output power for UL MIMO”.

Proposal 3: Apply the single antenna port related requirement for ATG UE, and the legacy requirements for TN UE could be reused.

# 4. References

[1] RP-242381, Revised WID on Enhancements for Air-to-ground network for NR