3GPP TSG-RAN WG4 Meeting # 113                                   R4-2418735

Orlando, US, 18th – 22nd November, 2024

Agenda item:	7.23.3

Source: 	CMCC

Title: 	Discussion on LP-WUS BS RF requirements

Document for:	Discussion

# 1. Introduction

In RAN plenary #103 meeting, revised WID of LP-WUS is approved with following RF part objectives:

- Specify the necessary RAN4 core requirement(s) to support the feature (RAN4).

  - Specify UE low-power wake-up receiver requirements, at least REFSENS, ACS and ASCS requirements with consideration of possible new methodology to assess the low-power wake-up receiver performance

    - Define guard RBs for ACS and ASCS cases

    - Study testability of above requirements

    - Consider impacts of different architecture and impairments, and set requirements that enable all types of reasonable implementation

  - Study and if necessary specify or support by declaration, the corresponding BS requirements, e.g., dynamic range for LP-WUS/LP-SS.

    - Current NR BS requirements is baseline

In RAN4#112-bis, the LP-WUS BS RF requirements had been discussed and the WF was approved in [1]. This paper will focus on some issue about LP-WUS BS RF requirements.

- Discussion

2.1 Minimum power boosting level for LP-WUS

Issue 1-3: On minimum power boosting level for LP-WUS - Proposals - Need to have it. - If power boosting will be defined as EPRE ratio. - Proposal 1: Use 3dB as minimum requirement. (CATT, Huawei, [vivo]) - If power boosting will not be defined as EPRE ratio. - Proposal 2: Use 3dB as minimum requirement. (Ericsson, [vivo]) - Proposal 3: The core requirement for the dynamic range for LP-WUS/LP-SS shall be specified the same as 64QAM, i.e., 0dB. (Nokia) - Proposal 4: No matter how to define power boosting, the requirement should consider the assumption in RAN1 of 3dB or 6dB means the EPRE ratio. The requirement can be different for different CBW. (vivo) - Proposal 5: No need to have it. (CMCC, ZTE, Nokia)

From our perspective, it is challenging to establish a minimum requirement specific to our network. Given that different operators employ this technology across a range of scenarios, setting a uniform minimum requirement is complex. To preserve the flexibility associated with power boosting, we propose not to stipulate a minimum level for power boosting. Instead, the power boosting level should be determined based on the specific declarations made.

Proposal 1: Not need to define the minimum power boosting level.

### 2.2 Cap for LP-WUS power boosting

Similar with the minimum power boosting level, we think there is no need to define the maximum value for LP-WUS power boosting. Since the power boosting level is based on the declaration.

Proposal 2: Leave the maximum power dynamic range to manufacturer’s declaration

### 2.3 Preclude small CBW for power boosting

There is a common understanding that compared to wide CBW, under the same level of power boosting, the impact on small bandwidths is greater. In our opinion, whether to preclude the small CBW for power boosting is depended on how to define the minimum power. If we don’t define the minimum power boosting level requirements, there is no need to preclude the small CBW.

Proposal 3: If the minimum power boosting level requirements is not defined, there is no need to preclude the small CBW.

### 2.4 Applicable BS type for LP-WUS feature

Issue 1-8: General consideration on other requirements - Proposals - Proposal 1: RAN4 to first determine whether a full set or simplified set of BS Tx requirements should be specified for LP-WUS. (Huawei) - Recommended WF - Discuss on Proposal 1.

To ensure the integrity of LP-WUS spec and meet all companies’ deployment demand, it’s better not limit BS type for LP-WUS feature. Besides, we think it is too early to preclude any BS type of the LP-WUS. We think it is better not to limit the applicable BS types for LP-WUS.

Proposal 4: All the BS types should be supported by LP-WUS.

For BS RF requirements, in order to regulate the use of LP-WUS and its performance, we suggest to define a set of the BS Tx requirements. Unwanted emissions should also to be defined.

Proposal 5: Define a set of the BS Tx requirements for LP-WUS.

# 3. Conclusions

Proposal 1: Not need to define the minimum power boosting level.

Proposal 2: Leave the maximum power dynamic range to manufacturer’s declaration

Proposal 3: If the minimum power boosting level requirements is not defined, there is no need to preclude the small CBW.

Proposal 4: All the BS types should be supported by LP-WUS.

Proposal 5: Define a set of the BS Tx requirements for LP-WUS.

# 4. Reference

- R4-2416566, Way Forward for [112bis][308] NR_LPWUS