3GPP TSG-RAN WG4 Meeting # 113                                    R4-2418738

Orlando, US, 18th – 22nd November, 2024

Agenda item:	7.20.1

Source: 	CMCC

Title: 	Discussion on SBFD general part

Document for:	Discussion

# 1. Introduction

In TR 38.858, the impacts on SBFD requirements have been analyzed. RAN4 has identified which legacy requirements are still applicable, which legacy requirements are not applicable and which new requirements are needed.

In last meeting, WF on SBFD BS requirement is approved with several agreements. [2]

In this contribution, we continue the general part discussion based on the summary approved in last meeting.

- Discussion

  - Band/channel bandwidth support for SBFD

In last meeting, the candidate options are listed as below.

Agreement: - To include the following text in Clause 5.2 in TS 38.104: subband full duplex can be applied to the following TDD bands given in Table 5.2-1, by giving the band numbers which support SBFD. - Detailed band list is provided based on the agreement for minimum channel bandwidth for SBFD - FFS the band for NR-U shall be supported

There is common understanding that the SBFD feature is only supported by licensed bands, and based on this the band for NR-U couldn’t be supported.

Proposal 1: NR-U band could not be supported.

  - Specification structure for SBFD requirements

Different from other BS spec, there are candidate new requirements for SBFD, e.g. transient period. For such new RF requirements, it’s better to create new sub-clauses in TS 38.104 since such requirements only apply for SBFD gNB, which is much similar as UE feature in TS 38.101. For the legacy RF requirements, some of them are still applicable for SBFD with the same limit but others may need to be updated with new limit, e.g. Tx inter-modulation. for spec consistency, it’s also capture all legacy RF requirements applicable for SBFD into the new sub-clause. For the RF requirements that are the same for legacy gNB and SBFD, it’s better just state that the RF requirements in sub-clause still applies instead of simple copy-paste.

Proposal 2: for RF spec structure of SBFD requirements, it’s better to create new sub-clauses in TS 38.104 which capture all new and existing RF requirements that applies for SBFD.

  - DL/UL subband configurations support for SBFD

In last meeting, the candidate options are listed as below.

Way Forward: - RAN4 to further study Option 2b by considering the below additional restrictions such as: - For DUD case, the UL subband should be placed in the middle of channel bandwidth - FFS the number of DL subband sizes to be declared and tested - Alt-1: For FR1 WA BS, one DL subband size is declared to support SBFD and tested, operation with other DL subband sizes than the tested one is not allowed. - FFS the necessity of this restriction is applicable to other FR1 BS classes and FR2 BS. - Alt-2: For FR1 WA BS, the DL subband sizes declared to support SBFD shall be tested - FFS the necessity of this restriction is applicable to other FR1 BS classes and FR2 BS. - Restriction on UL subband size based on the specific channel bandwidth - FFS to specify a range of ratio for UL subband size over channel bandwidth. - FFS the restriction is different for DU and DUD - FFS the restriction is applied for FR2 case.

Following the discussion from the previous meeting, we have arrived at a consensus based on Option 2b, which offers a moderate level of flexibility. However, there is still divergence among companies regarding the number of DL subband sizes to be supported for SBFD. Alternative 1 suggests that only one subband size is declared for SBFD support and testing, with no allowance for other DL subband sizes. In our view, one of the key benefits of SBFD is its flexible frame structure. To retain this advantage, we advocate for no restrictions on the size of subbands. As for Alternative 2, it is contingent upon implementation and cannot be ruled out at this stage. Alternative 2, which provides a balanced degree of flexibility, is deemed an appropriate method to sustain the advantages of SBFD.

Proposal 3: To keep the flexibility of SBFD, Alt-2 could be used as baseline.

  - Guard band

Since the sub-band size is not fixed in the spec, the guard band also should not be fixed. The maximum value of the guard band size should be specified in the spec, and some guard RB could be used as DL or UL in actual operation.

Proposal 4: The maximum value of the guard band size should be specified in the spec.

# 3. Conclusions

In this contribution, SBFD RF requirements are discussed with following observations and proposals.

Proposal 1: NR-U band could not be supported.

Proposal 2: for RF spec structure of SBFD requirements, it’s better to create new sub-clauses in TS 38.104 which capture all new and existing RF requirements that applies for SBFD.

Proposal 3: To keep the flexibility of SBFD, Alt-2 could be used as baseline.

Proposal 4: The maximum value of the guard band size should be specified in the spec.

# 4. Reference

[1] TR 38.858,

[2] R4-2416584, Way forward for [112bis][306] NR_duplex_evo_General