3GPP TSG-RAN WG4 Meeting # 113                                    R4-2418742

Orlando, US, 18th – 22nd November, 2024

Agenda item:	7.1.1.2.1

Source: 	CMCC

Title: 	Discussion on enhanced MPR scenario

Document for:	Discussion

# 1. Introduction

In RAN plenary #103 meeting, a new WID of UE power domain enhancement is approved with following RAN4 lead RF part objectives:

- Specify power domain enhancement, e.g., MPR reduction for NR single carrier and NR intra-band UL CA

  - Study the scenarios, and if feasible, specify the power domain enhancement, e.g., MPR reduction, for PC2 and PC3 with applicable ACLR/SEM/spurious emission modification with BS indication for NR FR1 on a single UL carrier

    - Include the following scenarios:

      - when there is no adjacent in-band/out-of-band co-existence issue

      - when a UE uses a narrower channel bandwidth within a wider BS bandwidth

    - Include both (e)RedCap UE (only PC3) and non-RedCap UE

    - Limited to QSPK and 16QAM

  - Specify MPR applicability based on the UL CCs with activated cells for NR intra-band UL CA configuration

    - Include both intra-band UL contiguous CA and intra-band non-contiguous UL CA for FR1

    - Include intra-band UL contiguous CA and intra-band DL contiguous CA with single UL for FR2

    - MPR requirement is not applicable until the SCell is activated

  - Necessary signaling to support the above objectives

After several meetings of discussion, we have made some progress., and this paper will focus on the scenario discussion.

# 2.Discussion

Following lists scenarios that lead to relaxation of ACLR/SEM/spurious emission requirements:

- Scenario 1-1: Scenario with no adjacent in-band/out-of-band co-existence issue (single operator)

- Scenario 1-2: Scenario with no adjacent in-band/out-of-band co-existence issue (adjacent operators)

- Scenario 2: Narrower UE channel BW within wider BS bandwidth.

After last meeting discussion, we had reached the WF below.

Issue 1-1-1: Approaches for scenarios 1-1 and 1-2 (see Reference for description of scenarios) - WF - Strive for a unified solution which can accommodate both scenario 1 and scenario 2 under the framework of converting outer RB allocation to inner RB allocation. - It does not preclude separate solutions for each applicable scenario No relaxation of ACLR/SEM/SE values

For scenario 1-1, to avoid spectrum fragment, the regulatory agency usually allocates the whole band spectrum for only one operator. For example, the band n34 (2010-2025MHz) with total 15MHz frequency range is only allocated for CMCC in China. For this case, to be honest, ACLR/SEM requirements may not be needed if only considering adjacent carrier interference among different operators. Without relaxation of ACLR/SEM/SE, converting outer RB allocation to inner RB allocation means shift the OOB applicable frequency range, and we think it is indeed a good approach.

Proposal 1: Converting outer RB allocation to inner RB allocation to shift the OOB applicable frequency range is a good approach to enhanced MPR.

However, for scenario 1, different bands have different deployment scenarios in different countries. For example, band n28 in China is only allocated to one individual operator without adjacent-carrier interference issue. Besides, its adjacent band is broadcasting services which is outside of 3GPP scope so it’s hard to identify whether outer RB allocation could be converted to inner RB allocation.

For the band n39, in China it is also only allocated to CMCC without adjacent carrier interference issue. But n39 is adjacent to both band n3 DL and band n1 UL. To avoid the gNB-to-gNB interference, guard spectrum is reserved, i.e. 5MHz as shown below. For this case, according to 3GPP, band n39 UE ACLR1 still applies in the frequency range of band n1 even with guard band. but it seems converting outer RB allocation is allowed due to the guard band. RAN4 should consider whether this feature is allowed or not when there is certain guard band and the guard band is less than the CBW assumption.

Based on the above analysis, we think when RAN4 define an unified solution, scenario 1-1 should be considered. Besides, during the discussion of these issues, the adjacent carrier interference and different deployment scenarios should also be considered.

Proposal 2: Scenario 1-1 needs to be taken into account

Proposal 3: Adjacent carrier interference and spectrum deployment should be factored into every scenario.

# 3. Conclusions

In this contribution, the scenario for ACLR/SEM relaxation are discussed with following observations and proposals.

Proposal 1: Converting outer RB allocation to inner RB allocation to shift the OOB applicable frequency range is a good approach to enhanced MPR.

Proposal 2: Scenario 1-1 needs to be taken into account

Proposal 3: Adjacent carrier interference and spectrum deployment should be factored into every scenario.

# 4. Reference

- R4-2417111, WF on power domain enhancements