3GPP TSG RAN WG4 Meeting #113	R4-2418747

Orlando, US, 18th – 22nd November, 2024

Agenda item:			7.17.1

Source:	vivo

Title:	Discussion on general aspects for AI/ML

Document for:	Discussion

- Introduction

On the general aspects for AI/ML, in the last meeting, RAN4 further discussed the issues on post-deployment testing, AI performance, etc. In this contribution, we further provide our views on the post deployment handling issue based on the way forward captured in [1].

- Discussion

  - Test post-deployment

Issue 1-2: Post deployment testing When operating in the field, two aspects of UE operation may impact performance: - Update or fine tuning or addition/removal of models, if applicable, resulting in a change of functionality which may impact functionality performance - Data drift / mismatch between the conditions encountered by the UE in the field and the training data, which may impact functionality performance. For dealing with potential changes in the performance of functionalities two options may be available. The options are not mutually exclusive: - Option 1: Conduct the validation of a change in AI model/functionality before its deployment/activation in already deployed UEs - Validation takes into account the UE hardware in which the model is to be deployed/activated. - FFS whether the validation takes place in the field (i.e., on each individual device) or in the lab conditions (i.e., per device type/model) - FFS whether validation can be done at the device along with inference for another active functionality(ies) - FFS on the feasibility - FFS on one possibility for consideration for option 1 is to capture model input (and if needed other test data such as ground truth) during conformance testing. This stored data can later be used to validate new or updated models. - Other aspects not precluded. - Option 2: Using performance monitoring and LCM procedures - Performance monitoring will be designed in other groups - RAN4 may consider the need and feasibility of requirements and tests to ensure consistency and accuracy of monitoring metrics or other monitoring related data sent from the UE, and set requirements as feasible/needed. - FFS on monitoring can be used for managing fallback, changes in functionality, model update/model switching/model transfer, if applicable FFS: RAN4 needs to clarify whether changed functionalities that did not pass conformance testing are expected to be activated at the device. For dealing with drift / mismatch between the conditions encountered by the UE in the field and the training data for the model, which may impact functionality performance, monitoring is needed. - Performance monitoring will be designed in other groups - RAN4 may consider the need and feasibility of requirements and tests to ensure consistency and accuracy of monitoring metrics or other monitoring related data sent from the UE, and set requirements as feasible/needed. - Monitoring can be used for managing fallback, change in functionality, model update/model switching/model transfer, if applicable

For Post deployment testing, the main purpose is to guarantee the performance for updated AI/ML-enabled functionalities deployed in the field. Without consideration of conformance testing, there are two directions in the ongoing discussion as listed in Issue 1-2. One is to conduct the specific validation procedure before its deployment. And another is to use performance monitor mechanism to monitor its performance. Generally, both of the two options can verify that model performance is still good, mainly by enabling a comparison between expected results and prediction results.

For Option 1, if the validation is done by the UE itself assuming no interaction with the network (or just inform the network that the updated model has passed the performance validation by UE), it looks more like some UE implementations. We think there needs to be more clarification about such a validation, how to verify that UE has conducted such a process, and what the potential specification impact implications are.

Next, we will focus on the analysis for Option 2 on conducting performance monitoring. For Option 2, one concern raised by some companies is that if there is performance degradation occurs in the updated model, how to find out the causes for degradation, e.g., due to model update/data drift or other factors that possibly affect functionality performance.

For this question, considering the issue may be more common for two-sided model, here we will take CSI compression as an example for clarifying whether the cause can be identified by conducting performance monitoring.

For two-sided model in CSI compression use case, there have been some studies in terms of this issue in RAN1 discussion. Based on the RAN1 study, performance monitoring is capable of identifying the cause of performance degradation. Here we will provide the main processes of cause identification respectively for both NW-side monitoring and UE-side monitoring.

- Cause identification with NW-side monitoring

- Illustration of cause identification with NW-side monitoring

The procedure of NW monitoring is the same for option 1/3/4/5 (at least for inter-mediate KPI based monitoring), i.e., UE reporting ground-truth CSI to help NW compute the inter-mediate KPI. Assuming an obvious performance degradation for ongoing model has been observed by NW, NW side can:

a) check whether NW side CSI reconstruction model is correctly deployed by comparing the expected output and real output given the same input. Actually, checking CSI reconstruction model does not rely on real-time measurement, which can be done at any time.

b) compare the performance between NW-side original model (i.e., green part in the above Fig. 1) with reported ground-truth CSI from UE (NW-side original model refers to the model firstly trained by NW, and the possible model transfer/dataset delivery in option 3/4/5 are all based on the original model) and ongoing model (i.e., UE-side generation and NW-side reconstruction). If the performance of NW-side original model is satisfying while that of ongoing model is not good, we can identify that at least UE side causes the degradation. If the performance of NW-side original model also degrades, we can identify that at least data drifts happen for the ongoing model.

c) compare reconstructed target CSI based on the expected CSI feedback (i.e., output of NW-side original model) or reconstructed target CSI’ based on reported CSI feedback. If the reconstructed target CSI and reconstructed target CSI’ is not aligned (e.g., the difference is larger than the threshold), we can identify it’s UE side cause for the degradation. Otherwise, the UE side cause can be excluded. Or, we can compare the expected CSI feedback (i.e., output of NW-side original model) or reported CSI feedback. If the expected CSI feedback and reported CSI feedback is not aligned (e.g., the difference is larger than the threshold), we can identify UE side cause for the degradation. Otherwise, the UE side cause can be excluded.

Based on the above analysis, we can observe that identifying the cause of performance degradation is not a challenge for NW-side monitoring.

- Cause identification with UE-side monitoring

- Illustration of cause identification for UE-side monitoring

Identifying the cause of performance degradation for UE-side monitoring will be more complicated. Basically, it is challenging for NW side to identify whether it is data drift and/or UE side cause, because ground-truth CSI is not available at NW side for UE side monitoring. The only thing that NW can do is to check whether NW side CSI reconstruction model is correctly deployed by comparing the expected output and real output given the same input. Meanwhile, it is also not easy for UE to identify data drifts, since UE can hardly acquire the original CSI generation and reconstruction model simultaneously. For UE-side cause identification, we may further discuss according to the training collaboration considered (also illustrated in Fig. 2):

a) If the original CSI generation model is available at UE side (e.g., for option 3a-1/5a-1 and option 3b), UE could compare the expected output of transferred CSI generation model and real output of deployed CSI generation model to identify whether it is the UE side cause for the performance degradation. We believe that the comparison of expected output and real output is effective enough for cause identification, because model optimization at UE side usually targets mimicking the original CSI generation model with another model structure in most cases where it is available at UE side.

b) If the original CSI generation model is not available at UE side (e.g., for option 3a/5a-2 and option 4), it is challenging for UE to identify whether the deployed UE side model is correctly developed. In such cases, assistance information such as testing datasets from NW can help to identify UE side cause.

If there is performance degradation but no gNB-side or UE-side causes are identified, the cause of the performance degradation would then should be determined as data drift.

Based on the above analysis, even though identifying data drifts is challenging at both NW and UE when considering UE-side monitoring, there are still some approaches to identify UE side cause (at UE side) especially for option 3a/5a-1 and 3b.

To sum up, for two-sided model, the causes for performance degradation (e.g., UE sided cause, NW sided cause, or data drift, etc.) can be identified by conducting performance monitoring. For the scenario where the model is updated on the UE side, if it is proved through the performance monitoring process that the performance degradation is the cause of the NW side, the impact of the model update can be excluded to a certain extent. If it turns out that the performance degradation is the cause on the UE side, UE needs to take responsibility for the result and go to the next step, such as checking whether it was caused by a model update or other aspects. In general, it can be observed that the performance monitoring is sufficient for verification in post-deployment stage and no additional validation procedures are needed.

Furthermore, regarding the bullet that monitoring can be used for managing fall-back, changes in functionality, model update/model switching/model transfer, the subsequent model management decisions after the malfunction is identified by monitoring may be related to many factors. For example, it may depend on UE decision on how to implement performance recovery. And also, it may be up to the design of the performance monitoring process with involvement from other WGs.

Observation 1: For two-sided model, the causes for performance degradation (e.g., UE sided cause, NW sided cause, or data drift, etc.) can be identified by conducting performance monitoring.

However, based on RAN1 progress, only one general agreement (shown as follows) for studying post-deployment performance monitoring was reached. RAN1 is still working on the details of this mechanism. From RAN4 perspective, it is suggested to wait for more progress before going into the detailed discussion of testing design.

RAN1 Agreement in RAN1#117 For option 1/3/4/5 and their sub-options, study mechanisms (e.g., post-deployment performance monitoring) for identifying the cause (e.g., NW side, UE side, data drift) of the performance degradation to guarantee good performance in the field.

Proposal 1: For Post-deployment testing handling, AI performance after device deployment can be verified by designing the test for performance monitoring in RAN4. On the details of testing design, RAN4 to wait for more RAN1 progress on the details of performance monitoring

- Summary

In this contribution, we provided our views on general aspects for AI/ML. Based on above analysis, following proposals are present.

Observation 1: For two-sided model, the causes for performance degradation (e.g., UE sided cause, NW sided cause, or data drift, etc.) can be identified by conducting performance monitoring.

Proposal 1: For Post-deployment testing handling, AI performance after device deployment can be verified by designing the test for performance monitoring in RAN4. On the details of testing design, RAN4 to wait for more RAN1 progress on the details of performance monitoring

- References

- R4-2417212, WF on requirements for AI/ML air interface, Qualcomm Incorporated