# 3GPP TSG-RAN WG4 Meeting #113	R4-2418769

Orlando, US, 18th – 22nd November, 2024

Title: 	Discussion on FR2-1 L3 measurement delay reduction by optimizing CSSF

Source: 	OPPO

Agenda item:	7.15.2.2

Document for:	Discussion

Introduction

In last meeting, a WF[1] was approved on FR2-1 SSB based L3 measurement delay reduction. No progress was achieved on CSSF optimization.

In this contribution, we will provide discussion on FR2-1 SSB based L3 measurement delay reduction by optimizing CSSFoutside_gap.

Discussion

- Issue 2-4: Searcher assumption to apply L3 measurement delay reduction by optimizing CSSF [Way forward]: FFS: - Option 1 (CATT): Not to preclude the solutions based on 3 searchers assumption in current stage. - Option 1a (CMCC, HW): it is proposed to consider both the CSSF optimization based on 2 searchers and the CSSF optimization based on 3 searchers. - Option 1b (QC): RAN4 to consider introducing a new UE optional capability regarding the number of cell search/L3-measurement engines for CSSF enhancement, with the details to be finalized (FFS). - Option 2 (Apple, CTC, Nokia, Ericsson): RAN4 only consider the enhancement based on 2 searchers, i.e., same as previous release, for L3 measurement delay reduction by optimizing CSSF. - Option 2a (Apple): If companies cannot achieve consensus, RAN4 work can start with the baseline assumption in WID, i.e., 2 searchers. - Option 2b (Nokia) RAN4 to confirm if one of the searchers is assumed for PCC/PSCC measurement and the other is assumed for the measurements on all SCCs.

We support option 2 because 2 searchers are basic assumption for CSSF definition in current spec. For some specific cases, UE may need one more searcher, like for supporting per FR gap configuration. Even if RAN4 only consider the enhancement based on 2 searchers for CSSF, i.e., same as previous release, there still exists room by optimizing CSSF for L3 measurement delay reduction.

Thus, in general, RAN4 can continue the 2 searchers assumption as baseline for discussion. Take SA as example, the current rule of CSSF can be summarized as follows:

- The 1st searcher is dedicated to PCC. The 2nd searcher is shared among the rest CCs.

- For DC case, the 2nd searcher is dedicated to PSCC for 50% of the time, and for the other 50%, the searcher is equally shared with the rest of the SCCs. For SA case, the 2nd searcher is dedicated to SCC where neighbor cell measurement is required for 50% of the time, and for the other 50%, the engine is equally shared with the rest of the SCCs.

Proposal 1: RAN4 can start the work on CSSF optimization with the baseline assumption in WID, i.e., 2 searchers.

And in option 2b, it provides a possible solution to modify the allocation of searchers among all CCs. The 1st searcher dedicated for PCC can be shared with PSCC or SCC under certain conditions.

- Here is option 1: Share the searcher dedicated for PCC/PSCC to other CCs.

The components like CSI-RS based L3 measurement are out of scope. We only focus on SSB based L3 measurements. But some companies also provide the solution to limit the occupation of CSI-RS related measurements due to some dependency on associated SSB. The measurement on associated SSB can be counted twice for overlapping cases among different CCs.

- Here is option 2: reduce the counted number of CCs with associated SSB due to CSI-RS L3 measurement.

Other solutions are also open to discuss, like to measure one serving CC per band if multiple serving CCs are in the same band. This would change the logic of Carrier Specific scaling factor to Band Specific scaling factor.

- Here is option 3: extent the scaling factor from Carrier Specific to Band Specific.

In our view, we can discuss these solutions in priority order.

Proposal 2：RAN4 to discuss the options for CSSF optimization in priority order:

1) Share the searcher dedicated for PCC/PSCC to other CCs.

2) Reduce the counted number of CCs with associated SSB due to CSI-RS L3 measurement.

3) Extent the scaling factor from Carrier Specific to Band Specific.

Conclusions

In this paper we provided our views on FR2-1 SSB based L3 measurement delay reduction for connected mode. The followings are provided.

Proposal 1: RAN4 can start the work on CSSF optimization with the baseline assumption in WID, i.e., 2 searchers.

Proposal 2：RAN4 to discuss the options for CSSF optimization in priority order:

1) Share the searcher dedicated for PCC/PSCC to other CCs.

2) Reduce the counted number of CCs with associated SSB due to CSI-RS L3 measurement.

3) Extent the scaling factor from Carrier Specific to Band Specific.

Reference

- R4-2415656, WF for NR_RRM_Ph5_Part1, Apple