# 3GPP TSG-RAN WG4 Meeting #113	R4-2418775

# Orlando, US, 18th – 22nd November, 2024

Title: 	Discussion on event triggered L1 measurement reporting

Source: 	OPPO

Agenda item:	7.24.2.1

Document for:	Discussion

Introduction

In last meeting, a WF was approved on RRM requirements for R19 mobility[1]. The RRM impact due to event triggered L1 measurement reporting could include the followings:

- Capabilities for Support of Event Triggering and Reporting Criteria

- Event-triggered L1 reporting delay requirement

- The accuracy requirement for event-triggered L1 reporting delay requirement

- Other potential impact of event-triggered L1 reporting to other LTM requirements, e.g., known TCI state condition for early TCI state activation and cell switch

In this contribution, we provide our views on measurements related enhancements for LTM, including event trigger L1 measurement reporting.

Discussion

Event triggered L1 measurement reporting

- Issue 1-1: RRM scope of Event triggered L1 measurement reporting - Agreement: - RAN4 to define event-triggered L1 reporting requirement for SSB based and CSI-RS based measurements. - To start RAN4 discussion for SSB based L1 measurement firstly, and check how to apply/extend the agreements for SSB based L1 measurement to accommodate CSI-RS based L1 measurement. - From RAN4 RRM requirement perspective, for FR2-1, RRM requirements for multi-Rx based LTM L1 measurement is not considered in the WI scope. - RRM scope of Event triggered L1 measurement reporting - In addition to intra-frequency measurement and inter-frequency measurement without gap, support gap-based inter-frequency measurement for SSB-based L1 measurement. Further discuss for CSI-RS based L1 measurement.

Agreement From RAN1 perspective, there is no restriction with regards to the frequency location of CSI-RS used for L1-measurement

In our view, CSI-RS based intra-frequency and inter-frequency measurements should be supported since there is no restriction with regards to the frequency location of CSI-RS used for L1-measurement from RAN1 perspective. Whether to only define requirements for measurements without gaps is depending on another discussion in RAN4, e.g., by restricting the intra-f and inter-f CSI-RS resources within the active BWP.

Proposal 1: For RRM scope of Event triggered L1 measurement reporting, support both CSI-RS based L1 intra-frequency measurement and inter-frequency measurement.

The requirements of “Capabilities for Support of Event Triggering and Reporting Criteria” specified in TS38.133 section 9.1.4 need to be updated to cover event triggered L1 measurement reporting. We support to keep the same value of Ecat for intra-frequency and inter-frequency measurement category, and update the note for each of events description.

| Measurement category | Ecat | Note |
| --- | --- | --- |
| Intra-frequency Note 1,2,3,4,5 | 9 | Events for any one or a combination of intra-frequency SS-RSRP, SSB based L1-RSRP, CSI-RS based L1 RSRP, SS-RSRQ, SS-SINR, CSI-RSRP, CSI-RSRQ, and CSI-SINR for NG-RAN intra-frequency cells |
| Inter-frequency Note 2,3,4,5 | 10 | Events for any one or a combination of inter-frequency SS-RSRP, SSB based L1-RSRP, CSI-RS based L1 RSRP, SS-RSRQ, SS-SINR, CSI-RSRP, CSI-RSRQ, and CSI-SINR for NG-RAN inter-frequency cells |

Proposal 2：RAN4 to update intra-frequency and inter-frequency measurement reporting criteria requirements for SSB and CSI-RS based L1-RSRP with the same Ecat.

- Issue 1-9: L1 event triggered reporting delay - Candidate solutions: - Option 1: The existing framework in L3 event triggered reporting RRM requirements can be as a starting point. (CTC, Samsung, ZTE) - Option 2: RAN4 to discuss how to consider the total measurement time if multiple cells are measured with different measurement period. (Xiaomi) - It’s possible that the periodicity of RS is different from each other and then the measurement period is different. How to define total measurement time for multiple cells is FFS. - Option 3: (CMCC) - for SSB based event triggered L1 measurement reporting, it is proposed to define L1 measurement reporting delay requirements. - The delay is less than TL1-RSRP_Measurement_Period or TL1-RSRP_Measurement_Perioa + TSSB_time_index, which are the L1-RSRP measurement period defined for Rel-18 LTM. - for CSI-RS based event triggered L1 measurement reporting, it is proposed to define L1 measurement reporting delay requirements, and L1 measurement reporting accuracy requirements - Option 4: Reporting delay is defined as one L1-RSRP measurement period on target cell. (Apple) - Option 4a: The event triggered measurement reporting delay, shall be less than LTM L1-RSRP measurement period based on SSB or CSI-RS. (Samsung) - Option 5: SSB-based LTM L1 event triggered reporting delay is TL1-RSRP_Measurement_Period_SSB_intra, TL1-RSRP_Measurement_Period_SSB_inter or TL1-RSRP_Measurement_Period_SSB_inter_withoutGap (Nokia) - Option 6: RAN4 to discuss and define the event evaluation time (i.e., the minimum time required to complete the event evaluation and be ready to send on the first reporting occasion from the measurement occasion). (Ericsson) - Issue 1-10: where to capture event triggered L1 report requirement - Candidate solutions: - Option 1: new requirements for event triggered L1 measurement reporting shall be introduced in 9.14.3 and 9.15.3, respectively for intra-frequency and inter-frequency. (CATT, Apple, vivo, HW)

About requirements for event triggered L1 measurement reporting, we support to introduce new clauses under sections 9.14.3 and 9.15.3 for intra-frequency and inter-frequency respectively.

About L1 event triggered reporting delay, current L3 event-triggered Reporting can be baseline.

- The measurement reporting delay is defined as the time between an event that will trigger a measurement report and the point when the UE starts to transmit the measurement report over the air interface.

- This requirement assumes that the measurement report is not delayed by other RRC signalling on the DCCH.

- This measurement reporting delay excludes a delay uncertainty resulted when inserting the measurement report to the TTI of the uplink DCCH. The delay uncertainty is: 2 × TTIDCCH.

- The event triggered measurement reporting delay shall be within TL1-RSRP_Measurement_Perioa  or Tidentify_inter_with_index.

The event triggered measurement reporting delay can be defined as the L1-RSRP measurement period.

Proposal 3: Reuse the definition of current L3 event-triggered Reporting requirements, and the L1 event triggered measurement reporting delay shall be within TL1-RSRP_Measurement_Perioa  or Tidentify_inter_with_index.

Proposal 4: Support to introduce new clauses under sections 9.14.3 and 9.15.3 for intra-frequency and inter-frequency respectively.

Conclusion

In the contribution we provide the following proposals:

Proposal 1: For RRM scope of Event triggered L1 measurement reporting, support both CSI-RS based L1 intra-frequency measurement and inter-frequency measurement.

Proposal 2：RAN4 to update intra-frequency and inter-frequency measurement reporting criteria requirements for SSB and CSI-RS based L1-RSRP with the same Ecat.

Proposal 3: Reuse the definition of current L3 event-triggered Reporting requirements, and the L1 event triggered measurement reporting delay shall be within TL1-RSRP_Measurement_Perioa  or Tidentify_inter_with_index.

Proposal 4: Support to introduce new clauses under sections 9.14.3 and 9.15.3 for intra-frequency and inter-frequency respectively.

Reference

- R4-24116862, WF on NR mobility enhancements Phase 4_part1, Apple