# 3GPP TSG-RAN WG4 Meeting #113	R4-2418776

# Orlando, US, 18th – 22nd November, 2024

Title: 	Discussion on CSI-RS based L1 measurements

Source: 	OPPO

Agenda item:	7.24.2.2

Document for:	Discussion

Introduction

In last meeting, a WF was approved on RRM requirements for R19 mobility[1]. RAN4 has agreed to consider the following requirements need to be discussed for CSI-RS based L1 RSRP measurement on LTM candidate cell(s):

-	Measurement delay

-	Measurement reporting requirements

-	Measurement restrictions

-	Scheduling availability

-	FFS on other potential impact.

In this contribution, we provide our views on CSI-RS based L1 measurements.

Discussion

CSI-RS based L1 measurements

- Issue 2-1: CSI-RS based L1 measurement for known/unknown cell Agreement: - RRM requirements for CSI-RS based L1-RSRP measurement only apply for known target cells - Issue 2-2: known cell definition in CSI-RS based L1 measurement For CSI-RS based L1-RSRP measurement, the cell is considered as known if the following conditions are met: Agreement: - The UE has performed L3 measurement on the target cell during the last 5 seconds, and - Further check whether to consider SSB-based L3 measurement only - The SSB from the target cell configured for L3[/L1] measurement remains detectable according to the cell identification requirements in clause 9.2. - L1 measurement in the 2nd bullet is not needed for FR1. - Further discuss whether to keep or remove L1 measurement for FR2-1 in the 2nd bullet. - The CSI-RS from the target cell configured for L1 measurement remains [detectable/measurable]. - Further discuss whether to use detectable or measurable. - Further discuss the side condition

About the definition of intra-frequency and inter-frequency for CSI-RS based L1 measurement, we prefer to follow CSI-RS based L3 measurement. And due to the cell detection also relies on SSB, the CSI-RS from the target cell configured for L1 measurement remains “measurable”, which is more suitable than “detectable”.

Proposal 1: The CSI-RS from the target cell configured for L1 measurement remains “measurable”.

- Issue 2-4: whether SSB based L3 measurement is a pre-requisite of CSI-RS based L1 measurement Tentative Agreement during online session: further discuss the two alternatives Alternative 1: - RAN4 to consider the following measurement procedure for CSI-RS based LTM candidate cell measurements: - Step 1: SSB based L3 measurements [and reports] - Step 2: For FR2-1, SSB-based L1 measurements and reports on the same candidate cell - Further discuss whether Step 2 can be skipped under certain conditions. FFS the details of the conditions. - Note: Step 2 is removed for FR1. - Step 3: CSI-RS based L1 measurements on the same candidate cell - Note: The CSI-RS in Step 3 should be QCL’ed with the SSB in Step 1 or 2. - Note: For FR1, the CSI-RS in Step 3 should be QCL’ed with the SSB in Step 1. Alternative 2: - RAN4 to consider the following measurement procedure for CSI-RS based LTM candidate cell measurements: - Step 1: SSB based L3 measurements [and reports] - Step 3: CSI-RS based L1 measurements on the same candidate cell - Note: The CSI-RS in Step 3 should be QCL’ed with the SSB in Step 1.

In principle, RAN4 has common understanding that CSI-RS is configured with associated SSB and the cell detection is relying on SSB measurement. SSB based L3 measurements is a pre-requisite of CSI-RS based L1 measurement.

Since both L1 and L3 triggered LTM are already supported in R18, we think it is fair to support both SSB based L3 and/or L1 measurement remains detectable and UE is allowed to perform L1 and/or L3 report during the last 5 seconds.

For FR1, SSB-based L1 measurements and reports on the same candidate cell can be skipped, but not for FR2-1 due to L1 fine beam is different from L3 rough beam. We think L3 measurement report is completed in step 1, step 2 can be skipped

Proposal 2: RAN4 to consider the following measurement procedure for CSI-RS based LTM candidate cell measurements: 1) SSB-based L1 measurements and reports on the same candidate cell. 2) SSB based L1 or L3 measurement report needs to be performed on the same candidate cell for FR2-1. 3) CSI-RS based L1 measurements on the same candidate cell.

- Issue 2-3: CSI-RS based L1 measurement outside active BWP or with measurement gap - Candidate solutions - Option 1: Deprioritize CSI-RS based L1-RSRP measurement outside active BWP (or with measurement gap) in R19. (MTK, QC, vivo, OPPO, Nokia) - Option 1a: RAN4 can first discuss measurement within active BWP for CSI-RS based L1 measurement and wait for progress in other WG about whether measurement gap will be supported. (Xiaomi) - Option 1b: RAN4 to define intra-frequency CSI-RS measurement requirements first. Support inter-frequency L1-RSRP CSI-RS measurements without gap. FFS for inter-frequency with gap. (Nokia) - Option 2: RAN4 at least support the following measurement types for CSI-RS based L1 measurement. (CATT) - Without gap - With gap - Option 2a: for CSI-RS based LTM, it is proposed to consider intra-frequency measurement, inter-frequency measurement without gap and inter-frequency measurement with gap, which are aligned with Rel-18 SSB based LTM. (CMCC) - Option 2b: prioritize intra-frequency measurement without GAP, inter-frequency measurement with GAP. (Apple, Samsung, HW, OPPO, CATT) - Option 2c: CSI-RS based L1 measurements for candidate cells shall support - Measurement within active BWP, Measurements outside active BWP. (E///, ZTE)

About the measurement types, we also agree to support intra-frequency without gap and inter-frequency with gap. If companies are ok to deprioritize CSI-RS based L1-RSRP measurement outside active BWP, we are also supportive. But it will impact the definition of intra-frequency measurement which cannot base on the center frequency of CSI-RS resource. Option 1 and 2b are ok to us. We further suggest to only define the requirements for CSI-RS based L1-RSRP measurement within active BWP and without gap.

Proposal 3: Support intra-frequency measurement and inter-frequency measurement, and deprioritize CSI-RS based L1-RSRP measurement outside active BWP (or with measurement gap) in R19.

Conclusion

In the contribution we provide the following proposals:

Proposal 1: The CSI-RS from the target cell configured for L1 measurement remains “measurable”.

Proposal 2: RAN4 to consider the following measurement procedure for CSI-RS based LTM candidate cell measurements: 1) SSB-based L1 measurements and reports on the same candidate cell. 2) SSB based L1 or L3 measurement report needs to be performed on the same candidate cell for FR2-1. 3) CSI-RS based L1 measurements on the same candidate cell.

Proposal 3: Support intra-frequency measurement and inter-frequency measurement, and deprioritize CSI-RS based L1-RSRP measurement outside active BWP (or with measurement gap) in R19.

Reference

- R4-24116862, WF on NR mobility enhancements Phase 4_part1, Apple