# 3GPP TSG-RAN WG4 Meeting #113	R4-2418778

Orlando, US, 18th – 22nd November, 2024

Title: 	Discussion on other RRM requirements of R19 NR NTN

Source: 	OPPO

Agenda item:	7.26.4.2

Document for:	Discussion

Introduction

In last RAN4 meeting, a WF on RRM requirements for Rel-19 NR NTN phase3 was approved [1], including downlink coverage enhancements, support of regenerative payload, and support of (e)RedCap UEs with NR FR1-NTN.

In this paper, we will provide our views on DL coverage enhancement.

Study and specify if beneficial downlink coverage enhancements targeting support for additional reference satellite payload parameters covering both GSO and NGSO constellations operating in FR1-NTN or FR2-NTN [RAN1, RAN2, RAN4] - Define additional reference satellite payload parameters assuming power sharing among satellite beams or different satellite beam patterns/size (i.e. wide or narrow) across the satellite footprint, such that satellite beams may not all be simultaneously active or may be active below the nominal EIRP density per satellite beam (see section 6.1.1 in TR 38.821) due to limited power and limited feeder link bandwidth. - Define the corresponding power sharing assumptions and necessary link level and system level evaluation methodology and relevant KPIs for evaluations of the coverage, to allow for identification of physical channels/signals and system-level aspects that need enhancements and the corresponding needed improvements. - Study and if needed specify solutions, including link level enhancements for FR1-NTN (e.g. for PDCCH, PDSCH) and/or system level enhancements for FR1-NTN and/or FR2-NTN, allowing dynamic and flexible power sharing between satellite beams or different satellite beam patterns/size (i.e. wide or narrow) across the satellite footprint. - RAN1 to report at the latest by RAN#106 with the list of targeted physical channels/signals for link level enhancements (if any), and with the targeted system-level enhancements (if any) - RAN1 should report on impact to backward compatibility, if any, for potential extension of the SSB periodicity at the latest by RAN#106, in conjunction with the targeted system-level enhancements. - Notes for this objective: - SSB channel enhancement other than SSB periodicity extension is not considered - RAN1 should consider issues such as UE’s cell search complexity and impact to initial cell selection, latency and success rate, for the above extension - The SSB periodicity enhancements potentially defined in this WID only apply to NTN operation - Antenna gain of UE shall be assumed to be -5.5dBi in case of smartphone in FR1-NTN, the UE is assumed to be a full duplex UE, and at least 2Rx are considered at the UE - NGSO to be considered in priority: LEO Set-1 @ 600 km - Rel-18 network energy saving techniques should be considered as baseline in the system level study

Discussion

### Issue 1-1: SSB periodicity enhancement Agreement: - RAN4 to wait for RAN1’s definitive decision on the maximum periodicity of SSB and the range of SMTC before discussing whether and how to modify the relevant RRM requirements. FFS on the detailed list of RRM requirements to be affected, e.g., - uplink timing requirements, RRM requirements during transmission between different SSB periodicities, default assumption on the SSB and SMTC periodicities in handover/satellite switch with re-sync/RRC re-establishment/RRC connection release with redirection, cell identification, measurements, etc. ### Issue 1-3: RRM impacts due to Satellite Tx Beam Hopping Agreement: - RAN4 to wait for more progress from RAN1/2 on the detailed solutions to support a satellite T/Rx beam sweeping pattern, e.g. to adopt the concept of Rel-18 cell DTX/DRX, before discussing whether and how to modify the relevant RRM requirements.

RAN4 discussed the impact of SSB periodicity extension (e.g., 320ms) in last meeting. IF RAN1/2 concluded on these, the related measurement requirements may need to be updated, e.g., timing, cell (re)-selection, L3 measurement/L1 measurement.

However, only few companies supported 320ms SSB periodicity in RAN4, and no consensus is reached for T/Rx beam sweeping. We think DTX/DRX, Dynamic and flexible power sharing could have limited impacts on RRM requirements of NTN. RAN4 can start the work after RAN1/2 reached more concrete conclusion. RAN4 can continuous to hold on these discussions in this meeting.

Proposal 1: RAN4 hold on the discussion on SSB periodicity extension and Satellite Tx Beam Hopping in RAN4#113 meeting.

Conclusions

In this paper we provided our views on other RRM requirements for R19 NTN.

Proposal 1: RAN4 hold on the discussion on SSB periodicity extension and Satellite Tx Beam Hopping in RAN4#113 meeting.

Reference

- R4-2416866, WF on RRM requirements for Rel-19 NR NTN phase3, Qualcomm