3GPP TSG-RAN WG4 Meeting #113	R4-2418788

# Orlando, US, 18th – 22nd November, 2024

Agenda Item:	5.22.2.2

Source: 	OPPO

Title: 	Discussion on RRM performance requirements for SL positioning

Document for:	Approval

1	Introduction

Based on the conclusions for SL positioning reached in the WF [1], this contribution will provide our consideration on the remaining issues.

2	Discussion

### Issue 1-2-5: Multiplexing of TX UEs in TCs Discuss further: - For 2 transmitting UEs, the time offset between the two SL-PRS configurations can be, e.g.: - MUX of multiple TX UEs: TX UE1: slot n, TX UE2: slot n + 1 and slot n + 160ms. - Time offset for 3 transmitting anchor UEs needs to be also decided. Come back next meeting.

SL-PRS-CommonProcCapabilityPerBand ::= SEQUENCE { maxSL-PRS-Bandwidth CHOICE { fr1 ENUMERATED {mhz5, mhz10, mhz20, mhz40, mhz50, mhz80, mhz100}, fr2 ENUMERATED {mhz50, mhz100, mhz200, mhz400} }, maxNumOfActiveSL-PRS-ResourcesInOneSlot CHOICE { fr1 ENUMERATED {n1, n2, n4, n6, n8, n12, n16, n24}, fr2 ENUMERATED {n1, n2, n4, n6, n8, n12, n16, n24, n32, n48, n64, n128} }, maxNumOfSlotsWithActiveSL-PRS-Resources CHOICE { fr1 ENUMERATED {n1, n2, n3, n4, n6, n8}, fr2 ENUMERATED {n1, n2, n4, n8, n12, n16, n24, n32, n48, n64} }, minTimeAfterEndofSlotCarryActiveSL-PRS-Resources ENUMERATED {ms20, ms30, ms40, ms50, ms80, ms100, ms160} }

minTimeAfterEndofSlotCarryActiveSL-PRS-Resources-r18 indicates the minimum time after the end of a slot carrying the active SL-PRS resource(s) assuming maximum number of symbols and maximum bandwidth for a UE to finish the SL-PRS resource and the associated PSCCH processing which is supported and reported by UE; ### NOTE: A SL PRS resource is considered as active starting at the end of the last symbol of the PSCCH carrying the SCI trigger and the occupancy is released at the end of timeline indicated minTimeAfterEndofSlotCarryActiveSL-PRS-Resources-r18.

When designing test case for SL positioning, the time offset between SL-PRS resources from 2 or 3 transmitting anchor UEs are not determined yet. One proposal during the last meeting is to take the maximum value of SL-PRS processing time as indicated via minTimeAfterEndofSlotCarryActiveSL-PRS-Resources-r18 as the time offset. We generally agree to consider such the processing time. However, as shown above, this processing time is defined after the end of SL-PRS slot. To guarantee enough time for SL-PRS processing, one more slot should be added in the time offset SL-PRS resources.

Proposal 1: The time offset between SL-PRS resources from multiple Tx anchor UEs should be: slot n for Tx UE 1, slot n+1+Tproc for Tx UE 2, and slot n+2+2*Tproc for Tx UE 3.

The value of Tproc in proposal 1 is related to the candidate values of SL-PRS processing time as indicated via minTimeAfterEndofSlotCarryActiveSL-PRS-Resources-r18. The proposed 160ms is too large in our view and will results in a long test delay. On the other hand, an additional time margin Y=32Tc is considered in the SL RSTD accuracy requirements, provided that the time offset between the two SL-PRS resource instances for a single SL RSTD estimation is no greater than 160ms. If Tproc = 160ms, no accuracy requirements will apply and the RSTD accuracy requirements cannot be tested. Therefore, a smaller value, e.g. 100ms should be considered.

Observation 1: The SL RSTD accuracy requirements apply provided that the time offset between the two SL-PRS resources for a single SL RSTD estimation is no greater than 160ms. Therefore, Tproc should be smaller than 160ms.

Proposal 2: Tproc in proposal 1 should be 100ms.

If a smaller value is agreed for Tproc, the UEs with lower capacities cannot be tested. To address this issue, an applicable rule could be added for SL test cases, e.g. “This test is applicable for UEs with capacities minTimeAfterEndofSlotCarryActiveSL-PRS-Resources-r18 <= Tproc or maxNumOfSlotsWithActiveSL-PRS-Resources > n1”

Proposal 3: Add the applicable rule for SL test cases, e.g. “This test is applicable for UEs with capacities minTimeAfterEndofSlotCarryActiveSL-PRS-Resources-r18 <= Tproc or maxNumOfSlotsWithActiveSL-PRS-Resources > n1”.

### Issue 1-2-1: Report mapping (RAN4 #107 meeting) Agreements: - Reuse the existing report mapping for SL positioning: - The report mapping for SL UE Rx-Tx is the same as for UE Rx-Tx. - The report mapping for SL RSTD is the same as for RSTD. - The report mapping for SL RTOA is the same as for UL-RTOA.

In previous meeting, it was agreed that the reporting mapping for SL RTOA is the same as that for UL-RTOA, which supports different timing resolution k = {0, 1, 2, 3, 4, 5}. However, UL-RTOA is estimated by gNB while SL RTOA is estimated by normal SL UE. The finer timing resolutions like k = {0, 1} cannot be achieved for SL RTOA in FR1, following the same principle as SL RSTD and SL UE Rx-Tx. Therefore, we propose to remove k = {0, 1} from the reporting mapping for SL RTOA and the corresponding chances are provided in our draft CR [2].

Proposal 4: Remove timing resolutions k = {0, 1} from the reporting mapping for SL RTOA.

3	Conclusion

This contribution discussed the remaining issues for SL positioning and gave the following proposals:

Proposal 1: The time offset between SL-PRS resources from multiple Tx anchor UEs should be: slot n for Tx UE 1, slot n+1+Tproc for Tx UE 2, and slot n+2+2*Tproc for Tx UE 3 if needed.

Observation 1: The SL RSTD accuracy requirements apply provided that the time offset between the two SL-PRS resources for a single SL RSTD estimation is no greater than 160ms. Therefore, Tproc should be smaller than 160ms.

Proposal 2: Tproc in proposal 1 should be 100ms.

Proposal 3: Add the applicable rule for SL test cases, e.g. “This test is applicable for UEs with capacities minTimeAfterEndofSlotCarryActiveSL-PRS-Resources-r18 <= Tproc or maxNumOfSlotsWithActiveSL-PRS-Resources > n1”.

Proposal 4: Remove timing resolutions k = {0, 1} from the reporting mapping for SL RTOA.

4	Reference

- R4-2416870, WF on RRM requirements for NR_pos_enh2_Part2, CATT

- R4-2418787, Draft CR on RRM perf requirements for SL positioning, OPPO