3GPP TSG-RAN WG4 Meeting #113	R4-2418793

Orlando, US, November 18th – 22nd 2024

Agenda Item:	7.20.2.3

Source:	Ericsson

Title:	Impact on SBFD BS RF RX requirements

Document for:	Approval

# 1	Introduction

In this contribution, we present overview of impact on existing BS RF RX requirements for SBFD slots. For convenience, the content in Clause 10.1 in TR 38.858 (V18.0.0) [1] was copied to the tables under each requirement for the purpose of making observations and proposals.

Discussion on existing BS RF TX requirements for SBFD slots, potentially new requirements for SBFD slots, and SBFD general aspects are presented in separate submissions [2-4].

# 2	Co-existence simulation for in-band blocking

2.1	Simulation assumption

## 1.6. In-band blocking Agreement: - For FR1 local area SBFD BS, re-use existing FR1 local area BS in-band blocking power requirements. WF: - FFS on the followings: - For FR1 WA BS, RAN4 need to discuss whether a unified in-band blocking requirement is defined? - Option 1: a unified blocking requirement to consider both scenarios: coexistence with legacy TDD system in adjacent channel, and coexistence with new SBFD system in adjacent channel. - Option 2: separate requirements are defined, - One set of requirements for coexisting with legacy TDD system in adjacent channel - One set of requirements for coexisting with new SBFD system in adjacent channel - For FR1 WA BS co-existence study, FFS grid-shift values should be considered. - Option 1: 10% - Option 2: 20% - Option 3: 50% - Option 4: 100% - In-band blocking requirement for FR1 MR SBFD BS - In-band blocking requirement for FR2-1 WA and MR SBFD BS - For FR2 local area SBFD BS, re-use existing FR2 local area BS in-band blocking power requirements.

In RAN4#112bis meeting, it was agreed that for FR1 local area SBFD BS, re-use existing FR1 local area BS in-band blocking power requirements. However, there are still some open issues that require further study.

About simulation assumptions for further study:

- About different grid-shift values, we believe the low grid-shift is a reality and thus the lowest grid-shift scenario should be considered.

The worst-case scenario is a grid-shift as small as possible, to account for uncoordinated deployments. We propose to consider 5%. As comparison, the results for 10% and 100% are also provided.

Here we would like to recall why 100% grid-shift was defined. The 100% grid-shift was originally considered during TDD-TDD co-existence simulations for UE, not because it was a feasible deployment, but because it was a worst-case scenario, where a UE might be close to another operator’s BS but far from its own, making it an extreme case, and it sets the most stringent requirements for TDD-TDD UE coexistence, ensuring robustness. However, in the current scenario for BS coexistence, 100% grid-shift is the best case and is unrealistic and impractical to deploy. Therefore, we propose to consider the worst case 5% grid-shift for in-band blocking simulation, to maintain a functional ecosystem​.

The worst-case scenario should be considered when defining in-band blocking requirements, i.e., grid-shift should be as small as possible, and 5% grid-shift is used.

- About how to use the power level probability distributions to derive the in-band blocking levels:

Find the X% tile of the UL SBFD wideband received power CDF to define the in-band blocking requirement in each of the down-selected scenarios. We propose X to be 99.

Find the 99% tile of the UL SBFD wideband received power CDF to define the in-band blocking requirement in each of the down-selected scenarios.

In the following sections, our simulation results for FR1 and FR2, as well as the discussion and proposals based on the results, are provided.

2.2	Discussion of simulation results for FR1

In this section we focus on discussing simulation results that have been obtained following the procedure described in section 2, and with focus on FR1 scenarios: scenarios 1 (urban macro) and 5 (micro). The simulation assumptions strictly follow those agreed for the coexistence simulation study conducted during the SI phase and captured in TR 38.858 [1], and are also summarized in the TR Annex.

2.2.1 FR1 Urban Macro

We consider a coexistence scenario where FR1 urban macro deployments of two different operators are deployed in the same area and in adjacent channels. The first operator (victim) deploys a SBFD network, while the second operator (aggressor) deploys a TDD network. We are interested in evaluating the wideband interference power received by the UL of the SBFD operator from the DL of the TDD operator.

Three different values of grid-shift are considered 1) a completely coordinated case with grid-shift 100% and 2) worst case scenarios where BSs of the two operators are very close to each other with grid-shift 10% and 5%. The grid-shift examples are depicted in Figure 2.2.1-1.

Figure 2.2.1-1: Grid-shift 10% and 100% between two UMa network deployments

Two different values of BS TRP are considered 1) 49 dBm, which is the default value in the assumptions of the SI; 2) 53 dBm, which is the second option considered among the assumptions of the SI.

The wideband interference power received by the UL of SBFD is depicted in Figure 2.2.1-2.

Figure 2.2.1-2: FR1 UMa: Received UL wideband power from DL of TDD network 2

The 99%-tile of the CDF of the received UL wideband power represents the in-band blocking requirement that should be defined. From Figure 2.2.1-2 it can be observed that the in-band blocking requirement is higher as the TRP increases or the grid-shift decreases. In addition, it can be observed that when considering TRP 49 dBm and grid-shift 100% (best case assumptions), we are already around 20 dB above the currently specified in-band blocking requirement (-43 dBm) for Wide Area BS.

- Considering TRP 53 dBm:

- When grid-shift is 5%, in-band blocking interferer signal power is 4.6 dBm;

- When grid-shift is 10%, in-band blocking interferer signal power is -0.5 dBm;

- When grid-shift is 100%, in-band blocking interferer signal power is -20.4 dBm

- Considering TRP 49 dBm:

- When grid-shift is 5%, in-band blocking interferer signal power is 1.3 dBm;

- When grid-shift is 10%, in-band blocking interferer signal power is -4.7 dBm;

- When grid-shift is 100%, in-band blocking interferer signal power is -23.4 dBm.

- The higher the TRP of the aggressor network BSs, the higher the needed in-band blocking requirement for the SBFD network. Note that 53 dBm TRP is quite common.

- The lower the grid-shift between the two network requirements, the higher the needed in-band blocking requirement for the SBFD network.

- For FR1 urban macro deployments, stricter in-band blocking requirements would be needed for a SBFD network to face the wideband interference received from the DL of adjacent channel deployments.

Define in-band blocking interferer signal power to 4.6 dBm for FR1 Wide Area BSs in SBFD networks, which applies for both DUD and DU/UD configurations.

2.2.2 FR1 Urban Micro

We consider a coexistence scenario where FR1 urban micro deployments of two different operators are deployed in the same area and in adjacent channels. The first operator (victim) deploys a SBFD network, while the second operator (aggressor) deploys a TDD network. We are interested in evaluating the wideband interference power received by the UL of the SBFD operator from the DL of the TDD operator.

Three different values of grid-shift are considered 1) a completely coordinated case with grid-shift 100% and 2) worst case scenarios where BSs of the two operators are very close to each other with grid-shift 5% and 10%. The grid-shift examples are depicted in Figure 2.2.2-1.

Figure 2.2.2-1: Grid-shift 10% and 100% between two UMi network deployments

Two different values of BS TRP are considered 1) 46 dBm, which is the default value in the assumptions of the SI; 2) 38 dBm, which is the second option considered among the assumptions of the SI.

The wideband interference power received by the UL of SBFD is depicted in Figure 2.2.2-2.

Figure 2.2.2-2: FR1 UMi: Received UL wideband power from DL of TDD network 2

The 99%-tile of the CDF of the received UL wideband power represents the in-band blocking requirement that should be defined. From Figure 2.2.2-2 it can be observed that the in-band blocking requirement is higher as the TRP increases or the grid-shift decreases.

Current in-band blocking level for Medium Range BS is -38 dBm, so we can observe that when we consider lower power levels and coordinated deployments (100% grid-shift), there would not be need for higher in-band blocking requirements. However, when the transmission power is higher or the grid-shift is reduced, more stringent requirements would be needed.

- Considering TRP 46 dBm:

- When grid-shift is 5%, in-band blocking interferer signal power is -1.7 dBm;

- When grid-shift is 10%, in-band blocking interferer signal power is -7.5 dBm;

- When grid-shift is 100%, in-band blocking interferer signal power is -31.5 dBm.

- Considering TRP 38 dBm:

- When grid-shift is 5%, in-band blocking interferer signal power is -9.3 dBm;

- When grid-shift is 10%, in-band blocking interferer signal power is -15.5 dBm;

- When grid-shift is 100%, in-band blocking interferer signal power is -39.4 dBm.

- In FR1 UMi deployments, with mid-power BSs, the need for more stringent in-band blocking requirements is deployment dependent. Coordinated deployments operating in lower power level may not need an update in requirements, but uncoordinated deployments may need it.

Define in-band blocking interferer signal power to -1.7 dBm for FR1 Medium Range BSs in SBFD networks, which applies for both DUD and DU/UD configurations.

2.3	Discussion of simulation results for FR2-1

In this section we focus on discussing simulation results that have been obtained following the procedure described in section 2, and with focus on FR2-1 urban macro and indoor office scenarios. The simulation assumptions strictly follow those agreed for the coexistence simulation study conducted during the SI phase and captured in TR 38.858, and are also summarized in the TR Annex.

2.3.1 FR2-1 Urban Macro

We consider a coexistence scenario where FR2-1 urban macro deployments of two different operators are deployed in the same area and in adjacent channels. The first operator (victim) deploys a SBFD network, while the second operator (aggressor) deploys a TDD network. We are interested in evaluating the wideband interference power received by the UL of the SBFD operator from the DL of the TDD operator.

Three different values of grid-shift are considered 1) a completely coordinated case with grid-shift 100% and 2) worst case scenarios where BSs of the two operators are very close to each other with grid-shift 10% and 5%.

Two different values of BS TRP are considered 1) 30 dBm, which is the default value in the assumptions of the SI; 2) 40 dBm, which is the second option considered among the assumptions of the SI.

The wideband interference power received by the UL of SBFD is depicted in Figure 2.3.1-1.

Figure 2.3.1-1: FR2-1 UMa: Received UL wideband power from DL of TDD network 2

The 99%-tile of the CDF of the received UL wideband power represents the in-band blocking requirement that should be defined. From Figure 2.3.1-1 it can be observed that the in-band blocking requirement is higher as the TRP increases and the grid-shift decreases. It can be observed that when considering TRP 40 dBm and grid-shift 5%, we are around 35 dB above the currently specified in-band blocking requirement (-58 dBm) for FR2-1 Wide Area BS.

- The higher the TRP of the aggressor network BSs, the higher the needed in-band blocking requirement for the SBFD network.

- The lower the grid-shift between the two network requirements, the higher the needed in-band blocking requirement for the SBFD network.

- For urban macro deployments, stricter in-band blocking requirements would be needed for a SBFD network to face the wideband interference received from the DL of adjacent channel deployments.

Define in-band blocking interferer signal power to -23 dBm for FR2-1 Wide Area BSs in SBFD networks.

2.3.2 FR2-1 Indoor office

We consider a coexistence scenario where FR2-1 indoor office deployments of two different operators are deployed in the same building and in adjacent channels, as shown in Figure 2.2.3-1. The first operator (victim) deploys a SBFD network, while the second operator (aggressor) deploys a TDD network. We are interested in evaluating the wideband interference power received by the UL of the SBFD operator from the DL of the TDD operator.

The indoor scenario facilitates coordination of the two operators, compared to outdoor deployments, so that it is inherently coordinated with ISD= 20 mt. The BS transmission power is fixed at 24 dBm. The maximum UE power is set to 23 dBm, so that there is not a big difference between UEs and BS in terms of potential in-band blocking level. This results in that the current requirement is still valid. Figure 2.3.2-1 shows the UL received wideband power from the neighbor operator. For comparison we consider the case where the victim is the UL of SBFD, and the in-band blocking level is represented by the TDD BSs of the neighbor operator transmitting DL, and the synchronized case where the victim is a TDD deployment so that the blockers are the UEs. It can be observed that both the curves are below the current in-band blocking level of -58 dBm for Local Area BSs. This means there is no need to modify the in-band blocking level for this BS class for SBFD networks.

Figure 2.3.2-1: FR2-1 InH: Received UL wideband power from DL of TDD network 2

- In FR2-1 InH deployments, with low-power BSs, the power level of the BSs (24 dBm) is comparable to the maximum power level of the UEs (23 dBm), so that the fact that the BSs are blockers for an SBFD deployment, does not introduce a need for more stringent requirements for this BS class.

- FR2-1 InH deployments are easier to coordinate, so that certain distance between BSs of different networks can be assumed, and this also facilitates that there is no need for more stringent requirements.

Re-use the existing in-band blocking requirement for FR2-1 Local Area BSs in SBFD networks.

# 3	Impact on BS RX requirements

Reference sensitivity level and OTA sensitivity

WF: - FFS on the followings: - Option 1: 1.0 dB degradation - Option 2: 0.5 dB degradation

It has been agreed that an OTA sensitivity requirement, based on sensitivity in the RX sub-band with the transmitter turned on in the TX sub-bands is needed for SBFD. Some degradation compared to the sensitivity requirement in non-SBFD slots should be allowed for, where it is important to consider that all sources of interference, i.e., self-interference, inter-sector interference, inter-site interference and interference from other operators, will cause the sensitivity of the receiver to degrade. An open issue for discussion is the value of degradation. The limiting factor can be IM3 in BS receiver, especially for wide area deployments, since the degradation of the receiver due to IM3 can worsen by many more dBs rapidly.

For example, if the gNB self-interference degradation is 1 dB (1.25 in linear with the corresponding self-interference relative to noise being 0.25) and the network can be designed such that the sensitivity degradation from other sector and site interferences (without self-interference) would also be 1 dB (1.25 in linear with the corresponding other interference relative to noise being 0.25), then the total sensitivity degradation for a real network experiencing both self-interference and other interferences (from other sectors and sites) would be 1.8 dB (1.5 in linear with the corresponding total interference relative to noise being 0.5) if the interferences would just be additive and uncorrelated. If the receiver is IM3 limited, considering the interference at the input to the LNA is doubled, the interference at the output increases by 2 ^ 3, i.e. 8 times, then the total interference becomes 8*0.25 = 2. Hence, the ratio of (interference + noise) / noise becomes (2+1)/1 = 3 in linear, or 4.8 dB. These two numbers represent lower and upper boundaries, so the real total sensitivity degradation would be somewhere between 1.8 dB and 4.8 dB for such a deployment.

If the self-interference degradation is 0.5 dB and the sites can be designed such that the degradation from other sectors and sites without self-interference would also be 0.5 dB then the desensitization in a real network with both sources present would be between 0.95 dB and 3.2 dB.

Thus, if the desensitization due to self-interference alone is 1 dB then in a real deployment, even with highly isolated sites, it would not be possible to keep the desensitization less than 2 - 4 dB.

- OTA sensitivity should be defined considering in real life the receiver is further desensitized by other sources of interference including inter-site interference and inter-sector interference.

- RAN4 requirements should be conservative enough that the SBFD BS can be expected to perform well in real deployments considering inter-site interference and inter-sector interference.

- Since the receiver performance is limited by receiver non-linearity, the sensitivity degradation is much greater than just the sum of the sensitivity degradation from each source alone.

For SBFD-capable BS OTA sensitivity requirement, select 0.5 dB as the maximum degradation value.

Dynamic range

## 1.4. Dynamic range WF: - FFS on the followings: - Option 1: RAN4 to discuss if the IoT level and wanted signal power level will be derived based on simulation work. - Option 2: The existing dynamic range requirement, RAN4 assume 20dB interference over thermal noise, which is enough to cover the co-channel interference from other base stations. - Option 3: Dynamic range of FR1/FR2-1 indoor scenario can reuse the existing requirement.

In normal uplink, the IoT depends on the expected interference from other UEs. While for SBFD, it actually depends on the interference from other base stations. The IoT level in SBFD slots differs because the in-band power is due to other base stations, which are not doing SBFD. If all base stations are doing SBFD, then the IoT level in SBFD slots is the same as in normal UL slots.

- RX dynamic range requirement is applicable for SBFD-capable BS. The IoT level in SBFD slots differs because the in-band power is due to other base stations, which are not doing SBFD. If all base stations are doing SBFD, then the IoT level in SBFD slots is the same as in normal UL slots.

In-band selectivity and blocking

Other operators BSs may be co-located with the SBFD BS or may be at some distance. There is currently no requirement for supporting co-location of non-synchronized operators in the same band. The nearest requirement is the FDD protection of own receiver requirement. A co-location blocking requirement could be created for SBFD and declaration based; declaring compliance to the requirement would demonstrate sufficient RX performance to enable co-location. However, the requirement would not necessarily ensure that emissions from the co-located BS would not desensitize the SBFD receiver.

- The BS co-location requirements can be applied as they are, with a declaration whether the SBFD BS meets the co-location requirements.

For non-co-located blocking, dedicated co-existence simulations were conducted to determine the expected blocker levels due to other operator site BS during SBFD slots. Results and discussion are presented in Section 2.

Moreover, it worth noting that, the definition of in-band blocking contains an offset, while the simulation results do not take the offset into account. However, the offset cannot be applied outside of both DUD and DU/UD configurations, which means that the proposed interferer level will be the ACS interferer level and needs to be applied directly outside the carrier supporting SBFD, as illustrated in Figure 3-1 and Figure 3-2.

Figure 3-1 Interferer applied outside of SBFD DU configuration

Figure 3-2 Interferer applied outside of SBFD DUD configuration

ACS

Agreement from RAN4#112bis: - The OTA sensitivity degradation should be taken into account for the baseline REFSENS for ACS requirement. Content from TR 38.858: Regarding ACS requirement and in-band blocking requirement, RAN4 reached the following consensus: - ACS requirement and the interference level shall be determined by RAN4 co-existence study, and for the definition of ACS requirement: - Conducted ACS: Take the existing wanted signal of ACS requirement by using the existing reference sensitivity level. - OTA ACS: The OTA sensitivity degradation shall be taken into account to determine the level of wanted signal and interference signal mean power.

Regarding ACS, there is one agreement reached in RAN4#112bis, as shown above. By looking at the content from TR 38.858 [1] regarding conducted ACS, we would like to further refine the agreement to the following wording, which is more precise:

The OTA sensitivity degradation should be taken into account for the baseline REFSENS for OTA ACS requirement and should not be taken into account when testing conducted ACS.

Out-of-band blocking

Regarding Out-of-band blocking requirement, the existing OOBB requirement is still applicable for SBFD-capable BS except for OTA sensitivity degradation with the power level of wanted signal taken into account.

- The OOB blocking requirement is the same in SBFD slots as for normal TDD.

Receiver spurious emissions

Regarding the receiver spurious emission requirement, apart from existing requirements for normal reception on UL symbols/slots, it’s not necessary to specify additional receiver spurious emissions requirement for SBFD operation in SBFD symbols/slots.

- The receiver spurious emissions requirement is the same for both SBFD and non-SBFD slots.

- The receiver spurious emission is only measurable with conducted testing and OTA testing with transmitter deactivated in SBFD slots.

Receiver intermodulation

## 1.7. Necessity of New RX intermodulation requirement with 1 interfering signal WF: - FFS on the followings: - Option 1: IMD requirement for single interfering signal scenario is not needed. Such a requirement is implicitly captured by the SBFD RX blocking requirement. - Option 2: RAN4 to study further RX intermodulation scenario with 1 interfering signal. ## 1.8. Necessity of New RX intermodulation requirement with 2 interfering signals WF: - FFS on the following: - Option 1: - For the receiver intermodulation requirements, BS2BS CLI should be taken into account for power level for interference signal. - For receiver intermodulation requirements, consider IMD between CW/NBB/general intermodulation interfering signal intermodulate with SBFD DL transmission with some performance degradation on SBFD receiver. - Option 2: New IMD requirement with 2 interfering signal scenario is not needed. - Option 3: RAN4 to study further

The RX intermodulation requirement for conventional TDD consists of a CW signal and a modulated signal that are located in frequency such that their intermodulation produce falls within the receiver frequency range. The RX intermodulation sets a minimum requirement on receiver linearity.

For SBFD, it is possible that a single signal can intermodulate in the receiver with the TX sub-band TX signal. It is necessary to supress such intermodulation sufficiently that the receiver is not desensitized. In this case, transmission in DL sub-band needs to be activated when testing RX IM, and the offsets are chosen in a way that intermodulation product between DL sub-band and interferer falls in the UL sub-band.

The scenarios for 2 input signals and 1 input signal + TX sub-band may need different levels of receiver linearity to be met, depending on the level of the TX sub-band signal in the receiver.

Since receiver linearity should meet both scenarios, in our view, the requirement should be set based on the tougher case.

As defined in Table 7.7.2-1 in TS 38.104 [5], the general intermodulation requirement for WA BS is -52 dBm, which is derived by the corresponding in-band blocking level (-43 dBm) – 9 dB. Hence, for scenario with two input signals, the interfering level for both signals should be equivalent to [the final agreed in-band blocking level] – 9 dB.

Table 7.7.2-1: General intermodulation requirement

| Base Station Type | Wanted Signal mean power (dBm) | Mean power of interfering signals (dBm) | Type of interfering signals |
| --- | --- | --- | --- |
| Wide Area BS | PREFSENS +6 dB | -52 |  |
| Medium Range BS | PREFSENS +6 dB | -47 | See Table 7.7.2-2 |
| Local Area BS | PREFSENS +6 dB | -44 |  |
| NOTE 1: PREFSENS depends on the RAT and the BS class. For NR, PREFSENS depends also on the BS channel bandwidth, see clause 7.2. For NB-IoT, PREFSENS depends also on the sub-carrier spacing as specified in tables 7.2.1-5, 7.2.1-5a and 7.2.1-5c of TS 36.104 [13]. |  |  |  |

Use FR1 WA as example for comparison of the two considered scenarios,

- considering the scenario of 2 inputs signals, the mean power of both CW and modulated interfering signals is [the final agreed in-band blocking level] – 9 dB.

- considering the scenario of 1 input signal + TX sub-band, the CW interfering level is [the final agreed in-band blocking level] – 9 dB, and the signal level of TX sub-band is -27 dBm, which equals to 53 dBm (TRP) – 80 dB (isolation).

As mentioned above, the in-band blocking level is proposed to be 4.6 dBm, which results that the power of the two input signals will be higher when comparing the two scenarios.

In addition, the signal level of TX sub-band is based on declaration, whereas RAN4 need to define a RX IMD requirement with fixed interfering signals. And for SBFD DU pattern, whether the intermodulated product will fall into the UL subband depends on the placement of the CW interfering signal. Therefore, the scenario for 2 input signals should be used when testing receiver intermodulation.

The scenario for 2 input signals can be regarded as the worst case when setting requirement on receiver linearity.

For RX IMD, the mean power of both CW and modulated interfering signals is [the final agreed in-band blocking level] – 9 dB.

# Conclusion

In this contribution, overview of impact on SBFD BS RF requirements is presented with the following observations and proposals.

Observation 1	Considering TRP 53 dBm:

When grid-shift is 5%, in-band blocking interferer signal power is 4.6 dBm;

When grid-shift is 10%, in-band blocking interferer signal power is -0.5 dBm;

When grid-shift is 100%, in-band blocking interferer signal power is -20.4 dBm

Observation 2	Considering TRP 49 dBm:

When grid-shift is 5%, in-band blocking interferer signal power is 1.3 dBm;

When grid-shift is 10%, in-band blocking interferer signal power is -4.7 dBm;

When grid-shift is 100%, in-band blocking interferer signal power is -23.4 dBm.

Observation 3	The higher the TRP of the aggressor network BSs, the higher the needed in-band blocking requirement for the SBFD network. Note that 53 dBm TRP is quite common.

Observation 4	The lower the grid-shift between the two network requirements, the higher the needed in-band blocking requirement for the SBFD network.

Observation 5	For FR1 urban macro deployments, stricter in-band blocking requirements would be needed for a SBFD network to face the wideband interference received from the DL of adjacent channel deployments.

Observation 6	Considering TRP 46 dBm:

When grid-shift is 5%, in-band blocking interferer signal power is -1.7 dBm;

When grid-shift is 10%, in-band blocking interferer signal power is -7.5 dBm;

When grid-shift is 100%, in-band blocking interferer signal power is -31.5 dBm.

Observation 7	Considering TRP 38 dBm:

When grid-shift is 5%, in-band blocking interferer signal power is -9.3 dBm;

When grid-shift is 10%, in-band blocking interferer signal power is -15.5 dBm;

When grid-shift is 100%, in-band blocking interferer signal power is -39.4 dBm.

Observation 8	In FR1 UMi deployments, with mid-power BSs, the need for more stringent in-band blocking requirements is deployment dependent. Coordinated deployments operating in lower power level may not need an update in requirements, but uncoordinated deployments may need it.

Observation 9	The higher the TRP of the aggressor network BSs, the higher the needed in-band blocking requirement for the SBFD network.

Observation 10	The lower the grid-shift between the two network requirements, the higher the needed in-band blocking requirement for the SBFD network.

Observation 11	For urban macro deployments, stricter in-band blocking requirements would be needed for a SBFD network to face the wideband interference received from the DL of adjacent channel deployments.

Observation 12	In FR2-1 InH deployments, with low-power BSs, the power level of the BSs (24 dBm) is comparable to the maximum power level of the UEs (23 dBm), so that the fact that the BSs are blockers for an SBFD deployment, does not introduce a need for more stringen requirements for this BS class.

Observation 13	FR2-1 InH deployments are easier to coordinate, so that certain distance between BSs of different networks can be assumed, and this also facilitates that there is no need for more stringent requirements.

Observation 14	OTA sensitivity should be defined considering in real life the receiver is further desensitized by other sources of interference including inter-site interference and inter-sector interference.

Observation 15	RAN4 requirements should be conservative enough that the SBFD BS can be expected to perform well in real deployments considering inter-site interference and inter-sector interference.

Observation 16	Since the receiver performance is limited by receiver non-linearity, the sensitivity degradation is much greater than just the sum of the sensitivity degradation from each source alone.

Observation 17	RX dynamic range requirement is applicable for SBFD-capable BS. The IoT level in SBFD slots differs because the in-band power is due to other base stations, which are not doing SBFD. If all base stations are doing SBFD, then the IoT level in SBFD slots is the same as in normal UL slots.

Observation 18	The BS co-location requirements can be applied as they are, with a declaration whether the SBFD BS meets the co-location requirements.

Observation 19	The OOB blocking requirement is the same in SBFD slots as for normal TDD.

Observation 20	The receiver spurious emissions requirement is the same for both SBFD and non-SBFD slots.

Observation 21	The receiver spurious emission is only measurable with conducted testing and OTA testing with transmitter deactivated in SBFD slots.

Based on the discussion in the previous sections we propose the following:

Proposal 1	The worst-case scenario should be considered when defining in-band blocking requirements, i.e., grid-shift should be as small as possible, and 5% grid-shift is used.

Proposal 2	Find the 99% tile of the UL SBFD wideband received power CDF to define the in-band blocking requirement in each of the down-selected scenarios.

Proposal 3	Define in-band blocking interferer signal power to 4.6 dBm for FR1 Wide Area BSs in SBFD networks, which applies for both DUD and DU/UD configurations.

Proposal 4	Define in-band blocking interferer signal power to -1.7 dBm for FR1 Medium Range BSs in SBFD networks, which applies for both DUD and DU/UD configurations.

Proposal 5	Define in-band blocking interferer signal power to -23 dBm for FR2-1 Wide Area BSs in SBFD networks.

Proposal 6	Re-use the existing in-band blocking requirement for FR2-1 Local Area BSs in SBFD networks.

Proposal 7	For SBFD-capable BS OTA sensitivity requirement, select 0.5 dB as the maximum degradation value.

Proposal 8	The OTA sensitivity degradation should be taken into account for the baseline REFSENS for OTA ACS requirement and should not be taken into account when testing conducted ACS.

Proposal 9	The scenario for 2 input signals can be regarded as the worst case when setting requirement on receiver linearity.

Proposal 10	For RX IMD, the mean power of both CW and modulated interfering signals is [the final agreed in-band blocking level] – 9 dB.

# References

TR 38.858, Study on evolution of NR duplex operation

R4-2418792, Impact on SBFD BS RF TX requirements, Ericsson

R4-2418791, Potentially new SBFD BS RF requirements, Ericsson

R4-2418790, Discussion on SBFD general aspects, Ericsson

TS 38.104, “NR; Base Station (BS) radio transmission and reception”, 3GPP