3GPP TSG RAN WG4#113	R4-2418798

Orlando, USA, 18th - 22nd November 2024

Agenda Item:	7.25.2

Source: 	Meta

Title:  	Discussion on RRM Impacts for XR

Document for:	Discussion and Decision

# Introduction

The Revised WID on XR (eXtended Reality) for NR Phase 3, one of the objectives is to enable transmission or reception in measurement gaps or scheduling restrictions [1]:

- Specify enhancements to enable transmission/reception in gaps/restrictions that are caused by RRM measurements (from inter-frequency RRM measurement gaps, or intra-frequency measurements, or other scheduling restrictions etc.). [RAN1, RAN2, RAN4] - Specify the corresponding measurement gap and scheduling restriction to enable the identified enhancements with RRM performance impact taken into consideration, work being triggered by LS. [RAN4]

In the RAN4#113-bis meeting, seven topics were discussed and the WF is in [4]. In this paper, we discuss our views on the UE assistance information related to measurement occasions.

# Discussion

Regarding the need and the feasibility of the UE assistance information, the WF is:

- Companies are tasked to discuss the need and feasibility of UAI, based on the following options

- Option 1: UAI is specified

  - Companies supporting this option are encouraged to bring contribution with exact proposals on what information is to be included in UAI.

- Option 2: UAI is not specified

UE assistance information plays a vital role in enabling the gNB to make well-informed decisions about activating transmissions/receptions for XR traffic during gaps or restrictions resulting from RRM measurements. The existing CSI reporting and measurement reporting mechanisms may not provide adequate information for the gNB to make optimal choices. By furnishing the gNB with UE-reported assistance information, it can gain valuable insights into the necessary conditions and requirements for enabling transmissions or receptions across gaps or restrictions, ultimately leading to better-informed decisions.

Observation 1: Incorporating UE assistance information related to skipping measurement occasions can be valuable in reducing RRM performance degradation.

In Rel-18, the UE assistance information was discussed in RAN2 and supported for XR. For enabling transmission/reception in gaps/restrictions that are caused by RRM measurements, UE assistance information related to measurement occasions should be considered.

Proposal 1: UE assistance information related to measurement occasions should be considered.

During the RAN4#113-bis meeting, some companies propose to consider semi-static solutions and the WF is:

- Option 1: In addition to the dynamic indication scheme to deactivate MGs as agreed by RAN1, support semi-static schemes such as RRC and MAC-CE based signaling as well. Send LS to RAN2 and RAN1.

- Option 2: RAN4 also suggest RAN1/2 to introduce the semi-static approach (Alt.3) that NW indicate the indicate occasions where to skip gaps/restrictions, and UE can have a stable measurement pattern/period.

- Option 3: To guarantee mobility performance, one potential improvement on dynamic indication by DCI is configuring gaps to be skipped during a time period by RRC in advance and gaps indicated by DCI should be subset of gaps configured by RRC

Since RAN4 is still evaluating the feasibility of the working assumption from the RAN1 LS on the DCI based solution, we think it is better to first confirm the working assumption or to identify issues before introducing alternative solutions.

Proposal 2: RAN4 confirms the RAN1 working assumption or identifies issues first, before introducing alternative solutions.

# Conclusion

In this paper, we discuss our views on the UE assistance information related to measurement occasions.

Observation 1: Incorporating UE assistance information related to skipping measurement occasions can be valuable in reducing RRM performance degradation.

Proposal 1: UE assistance information related to measurement occasions should be considered.

Proposal 2: RAN4 confirms the RAN1 working assumption or identifies issues first, before introducing alternative solutions.

# References

[1] RP-241771, “Revised WID on XR (eXtended Reality) for NR Phase 3” Melbourne, Australia, September 9-12, 2024

[2] TS 38.133, “Requirements for support of radio resource management”

[3] Chair’s notes RAN1#118 meeting

[4] R4-2416864, “WF on RRM requirements for XR_Ph3”, Nokia

[5] R1-2407512, “LS on gaps/restrictions that are caused by RRM measurements”

[6] R4-2411007, “LS on UE assistance information”