3GPP TSG-RAN WG4 Meeting #113	R4-2418804

# Orlando, United States,18 – 22 Nov., 2024

Source: 	Huawei, HiSilicon

Title:	On the RX RF requirements for Ambient IoT intermediate UE

Agenda Item:	7.21.3.3

Document for:	Endorsement

1	Introduction

The SID on solutions for Ambient IoT was revised in RAN#103 [1]. A WF on the RF requirements for Ambient IoT was agreed last meeting [2]. This contribution we discuss the open issues and propose a text proposal for TR 38.769 on Ambient IoT intermediate UE RX requirements.

2	Discussion

Based on the WF [2], for Ambient IoT intermediate UE, the RX TBD requirements include narrow band blocking and Spurious response.

Narrow band blocking can be discussed in WI phase.

Spurious response is a measure of the ability of the receiver to receive a wanted signal on its assigned channel frequency without exceeding a given degradation due to the presence of an unwanted CW interfering signal at any other frequency for which a response is obtained. The existing spurious emission requirement for legacy NR UE is applicable for intermediate UE.

Proposal 1: For intermediate UE RX requirements, we recommend:

| A-IoT intermediate UE- RX RF Requirement | Potentially needed or not |
| --- | --- |
| Narrow band blocking | Discussed in the WI phase |
| Spurious response | YES |

3	Conclusion

Based on the analysis in this contribution, we have the following proposals:

Proposal 1: For intermediate UE RX requirements, we recommend:

| A-IoT intermediate UE- RX RF Requirement | Potentially needed or not |
| --- | --- |
| Narrow band blocking | Discussed in the WI phase |
| Spurious response | YES |

4	References

- RP-240826, Revised SID: Study on solutions for Ambient IoT (Internet of Things) in NR, CMCC, Huawei, T-Mobile USA, RAN #103, Mar. ,2024

- R4-2417088, WF on impacts of A-IoT on RF requirements, Moderator (Huawei), RAN4#112bis, Oct.,2024

5	Text Proposal to TR 38.769

<Start of Text Proposal>

6.7.3	Intermediate node (UE)

6.7.3.1 TX

6.7.3.2 RX

- Reference sensitivity power level

The reference sensitivity power level REFSENS is the minimum mean power applied to each one of the intermediate UE antenna ports at certain performance metric. For Ambient IoT system, this metric may not be the throughput. Distinguished from the legacy NR UE, Ambient IoT intermediate UE may suffer from residual noise from CW interference, So it should be considered when defining receiver sensitivity.Reference sensitivity requirement for intermediate UE could be derived based on the following equation:

PREFSENS(dBm)=-174dBm+10*log10BW+NF+IM+ICW+SNR

Where:

-	BW is the maximum transmission bandwidth

-	NF is BS noise figure, equal to 10 dB for Medium Range BS and 13 dB for Local Area BS.

-	IM is the implementation margin, equal to 2dB.

-	Icw is IoT due the residual CW interference, value will be defined in the WI.

-	SNR is the SNR value for which we reach 10% BLER target. Each company provided simulation results, and average will be done for each BW.

- Maximum input power

Maximum input level for legacy NR UE is defined as the maximum mean power received at the UE antenna port.With the assumption that the intermediate UE is usually hardware sharing with the legacy NR UE, it may be reuse the requirements of legacy NR UE.

- Adjacent channel selectivity

Adjacent channel selectivity (ACS) is a measure of a receiver's ability to receive an NR signal at its assigned channel frequency in the presence of an adjacent channel signal at a given frequency offset from the centre frequency of the assigned channel.

ACS requirement and the interference level depend on co-existence study. The coexistence studies outcome shows that [the legacy ACS values as for NR FR1 UE are appropriate, and therefore reused].

- Blocking characteristics

The blocking characteristic is a measure of the receiver's ability to receive a wanted signal at its assigned channel frequency in the presence of an unwanted interferer on frequencies other than those of the spurious response or the adjacent channels, without this unwanted input signal causing a degradation of the performance of the receiver beyond a specified limit.

In-band blocking requirement and the interference level depend on co-existence study.

The existing out-of-band blocking requirement for legacy NR UE is applicable for intermediate UE.

Narrow band blocking can be discussed in WI phase.

- spurious response

Spurious response is a measure of the ability of the receiver to receive a wanted signal on its assigned channel frequency without exceeding a given degradation due to the presence of an unwanted CW interfering signal at any other frequency for which a response is obtained. The existing spurious emission requirement for legacy NR UE is applicable for intermediate UE.

- Receiver intermodulation

Intermodulation response rejection is a measure of the receiver's capability to receive a wanted signal on its assigned channel frequency in the presence of two or more interfering signals with a specific frequency relationship to the wanted signal. It depends on the RAN1 design. If multiple-tone carrier wave is supported, intermodulation requirements potentially needed.

- Spurious emissions

The spurious emissions power is the power of emissions generated or amplified in a receiver that appear at the UE antenna connector. The legacy NR UE receiver spurious emission requirement could be applicable.

A summary for RX part requirements is captured in the Table X.

| A-IoT intermediate UE- RX RF Requirement |  | Potentially needed or not |
| --- | --- | --- |
| Reference sensitivity |  | YES |
| Maximum input power |  | YES |
| Adjacent channel selectivity |  | YES |
| Blocking characteristics | In-band blocking | YES |
|  | Out-of-band blocking | YES |
|  | Narrow band blocking | [Discussed in the WI phase] |
| Spurious response |  | YES |
| Receiver intermodulation |  | YES |
| Spurious emissions |  | [YES] |

<End of Text Proposal>