3GPP TSG-RAN WG4 Meeting #113	R4-2418810

Orlando, USA, 18 – 22 November 2024

Source:	Ericsson

Title:	TP for TR 38.718-11-11: DC_68A_n8A

Agenda item:			6.2.2

Document for:	Approval

# 1. Introduction

This contribution is a text proposal for TR 37.719-11-11 to include DC_68A_n8A as defined in WID [1].

# ---Start of changes---

## 6.1	Inter-band DC within FR1

### 6.1.X	DC_68_n8

#### 6.1.X.1	Configuration for DC

Table 6.1.X.1-1:  Inter-band EN-DC configurations of 1 LTE band + 1 NR band

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | Single UL allowed |
| --- | --- | --- |
| DC_68A_n8A | DC_68A_n8A | No |

#### 6.1.X.2	Maximum output power for DC

Table 6.1.X.2-1: Maximum output power for inter-band EN-DC of 1 LTE band + 1 NR band

| DC configuration | Power class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| DC_68A_n8A | 23 | +2/-3 |

#### 6.1.X.3	Spurious emission band UE co-existence for DC

Table 6.1.X.3-1: Spurious emissions for inter-band EN-DC of 1 LTE band + 1 NR band

| E-UTRA and NR DC Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| DC_68_n8 | Frequency range | 470 | - | 694 | -42 | 8 | 5, 17 |
|  | Frequency range | 470 | - | 710 | -26.2 | 6 | 14 |
|  | Frequency range | 662 | - | 694 | -26.2 | 6 | 5 |
|  | Frequency range | 758 | - | 773 | -32 | 1 | 5 |
|  | Frequency range | 773 | - | 803 | -50 | 1 |  |
| NOTE 5: These requirements also apply for the frequency ranges that are less than FOOB (MHz) in Table 6.6.3.1-1, Table 6.6.3.1A-1 in TS 36.101 [4] or in Table 6.5.3.1-1 in TS 38.101-1 [2] from the edge of the channel bandwidth. NOTE 14: This requirement is applicable for 5 and 10 MHz E-UTRA or NR channel bandwidth allocated within 718-728MHz. For carriers of 10 MHz bandwidth, this requirement applies for an uplink transmission bandwidth less than or equal to 30 RB with RBstart > 1 and RBstart < 48. NOTE 17: This requirement is applicable in the case of a 10 MHz E-UTRA or NR carrier confined within 703 MHz and 733 MHz, otherwise the requirement of -25 dBm with a measurement bandwidth of 8 MHz applies. |  |  |  |  |  |  |  |

#### 6.1.X.4	Co-existence analysis for DC

Table 6.1.X.4-1 Band 68 and Band n8 UL/DL harmonics/harmonic mixing analysis

| UL/DL harmonics |  | 68 | UL12 | UL2 | UL33 | UL4 | UL5 | MSD type |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | flow | 698 | 1396 | 2094 | 2 792 | 3 490 |  |
| n8 | flow | fhigh | 728 | 1456 | 2184 | 2908 | 3 606 |  |
| DL1 | 925 | 960 | N/A | - | - | - | - | UL harmonic |
| DL23 | 1850 | 1920 | - | N/A | - | N/A | N/A | Harmonic mixing |
| DL34 | 2775 | 2880 | - | - | N/A | - | N/A |  |
| DL4 | 3700 | 3840 | - | N/A | N/A | N/A | N/A |  |
| DL54 | 4625 | 4800 | - | - | N/A | N/A | N/A |  |
| Analysis |  |  | No harmonic impacts |  |  |  |  |  |
| UL/DL harmonics |  | n8 | UL14 | UL2 | UL33 | UL4 | UL5 | MSD type |
|  |  | flow | 880 | 1760 | 2640 | 3250 | 4400 |  |
| 68 | flow | fhigh | 915 | 1830 | 2745 | 3660 | 4575 |  |
| DL1 | 753 | 783 | N/A | - | - | - | - | UL harmonic |
| DL23 | 1506 | 1566 | - | N/A | - | N/A | N/A | Harmonic mixing |
| DL34 | 2259 | 2349 | - | - | N/A | - | N/A |  |
| DL4 | 3012 | 3132 | - | N/A | N/A | N/A | N/A |  |
| DL54 | 3765 | 3915 | - | - | N/A | N/A | N/A |  |
| Analysis |  |  | No harmonic mixing impact |  |  |  |  |  |
| NOTE 1: ULx means UL xth harmonic frequency, and DLy means DL yth harmonic frequency range. NOTE 2: When a collision is detected with an overlap >0Hz between the ULx with DLy frequency ranges, the ULx/DLy cell is marked “D” for direct hit. When the gap between ULx and DLy frequency range is from 0Hz to x*MinULCBW, the ULx/DLy cell is marked “N” for Near miss. NOTE 3: UL3/DL2 harmonic mixing direct hit case for PC3/5 only applies for DL>3GHz. NOTE 4: For harmonic mixing, near-miss cases only apply for UL1 and odd DLy orders. |  |  |  |  |  |  |  |  |

Table 6.1.X.4-3: Band 68 and Band n8 UL IMD products

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 152 - 217 |  | 1578 - 1643 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 481 - 576 |  | 1032 - 1132 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 2276 - 2371 |  | 2458 - 2558 |  |
| Two-tone 4th order IMD products | ¦3*fx_low – 1*fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 1179 - 1304 |  | 1912 - 2047 |  |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ |  |  |
| IMD frequency limits (MHz) | 304 - 434 |  |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low + 1*fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 2974 - 3099 |  | 3338 - 3473 |  |
| Two-tone 4th order IMD products | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |  |  |
| IMD frequency limits (MHz) | 3156 - 3286 |  |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 2792 - 2962 |  | 1877 - 2032 |  |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 1184 - 1349 |  | 264 - 424 |  |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 4218 - 4388 |  | 3672 - 3827 |  |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 4036 - 4201 |  | 3854 - 4014 |  |
| NOTE 1: For each IMD item, when two bound values before taking absolute have different signs, the relevant IMD range shall be set such that (1) the lower bound is 0 and (2) the upper bound is the bigger value of the two after taking absolute. NOTE 2: The lowest even order and lowest odd order IMD MSDs shall be considered. |  |  |  |  |

Based on Table 6.1.X.4-3, there is no IMD issue for this combination.

#### 6.1.X.5	∆TIB and ∆RIB values

The TIB,c and RIB,c values are reused from DC_28_n8 as specified in TS 38.101-3.

Table 6.1.X.5-1: ΔTIB,c

| Inter-band DC configuration | ΔTIB,c for E-UTRA band / NR band (dB)1 |  |
| --- | --- | --- |
|  | Component band in order of bands in configuration2 |  |
| DC_68_n8 | 0.5 | 0.6 |
| NOTE 1: “-” denotes ΔTIB,c = 0. NOTE 2: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively. |  |  |

Table 6.1.X.5-2: ΔRIB,c

| Inter-band DC configuration | ΔRIB,c for NR band / E-UTRA band (dB)1 |  |
| --- | --- | --- |
|  | Component band in order of bands in configuration2 |  |
| DC_68_n8 | 0.1 | 0.2 |
| NOTE 1: “-” denotes ΔRIB,c = 0. NOTE 2: The component band order in the configuration should be listed by the order of NR band and E-UTRA band respectively. |  |  |

#### 6.1.X.6	Analysis of MSD requirements

Based on the co-existence studies for DC_68_n8, no MSD requirements are needed for this combination.

---End of changes---

# Reference

[1] 	 RP-241786, Rel-19 Dual connectivity (DC) of x LTE band(s), y NR band(s) (1<=x<6, 1<=y<6, x+y<=6) and single or two NR Supplementary Uplink (SUL) bands, Nokia