# 3GPP TSG-RAN WG4 Meeting #113                               R4-2418827

# Orlando, USA, 18th-22th Nov, 2024

# Source:        ZTE Corporation, Sanechips

# Title:           Discussion on BS RF requirements for LP-WUS/WUR

# Agenda Item:   7.23.3

# Document for:  Approval

- Introduction

After the discussion in RAN4#112bis meeting, there are still many controversial issues left. In this contribution, we continue to discuss the BS requirements for LP-WUS/LP-WUR.

- Discussion

## 2.1 Concept of LP-WUS Power Boosting

In last meeting, RAN4 discussed the definition of power boosting of LP-WUS and still did not achieve an agreement on this issue.

Issue 1-2: Concept of LP-WUS power boosting - Proposals - Proposal 1: Since EPRE ratio has the merit that power boosting declaration for LP-WUS does not vary with different CBWs, RAN4 to introduce and specify power boosting represented by EPRE ratio between LP-WUS and NR REs excluding LP-WUS. (CATT, Huawei, [vivo]) - The LP-WUS power boosting is the difference between the average power of LP-WUS REs (which occupy certain REs within a NR transmission bandwidth configuration and the average power NR REs (the NR carrier excluding the LP-WUS REs). - Proposal 2: Adopt following definition rather than introduce EPRE ratio. (Ericsson, ZTE, Nokia) - The LP-WUS RB power dynamic range (or LP-WUS power boosting) is the difference between the average power of LP-WUS REs (which occupy certain REs within a NR transmission bandwidth configuration) and the average power over all REs (from both LP-WUS and the NR carrier containing the LP-WUS REs). - WF - Compare the Pros and Cons of the two definitions - At least the following aspects should be considered - Convenience for manufacturer’s declaration - Compatibility with other issues relevant to power boosting, e.g. the possible upper bound, applicable lowest CBW, etc. - Cases of power boosting with different CBW and LP-WUS / NR RE allocation should be provided - Comparison of power boosting with NB-IoT with the same definition should be provided - Test aspects of power boosting should be considered - Other aspects are not precluded Try to define the power boosting concept next RAN4 meeting with one definition as baseline but allow flexibility to manufacturer declaration of boosting level for the supported CBW

The difference between dynamic range and EPRE ratio is shown in Fig 1. The main difference is that EPRE ratio is a relative value and dynamic range is an absolute value.

Fig. 1  The difference between dynamic range and EPRE ratio.

We understand that EPRE ratio is proposed for the purpose to declare power boosting level without considering channel bandwidth. However, there are some aspects need to be noticed as follows.

First, EPRE ratio has a premise that there are both LP-WUS RBs and NR RBs at the same time. When there is only LP-WUS transmitted, this definition does not work while dynamic range can work well no matter whether NR signal is transmitted. In Table 1, we show the impact on power boosting level for different CBWs with 46dBm rated carrier output power, which is shared by NR signal and LP-WUS, and there is no guard RBs configured between NR signal and LP-WUS.

Table 1  The impact of LP-WUS power boosting with different channel bandwidth

| Rated carrier output power (dBm) | CBW (MHz) | SCS (kHz) | N_RB | LP-WUS RB | LP-WUS power boosting (dB) | NR power deboosting (dB) | EPRE ratio (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 46 | 5 | 30 | 11 | 11 | 01 | Not Exist | Not Exist |
|  | 10 | 30 | 24 | 11 | 3 | 8.02 | 11.02 |
|  | 15 | 30 | 38 | 11 | 3 | 2.27 | 5.27 |
|  | 20 | 30 | 51 | 11 | 3 | 1.39 | 4.39 |
|  | 25 | 30 | 65 | 11 | 3 | 0.98 | 3.98 |
|  | 30 | 30 | 78 | 11 | 3 | 0.78 | 3.78 |
|  | 35 | 30 | 92 | 11 | 3 | 0.63 | 3.63 |
|  | 40 | 30 | 106 | 11 | 3 | 0.53 | 3.53 |
|  | 45 | 30 | 119 | 11 | 3 | 0.46 | 3.46 |
|  | 50 | 30 | 133 | 11 | 3 | 0.41 | 3.41 |
|  | 60 | 30 | 162 | 11 | 3 | 0.33 | 3.33 |
|  | 70 | 30 | 189 | 11 | 3 | 0.28 | 3.28 |
|  | 80 | 30 | 217 | 11 | 3 | 0.24 | 3.24 |
|  | 90 | 30 | 245 | 11 | 3 | 0.21 | 3.21 |
|  | 100 | 30 | 273 | 11 | 3 | 0.19 | 3.19 |
| NOTE 1: For 5MHZ CBW, LP-WUS power can be boosted if BS can transmit signal with more than 46dBm transmission power. Otherwise, it should be 0dB. |  |  |  |  |  |  |  |

Maybe for small CBWs LP-WUS power boosting is not permitted because of the large power deboosting of NR signal. It depends on implementation to some degree and this is why we want to leave it to manufacture declaration. Although for small CBWs the permitted LP-WUS power boosting is 0dB, this definition does exist, while EPRE ratio in Option 1 does not always exist, e.g. for 5MHz CBW with 30kHz SCS, which is not preferred as a definition.

Second, EPRE ratio is a relative value, which can not reflect the effective information we want directly. What we really care about are how much power LP-WUS can be boosted, and at the same time how much power NR signal has to be deboosted. Dynamic range can reflect the boosted power of LP-WUS by original LP-WUS RE power and the deboosted power of NR by original NR RE power quite directly.

Third, dynamic range comes from legacy definition of power boosting for NB-IoT, at the same time this definition is also used in legacy conformance test. It is friendly for manufactures to reuse some design for NB-IoT in LP-WUS. When talking about power boosting, the only difference between NB-IoT and LP-WUS is that NB-IoT occupies only 1 RB while LP-WUS occupies 11 RBs. Power boosting for NB-IoT and LP-WUS is the same operation, such that to avoid ambiguity and unpredictable work in the future, we should not introduce another different definition for this kind of operation.

For all the reasons above, we still prefer Option 2 as the concept of LP-WUS power boosting.

Proposal 1: Define LP-WUS dynamic range/power boosting in Option 2 rather than EPRE ratio: The LP-WUS RB power dynamic range (or LP-WUS power boosting) is the difference between the average power of LP-WUS REs (which occupy certain REs within an NR transmission bandwidth configuration) and the average power over all REs (from both LP-WUS and the NR carrier containing the LP-WUS REs).

## 2.2 Manufacture Declaration

RAN4 has achieved a consensus that the transmit power is shared between NR signal and LP-WUS for the same carrier and the total transmit power is fixed[2]. In this case, when the transmission power of LP-WUS increases, the transmission power of NR signal decreases. As we discussed in our previous contribution [3], the permitted power boosting of LP-WUS depends on the channel bandwidth and the position of WUS RBs, which is up to implementation.

Besides according to RE power control dynamic range in TS38.104 as follows, if NR signal is modulated by 64QAM, 256QAM or 1024QAM, the up and down range is 0dB.

In this case, if LP-WUS is transmitted together with NR PDSCH modulated by 64QAM, 256QAM or 1024QAM, no LP-WUS power boosting is permitted. Therefore, the permitted LP-WUS power boosting also depends on the NR modulation orders, but the modulation orders depend on implementation.

Considering that the power boosting of LP-WUS depends on implementation, LP-WUS power boosting should be completely manufacture declared and no minimum limitation or maximum limitation should be introduced. If it is a manufacture declaration feature, there is no need to preclude small channel bandwidth in spec.

Proposal 2: Set the LP-WUS power boosting a complete manufacture declaration feature.

Proposal 3: Not to introduce a minimum limitation or a maximum limitation for LP-WUS power boosting.

## 2.3 Transmitted Signal Quality

Regarding transmit signal quality, based on the signal design, LP-WUS information is contained in time-domain for envelope detection and time-domain correlation, and is contained in frequency-domain for frequency-domain correlation. Since RAN1 has not finished the design of correlation detection for LP-WUS, here we only talk about the transmitted signal quality requirements for envelope detection.

Traditional Tx EVM is tested after FFT process based on constellations, which may not work for LP-WUS with envelope detection. Therefore, new RF envelope requirements is needed for LP-WUS to guarantee transmitted signal quality and decoding performance.

In A-IoT, companies are considering to refer to RFID RF envelope related parameters to define signal transmission quality requirement [4], such as modulation depth, RF envelope ripple, RF pulse width. The waveform generation methods of LP-WUS and A-IoT are not completely same, which results in different time-domain signal characteristics as shown in Fig. 2. The time-domain signal of A-IoT has stable ripple while the time-domain signal of LP-WUS is severe.

Fig. 2  The difference between time-domain signals of LP-WUS and A-IoT (2 OFDM symbols with CP)

However, it can also be a reference direction for LP-WUS transmitted signal quality because they are both OOK-based signal. In LP-WUS, we care about the power difference between OOK ON and OOK OFF most. Therefore, we can FFS whether the modulation depth can be used as transmitted signal quality for LP-WUS.

Proposal 4: FFS whether the modulation depth can be used as transmitted signal quality for LP-WUS.

## 2.4 Applicable BS Types

In last meeting, no agreement on the applicable BS types to support LP-WUS has been achieved yet[1]. Currently in TS38.104, only BS Type 1-C supports NR with in-band NB-IoT operation. BS Type 1-H and BS Type 1-O are excluded considering the power consumption issues. When it comes to LP-WUS, similar issues exist. LP-WUS occupies more RBs than NB-IoT. In this case, we prefer to consider BS Type 1-C as applicable type and FFS other types.

Proposal 5: It is propose to use BS Type 1-C as applicable BS type as a start point.

- Conclusion

Proposal 1: Use concept of LP-WUS dynamic range/power boosting in Option 2 rather than EPRE ratio: The LP-WUS RB power dynamic range (or LP-WUS power boosting) is the difference between the average power of LP-WUS REs (which occupy certain REs within an NR transmission bandwidth configuration) and the average power over all REs (from both LP-WUS and the NR carrier containing the LP-WUS REs).

Proposal 2: Set the LP-WUS power boosting a complete manufacture declaration feature.

Proposal 3: Not to introduce a minimum limitation or a maximum limitation for LP-WUS power boosting.

Proposal 4: FFS whether the modulation depth can be used as transmitted signal quality for LP-WUS.

Proposal 5: It is propose to use BS Type 1-C as applicable BS type as a start point.

- Reference

- R4-2416589 Way For [112bis][308] NR_LPWUS, RAN4#112bis, Hefei, China, Oct. 2024.

- R4-2406140 Way Forward for [110bis][314] NR_LPWUS, RAN4#110bis, Changsha, China, Apr. 2024.

- R4-2411894 Discussion on BS RF requirements for LP-WUS/WUR, RAN4#112, ZTE, Maastricht, Netherlands, Aug. 2024.

- R4-2415234 Topic summary for [112bis][130] FS_Ambient_IoT_solutions_part2.