# 3GPP TSG-RAN WG4 Meeting #113                               R4-2418830

# Orlando, USA, 18th-22th Nov, 2024

# Source:        ZTE Corporation, Sanechips

# Title:           Discussion on testability of LP-WUR

# Agenda Item:   7.23.2.4

# Document for:  Approval

- Introduction

In previous RAN4 meetings, there was very limited time to discuss the testability issues. One of the most important issues is that we should align the test procedure for IDLE mode and CONNECTED mode first. Then the detailed implementation can be left to RAN5.

In this contribution, we discussed the test procedure for IDLE mode and CONNECTED mode.

- Discussion

Usually legacy test for NR receiver is performed in CONNECTED mode where there is ACK/NACK feedback to TE to calculate the throughput. The test for LP-WUR is to check the capability to wake up the main radio (MR) and the metric is BLER/MDR. LP-WUS can be used in IDLE mode to wake up MR to receive Paging message and CONNECTED mode to trigger MR to receive PDCCH.

## 2.1 Test in IDLE Mode

For test procedure of LP-WUS mechanism in IDLE mode is as follows.

- Step 1: TE transmits LP-WUS to the UE, which carries UE group ID. - Step 2: UE receives and demodulate the signal. If UE group ID is correct, go to step 3. Step 3: Wake up the MR to receive corresponding Paging message from TE and feed back PRACH to the TE. Then TE record a successful LP-WUS detection.

For Step 1, if there is no WUS transmitted from TE but UE wakes up the MR, it is a false alarm. In this case, even if MR is waken up, there is no corresponding Paging message at all, the connection will not be established. For Step 2, if there is something wrong with the received WUS caused by low SNR for example and CRC check does not pass or correlation power does not exceed the threshold, UE will not wake up the MR correctly. This is a missed detection.

However, MR may stay in a cycle of being waken up and sleeping. The time cost is unacceptable.

Proposal 1: The test in IDLE mode follows the steps below:

- Step 1: TE transmits LP-WUS to the UE, which carries UE group ID.

- Step 2: UE receives and demodulate the signal. If UE group ID is correct, the MR will be waken up.

Step 3: Wake up the MR to receive corresponding Paging message from TE and feed back PRACH to the TE. Then TE records a successful LP-WUS detection.

## 2.2 Test in CONNECTED Mode

The test procedure of LP-WUS mechanism in CONNECTED mode is as follows.

- Step 1: TE transmits LP-WUS to UE, which carries UE group ID. - Step 2: UE receives and demodulates the signal. If UE group ID is correct, go to step 3. - Step 3: MR will be triggered to receive PDCCH/PDSCH and feed back ACK/NACK. Then TE record a successful LP-WUS detection.

In this case, legacy test method can be reused while the test metric is MDR (or with FAR together) rather than throughput. If TE transmits LP-WUS and corresponding PDCCH and PDSCH but there is no ACK/NACK fed back, it is regarded as a missed detection. If LP think a LP-WUS is received but MR does not find corresponding PDCCH, it is regarded as a false alarm.

Proposal 2: The test in CONNECTED mode follows the steps below:

- Step 1: TE transmits LP-WUS to UE, which carries UE group ID.

- Step 2: UE receives and demodulates the signal. If UE group ID is correct, go to step 3.

Step 3: MR will be triggered to receive PDCCH/PDSCH and feed back ACK/NACK. Then TE records a successful LP-WUS detection.

Compared to the test in IDLE mode that MR needs to stay in a cycle of waken-up and sleep, test in CONNECTED mode is relatively easy and some legacy test mechanism can be reused. Thus, we prefer to test the LP-WUR in CONNECTED mode. The detailed test implementation can be left to RAN5.

In last meeting, some companies proposed that a new test mode that there is no need to wake up the MR can be considered. If this new test mode is possible in terms of implementation, we can also agree with it.

Proposal 3: If the new test mode that does not need to wake up MR is possible in terms of implementation, we also agree with it. Otherwise, we prefer to test the LP-WUR in CONNECTED mode. The detailed test implementation can be left to RAN5.

- Conclusion

Proposal 1: The test in IDLE mode follows the steps below:

- Step 1: TE transmits LP-WUS to the UE, which carries UE group ID.

- Step 2: UE receives and demodulate the signal. If UE group ID is correct, the MR will be waken up.

Step 3: Wake up the MR to receive corresponding Paging message from TE and feed back PRACH to the TE. Then TE record a successful LP-WUS detection.

Proposal 2: The test in CONNECTED mode follows the steps below:

- Step 1: TE transmits LP-WUS to UE, which carries UE group ID.

- Step 2: UE receives and demodulates the signal. If UE group ID is correct, go to step 3.

Step 3: MR will be ready to receive PDCCH and PDSCH, then feed back ACK/NACK. Then TE record a successful LP-WUS detection.

Proposal 3: If the new test mode that does not need to wake up MR is possible in terms of implementation, we also agree with it. Otherwise, we prefer to test the LP-WUR in CONNECTED mode. The detailed test implementation can be left to RAN5.

- Reference

- R4-2414309 WF for LP-WUS UE RF, RAN4#112, Maastricht, Netherlands, Aug. 2024.

- R4-2417112 WF on UE RF requirements for LP-WUS, RAN4#112bis, Hefei, China, Oct 2024.